Informative Inventory Report

about Belgium’s annual submission of air emission data reported in February 2010 under the Convention on Long Range Transboundary Air Pollution CLRTAP

March 2010

Informative inventory report about Belgium’s annual submission of air emission data reported in February 2010 under the Convention on Long Range Transboundary Air Pollution CLRTAP

Chapter 0.	Executive Summary	3

Chapter 1.	Introduction	3

1.1.	Institutional arrangements for inventory preparation and process of inventory preparation	3

1.2.	Methods and data sources	4

1.2.1.	Flemish region	4

1.2.2.	Walloon region	4

1.2.3.	Brussels region	4

1.2.4.	Reporting of emissions from mobile sources	5

1.3.	Keys Categories	5

1.3.1.	Level assessment	5

1.3.2.	General remarks	8

1.4.	QA/QC and Verification methods	11

1.5.	General assessment of completeness	11

1.5.1.	Sources Not Estimated (NE)	11

1.5.2.	Sources Included Elsewhere (IE)	11

1.5.3.	An account of sub-sources included in reporting codes ‘Other’	11

Chapter 2.	Explanation of key sources	12

Chapter 3.	Energy (NFR sector 1)	15

3.1.	General methodology	15

3.2.	Default Emission factors	15

3.2.1.	SO2 and NOx	15

3.2.2.	NH3	17

3.2.3.	Others pollutants	17

3.3.	Energy industries (category 1.A.1)	19

3.3.1.	Public electricity and heat plants (category 1.A.1.a)	19

3.3.2.	Petroleum refining (category 1.A.1 b)	19

3.3.3.	Manufacture of solid fuels and other energy industries  (category 1.A.1.c)	19

3.4.	Manufacturing industry and construction (category 1.A.2)	20

3.4.1.	General	20

3.4.2.	Iron and steel sector	20

3.5.	Transport (category 1.A.3 and 1.A.5.b)	20

3.5.1.	Road transport	20

3.5.2.	Air transport	20

3.5.3.	Railways	21

3.5.4.	Navigation	21

3.6.	Other sectors (category 1.A.4)	21

3.7.	Other (category 1.A.5.a and 1.A.5.b)	22

3.8.	Fugitive emissions from fuels (category 1.B.1 and 1.B.2)	22

3.8.1.	Solid fuel transformation (category 1.B.1.b)	23

3.8.2.	Petroleum refineries (category 1.B.2.a)	23

3.9.	International bunkers (category Memo Items – International Bunkers)	23

Chapter 4.	Industrial processes (NFR sector 2)	23

4.1.	Overview	23

4.1.1.	Description of the sector	23

4.1.2.	Allocation of emissions	23

4.2.	Methodological issues	24

4.2.1.	Mineral products (category 2.A)	24

4.2.2.	Chemical industry (category 2.B)	24

4.2.3.	Metal production (category 2.C)	25

Chapter 5.	Solvent and other product use (NFR sector 3)	25

Chapter 6.	Agriculture (NFR sector 4)	26

6.1.	Data sources	26

6.2.	Direct soil emission (category 4D)	29

Chapter 7.	Waste (NFR sector 6)	30

7.1.	Solid waste disposal on land (category 6A)	30

7.2.	Wastewater treatment (category 6B)	30

7.3.	Waste incineration (category 6C)	30

7.4.	Others (category 6D)	31

Chapter 8.	Recalculations and Improvements	32

8.1.	Recalculations	32

8.2.	Allocation changes of emissions	33

8.2.1.	Flemish Region	33

8.2.2.	Walloon Region	34

8.2.3.	Brussels Capital Region	34

8.3.	Optimisation and actualisation of emission reporting	34

8.3.1.	Flemish Region	34

8.3.2.	Walloon Region	35

8.3.3.	Brussels Capital Region	35

# Executive Summary

This report gives some more explanation about the air emissions of Belgium reported in February 2010 under the Convention on Long-range Transboundary Air Pollution CLRTAP.

In an objective to improve continuously this report, a first outline was written mainly concerning the methodological issues of the SO2 ,NOx  and NH3 emissions.  There are some more information’s about others pollutants collected during the in-depth review of the Belgium inventory in July 2009.  Most of the answers to the review team are incorporated in this report.

Main differences from last submission

- The Informative Inventory Report is submitted in the new format.

- The methodological issues have been extended with all the answers to the review team during the in-country review.

- The activity data by NFR category are incorporated in the NFR tables for the complete time series.

- To continue the quality improvement, the notation keys have been further completed and controlled.

- An extended streamlining exercise has still been carried out between the emission values reported for EMEP/LRTAP and for the MM.

Priorities for improvement/incompleteness

- Currently, the following studies take place in order to improve and optimise the emission inventory:

  - Optimization NMVOC emission inventory of the sector “Domestic Solvent Use” (study performed by Arcadis Belgium under the authority of Brussels Environment).

  - In Wallonia, optimization of NMVOC emission inventory of the sector “use of solvents” by Econotec for the years 2005-2006 and 2007

  - In Wallonia, optimization of POPs emission inventory for the years 2005-2006 and 2007 by Econotec.

  - Update of the EMMOSS model used to calculate emissions in Flanders from railways, inland waterways navigation and sea navigation on Belgian territory.

# Introduction

## Institutional arrangements for inventory preparation and process of inventory preparation

To understand the Belgian inventory, it is important to know that the major part of environmental responsibilities lies within the regions. Preparing the atmospheric emission inventories is one of these responsibilities. Each region implements the necessary means to establish its own emission inventory in the NFR format. The emission inventories of the three regions are subsequently combined to form the national emission inventory. Since 1980, the three regions have been developing their own methodologies (depending on various external factors) for compiling their atmospheric emission inventories. Obviously, it requires some coordination to ensure the consistency of the data and the establishment of the national inventory. This coordination is one of the permanent duties of the Working Group on ‘Emissions’ of the Coordination Committee for International Environmental Policy (CCIEP). In this working group the different actors decide how the regional data will be aggregated to a national total and which data will be sent officially for Belgium – taking into account the specific characteristics and interests of each region as well as the available means. The Interregional Environment Unit (CELINE - IRCEL) is responsible for compiling the emission data from the inventories of the three regions to the national inventory.

In spite of the intensive work that is already carried out and is still going on to improve estimates of air emissions in Belgium, it remains a continuous task of all members involved to improve the quality of emission inventories. This is very important, especially in the framework of the NEC (National Emission Ceiling) Directive where the emission data of this submission are of an indispensable value to the baseline scenario to be developed under the CAFE (Clean Air for Europe) programme.

## Methods and data sources

As a consequence of the responsibility of the regions in compiling gas inventories, concomitant methodologies have been developed by the three regions for compiling their inventory from basic data. This section describes the general approach developed by each region.

### Flemish region

In Flanders, the inventory is set up by the Department Air, Environment and Communication of the Flemish Environmental Agency (VMM).

Since the reporting year of 1993 most important industrial companies in the Flemish region in relation with air pollution are obliged to report annually about their emissions when exceeding a defined threshold value.

From 2006 on this reporting obligation was harmonized in the Flemish region with the EPER-decision (2000/479/EC) and with the EPRTR-regulation (166/2006/EC).

In total approximately 350 industrial companies are registered in an industrial database in the Flemish region due to this obliged emission reporting by the industrial companies.

Mainly for the refineries, iron and steel sectors and the chemical industry (process emissions) this obliged reporting of emissions remains since that time an important source of information for the European and international reporting obligations.

To have a total picture of all emissions by industrial activities in Flanders, also activities with emissions below the threshold have to be taken into account. These emissions are estimated in a collective way. The collective estimation of the emissions due to combustion processes is done by multiplying the energy data, obtained from the Energy Balance for Flanders, made by the Flemish Institute for Technological Research (VITO, Vlaams Instituut voor Technologisch Onderzoek), with specific emission factors.

### Walloon region

The emission inventories of the Walloon region are compiled by the Walloon Agency for Air and Climate (AWAC) using the EMEP/EEA methodology. Emission factors used, are performed for all industrial sectors. In some cases as agriculture and forestry, the emissions estimates are based on a specific study reflecting the Walloon environment.

One main data source for the inventory preparation is the energy balance delivered yearly by the Energy and Sustainable Building Department.  The energy balance describes the quantities of energy imported, produced, transformed and consumed in the Walloon Region in a given year. In 2003, an environmental integrated survey has been created which includes all pertinent environment-related reporting requirements for 300 companies. The environmental integrated survey is personalised to the 300 operators of the activities/installations pointed out by one or several regulations (four international Conventions and their protocols, seven European Directives, three European Regulations, two European Decisions, one European Recommendation, two Walloon laws, one Walloon Decree and several non legally binding agreements).

### Brussels region

The emission inventory in the Brussels-Capital region is compiled by Brussels Environment (the Institute for Environmental Management) using the EMEP/EEA methodology. The emissions are calculated by multiplying activity data by an emission factor. Generally, these activity data and emission factors used in the Brussels inventory are estimated on the basis of research projects funded by Brussels Environment (for instance, the annual energy balance that is established on a survey among energy suppliers, federations and individual consumers). These projects combine the socio-economic Brussels specificities and the reference values of the IPCC Guidelines, specific bibliographies like PARCOM, TNO, EPA,… as well as the joint EMEP/EEA guidebook . The different sectors taken into account in the Brussels emissions inventory reflect the characteristics of a strict urban environment.

Nearly all the emissions of this urban region originate from energy consumption (Residential, Commercial and Road Transport).

### Reporting of emissions from mobile sources

Belgium reports memo items. The following categories are included in the national totals: civil aviation (LTO), international aviation (LTO) and national navigation.

The following categories are reported in memo items and are not included in the national totals: domestic aviation (cruise), international aviation (cruise), international maritime navigation and international inland waterways.

Some additional information on requested memo items from mobile sources is given in the Excel footnotes Table IV 1 F5.

Notice: In the Flemish inventory, emissions from maritime navigation were calculated with the emission model EMMOSS. The emissions originating from maritime shipping starting and arriving in Belgium (including sand extraction at sea, dredging activities and tugboats) are reported in the category 1A3di(ii) (international inland waterways). The emissions coming from maritime shipping between a (Flemish) and a foreign harbour (including emissions originating in the Flemish harbour) are reported in the memo item ‘international maritime navigation’. The Flemish emissions originating from inland shipping are for the first time calculated with the EMMOSS emission model are reported in the IPCC category 1A3dii (national navigation).

As was mentioned in the previous IIR the air emissions of railway traffic are also recalculated with the EMMOSS model. Emissions are reported in the IPCC category 1A3c (railways).

As a consequence, the emission time series of both the IPCC categories 1A3dii and 1A3c are completely revised compared to previous submissions.

## Keys Categories

### Level assessment

A key source analysis (level assessment) is performed for 2007 (Excel tables ‘key source (level) 2007-IIR2010) and 2008 (Excel tables ‘key source (level) 2008-IIR2010) at the least aggregated level (columns D-G). To evaluate the key sources over the years, the key source analysis (level) for 1990 (all contaminants except particulate matter) and for 2000 (particulate matter) are actualised as well. These level assessments (see Excel tables) cover 95% of the total national aggregated emissions.  The key source analysis of the NEC pollutants and PM10 are presented in the following tables for the year 2007.

| Source category | 2007 | Level ass. | Cum. Total |
| --- | --- | --- | --- |
| NOx |  |  |  |
| 1 A 3 b iii Road transport:, Heavy duty vehicles | 59.36185 | 0.2295914 | 0.229591 |
| 1 A 3 b i Road transport: Passenger cars | 45.58363 | 0.17630195 | 0.405893 |
| 1 A 1 a Public electricity and heat production | 24.76147 | 0.09576892 | 0.501662 |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 15.76293 | 0.06096563 | 0.562628 |
| 1 A 4 b i Residential: Stationary plants | 15.29751 | 0.05916556 | 0.621793 |
| 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 13.88774 | 0.05371306 | 0.675507 |
| 2 A 1 Cement production | 13.69707 | 0.0529756 | 0.728482 |
| 1 A 3 b ii Road transport:Light duty vehicles | 11.09566 | 0.04291424 | 0.771396 |
| 1 A 2 c Stationary combustion in manufacturing industries and construction: Chemicals | 7.715953 | 0.02984268 | 0.801239 |
| 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 7.11414 | 0.02751507 | 0.828754 |
| 1 A 1 b Petroleum refining | 5.579869 | 0.02158103 | 0.850335 |
| 1 A 3 d ii National navigation (Shipping) | 4.962685 | 0.01919398 | 0.869529 |
| 1 A 4 a i Commercial / institutional: Stationary | 4.695389 | 0.01816017 | 0.887689 |
| 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 3.665667 | 0.01417755 | 0.901867 |
| 1 A 2 e Stationary combustion in manufacturing industries and construction: Food processing, beverages and tobacco | 3.61225 | 0.01397095 | 0.915838 |
| 1 A 3 d i (ii) International inland waterways | 3.573 | 0.01381915 | 0.929657 |
| 2 A 2 Lime production | 3.47529 | 0.01344124 | 0.943098 |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 2.23903 | 0.00865981 | 0.951758 |

| Source category | 2007 | Level ass. | Cum. Total |
| --- | --- | --- | --- |
| NMVOC |  |  |  |
| 3 D 2 Domestic solvent use including fungicides | 19.83325 | 0.15952 | 0.15952 |
| 3 A 2 Industrial coating application | 15.83568 | 0.127367 | 0.286887 |
| 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 15.34749 | 0.12344 | 0.410327 |
| 3 D 1 Printing | 6.969575 | 0.056057 | 0.466384 |
| 1 A 4 b i Residential: Stationary plants | 6.4453 | 0.05184 | 0.518224 |
| 1 A 3 b i Road transport: Passenger cars | 6.231099 | 0.050117 | 0.568341 |
| 1 A 3 b iv Road transport: Mopeds & motorcycles | 5.63665 | 0.045336 | 0.613676 |
| 3 A 1 Decorative coating application | 5.2833 | 0.042494 | 0.65617 |
| 3 D 3 Other product use | 4.24385 | 0.034133 | 0.690304 |
| 1 B 2 a iv Refining / storage | 3.870756 | 0.031133 | 0.721436 |
| 1 B 2 b Natural gas | 3.3307 | 0.026789 | 0.748225 |
| 3 C Chemical products | 3.11614 | 0.025063 | 0.773288 |
| 2 D 2 Food and drink | 2.973552 | 0.023916 | 0.797205 |
| 1 B 2 a v Distribution of oil products | 2.443286 | 0.019651 | 0.816856 |
| 1 A 3 b iii Road transport:, Heavy duty vehicles | 2.248332 | 0.018083 | 0.83494 |
| 3 B 1 Degreasing | 1.7505 | 0.014079 | 0.849019 |
| 1 B 1 b Fugitive emission from solid fuels: Solid fuel transformation | 1.26803 | 0.010199 | 0.859218 |
| 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 1.207693 | 0.009714 | 0.868931 |
| 1 A 1 a Public electricity and heat production | 1.137494 | 0.009149 | 0.87808 |
| 2 C 1 Iron and steel production | 1.04842 | 0.008432 | 0.886513 |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.958879 | 0.007712 | 0.894225 |
| 3 B 2 Dry cleaning | 0.948487 | 0.007629 | 0.901854 |
| 1 A 3 b ii Road transport:Light duty vehicles | 0.934 | 0.007512 | 0.909366 |
| 4 G Agriculture other(c) | 0.93273 | 0.007502 | 0.916868 |
| 1 A 1 c Manufacture of solid fuels and other energy industries | 0.92073 | 0.007405 | 0.924273 |
| 2 A 7 d Other Mineral products (Please specify the sources included/excluded in the notes column to the right) | 0.87407 | 0.00703 | 0.931304 |
| 2 D 1 Pulp and paper | 0.79015 | 0.006355 | 0.937659 |
| 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.762296 | 0.006131 | 0.94379 |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 0.710926 | 0.005718 | 0.949508 |
| 2 A 2 Lime production | 0.68284 | 0.005492 | 0.955 |

| Source category | 2007 | Level ass. | Cum. Total |
| --- | --- | --- | --- |
| SOx |  |  |  |
| 1 A 1 b Petroleum refining | 22.7998 | 0.181738 | 0.181738 |
| 1 A 1 a Public electricity and heat production | 18.3419 | 0.146204 | 0.327941 |
| 1 A 4 b i Residential: Stationary plants | 16.66172 | 0.132811 | 0.460752 |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 11.89201 | 0.094791 | 0.555544 |
| 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 10.58101 | 0.084341 | 0.639885 |
| 2 G Other production, consumption, storage, transportation or handling of bulk products (Please specify the sources included/excluded in the notes column to the right) | 6.485467 | 0.051696 | 0.691581 |
| 2 A 1 Cement production | 6.25314 | 0.049844 | 0.741425 |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 4.2519 | 0.033892 | 0.775317 |
| 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 3.996003 | 0.031852 | 0.807169 |
| 1 A 2 c Stationary combustion in manufacturing industries and construction: Chemicals | 3.867507 | 0.030828 | 0.837997 |
| 1 A 2 e Stationary combustion in manufacturing industries and construction: Food processing, beverages and tobacco | 3.803102 | 0.030315 | 0.868312 |
| 2 A 2 Lime production | 3.1082 | 0.024776 | 0.893087 |
| 2 C 5 e Other metal production (Please specify the sources included/excluded in the notes column to the right) | 2.822688 | 0.0225 | 0.915587 |
| 1 A 4 a i Commercial / institutional: Stationary | 2.055196 | 0.016382 | 0.931969 |
| 1 A 1 c Manufacture of solid fuels and other energy industries | 1.80846 | 0.014415 | 0.946384 |
| 1 A 3 d i (ii) International inland waterways | 1.222 | 0.009741 | 0.956125 |

| Source category | 2007 | Level ass. | Cum. Total |
| --- | --- | --- | --- |
| NH3 |  |  |  |
| 4 B 8 Swine | 21.99694 | 0.316573 | 0.3165 |
| 4 B 1 b Cattle non-dairy | 14.08669 | 0.202731 | 0.51930 |
| 4 B 1 a Cattle dairy | 11.57992 | 0.166655 | 0.68595 |
| 4 D 1 a Synthetic N-fertilizers | 8.877488 | 0.127762 | 0.81372 |
| 4 D 2 c N-excretion on pasture range and paddock unspecified (Please specify the sources included/excluded in the notes column to the right) | 2.810807 | 0.040452 | 0.8541 |
| 4 B 9 a Laying hens | 2.257025 | 0.032482 | 0.8866 |
| 6 B Waste-water handling | 1.88835 | 0.027177 | 0.91383 |
| 4 B 9 b Broilers | 1.57187 | 0.022622 | 0.936451 |
| 1 A 3 b i Road transport: Passenger cars | 1.350382 | 0.019434 | 0.9558 |

| Source category | 2007 | Level ass. | Cum. Total |
| --- | --- | --- | --- |
| PM10 |  |  |  |
| 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 4.60254 | 0.140827 | 0.140827 |
| 2 C 1 Iron and steel production | 4.55938 | 0.139506 | 0.280333 |
| 1 A 3 b i Road transport: Passenger cars | 2.310006 | 0.070681 | 0.351013 |
| 2 A 7 a Quarrying and mining of minerals other than coal | 2.24019 | 0.068544 | 0.419558 |
| 1 A 3 b iii Road transport:, Heavy duty vehicles | 2.229596 | 0.06822 | 0.487778 |
| 1 A 4 b i Residential: Stationary plants | 1.942817 | 0.059446 | 0.547224 |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 1.435414 | 0.04392 | 0.591144 |
| 4 B 8 Swine | 1.27802 | 0.039104 | 0.630248 |
| 1 A 3 b vi Road transport: Automobile tyre and brake wear | 1.26428 | 0.038684 | 0.668932 |
| 1 A 3 b vii Road transport: Automobile road abrasion | 0.87349 | 0.026727 | 0.695659 |
| 1 A 1 b Petroleum refining | 0.8131 | 0.024879 | 0.720538 |
| 1 B 1 b Fugitive emission from solid fuels: Solid fuel transformation | 0.69844 | 0.021371 | 0.741909 |
| 2 A 7 d Other Mineral products (Please specify the sources included/excluded in the notes column to the right) | 0.6467 | 0.019787 | 0.761696 |
| 1 A 1 a Public electricity and heat production | 0.630744 | 0.019299 | 0.780995 |
| 1 A 3 b ii Road transport:Light duty vehicles | 0.585789 | 0.017924 | 0.798919 |
| 4 B 1 b Cattle non-dairy | 0.584121 | 0.017873 | 0.816792 |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 0.53621 | 0.016407 | 0.833198 |
| 2 D 2 Food and drink | 0.4966 | 0.015195 | 0.848393 |
| 1 A 2 c Stationary combustion in manufacturing industries and construction: Chemicals | 0.47012 | 0.014385 | 0.862778 |
| 1 A 3 c Railways | 0.458079 | 0.014016 | 0.876794 |
| 4 B 9 b Broilers | 0.4017 | 0.012291 | 0.889085 |
| 4 B 9 a Laying hens | 0.39828 | 0.012186 | 0.901271 |
| 1 A 2 e Stationary combustion in manufacturing industries and construction: Food processing, beverages and tobacco | 0.31777 | 0.009723 | 0.910994 |
| 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 0.2635 | 0.008062 | 0.919057 |
| 2 G Other production, consumption, storage, transportation or handling of bulk products (Please specify the sources included/excluded in the notes column to the right) | 0.2469 | 0.007555 | 0.926611 |
| 6 A Solid waste disposal on land | 0.2415 | 0.007389 | 0.934001 |
| 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | 0.21604 | 0.00661 | 0.940611 |
| 1 A 3 d ii National navigation (Shipping) | 0.200321 | 0.006129 | 0.94674 |
| 4 B 1 a Cattle dairy | 0.174801 | 0.005348 | 0.952089 |

The contribution of the trend of a particular source category to the trend of the total Belgian emission inventory, as well as the determination of its key sources are calculated for the main pollutants in the Excel table ‘Key source (trend) base-2007_IIR2010’ and the excel table ’. Key source (trend) base-2008_IIR2010’.

The level assessments result in the identification of a number of key sources, as do also the trend assessments. Key categories identified in either the level assessment or the trend assessment (or both, as a number of key categories overlap to a large extent in level and trend assessments) are ranked. The absolute change in emission values of these key sources for a particular pollutant over the period 1990-2007 is given in the figures of the file ‘Key source absolute emission trends(base-2007)_IIR2010’ and see “Chapter 2- Explanation of key trends”..

### General remarks

By comparing the key sources between 1990 and 2007, some remarks can be made. Besides some (smaller) shifts in the order of ranking, a number of more structural shifts in the following sectors can be seen:

- 1A1a Public electricity and heat production: appears as a key source for NMVOC, CO and dioxin and disappears for Se in 2007 compared to 1990.  NMVOC emissions increase with the increase of wood consumption in the power plants and Se emissions decrease with the termination of some coal power plants.

- 1A1b Petroleum refining: appears as a key source for CO, Cd, As, Cr and Ni in 2007 compared to 1990. In 1990, the refining plants (all situated in the Flemish region) were not yet obliged to report their emissions in 1990 (obligation from 1993 described in Vlarem II). As a result, very little information at plant level is available for CO, Cd, As, Cr and Ni emissions before 1993. Emissions were only estimated collectively based on the existing knowledge.

- 1A1c Manufacture of solid fuels: disappears as a key source for NOx in 2007, mainly because of the termination of the activities of the Brussels and Flemish coke ovens respectively in 1993 and 1996 and the last Flemish mining activities in 1992; appears as a key source for CO, NMVOC and PM10, mainly due to absolute emission changes in other sectors

- 1A2a Iron and steel: appears as a key source in 2007 for NMVOC, Cd, Cr, Cu and Zn. The increasing importance of the sector in the emissions of NMVOC, Pb and Cd is due to the relatively stable emissions compared to other sectors that have reached higher emission reductions.

- 1A2b Non-ferro: for a number of heavy metals (Pb/Cd/Cu/Ni/Zn), the non-ferro industry disappears as key source in 2007 compared to 1990. This is due to the efforts of one Flemish company with several exploitations. The dusty emissions decrease significantly due to air purification installations that came into force in 1998 (to meet the Flemish environmental law Vlarem II).

- 1A2c chemical: for a number of heavy metals (Cr and Cu), the chemical industry disappears as key source in 2007 compared to 1990. This is mainly due to the reduction of the use of coal and residual oil as fuels.

- 1A2d Pulp, paper and print: appears for PM2.5, PM10, and As as a key source in 2007. The change of the relative importance of the pollutants is mainly due to absolute emission changes in other sectors (e.g. total As emissions decrease from 6.8 to 3,8 tonnes, while absolute As emissions in 1A2d slightly increase).

- 1A2e Food processing, beverages and tobacco : appears for NOx and disappears for Cr as key source in 2007 compared to 1990.  It can be explained by the relative stable emissions of NOx in this sector while the total emission is decreasing.  The reduction of Cr is due to the reduction of the residual oil as fuel.

- 1A2fi other: appears for CO and Hg as key source in 2007 compared to 1990. The relative greater importance of the sector for CO and Hg can be explained by absolute emission changes in other sectors (which become relatively less important).  .

- 1A3bi Road transportation (passenger cars): for SOx, the sector ‘passenger cars’ disappears as a key source, due to the enhanced use of petrol with a low sulphur content. For NH3, ‘passenger cars’ appears as a key source. This can probably be explained by the increased use of catalytic converters. It might be noted that passenger cars are still key source of Pb emissions, in spite of the removing of leaded petrol and the decrease of the lead content in low-lead petrol. The absolute Pb emissions of passenger cars have however strongly decreased. The sector appears as a key source for Cr, due to the higher fuel use and disappears for Ni.

- 1A3bii Road transportation (light duty): disappears as key source for Pb, due to the decrease of the lead content in fuel, as is mentioned above.  Zn appears as a key source in 2007 due to the use of COPERT IV in 2007 by two regions.

- 1A3biii Road transportation (heavy duty): disappears as key source for SOx, Pb and Ni and appears as a key source for Cd. The relatively lower sector importance in the SOx emissions is due to the decreased sulphur content in petrol. SOx emissions decrease also significantly for light duty vehicles, but this is not a key source in 1990 nor in 2007. Pb emissions decrease significantly due the already mentioned decrease of the lead content in fuel. The relatively greater importance of 1A3biii in the Cd emissions is due to the change of emissions in other sectors.

- 1A3biv Road transportation (mopeds and motorcycles): appears as a key source for the CO emissions due to the increasing number of motorcycles > 50cc.

- 1A3bv Gasoline evaporation.  NMVOC disappears as a key source in 2007 due to the use of COPERT IV in 2007 and the decrease of gasoline use.

- 1A4ai Commercial/institutional: appears as a key source in 2007 compared to the base year for SOx, Cd and As  mainly due to absolute emission changes in other sectors. PAH-total14 and Hg appears as a key source because of a shift (a.o. due to the economic situation) from fuel towards wood, which has a higher emission factor.

- 1A4bi Residential: appears as a key source for Pb, due to absolute emission changes in other sectors (mainly road transportation).

- 1A4ci Off road vehicles and other machinery: appears as a key source for NMVOC, due to emission changes in other sectors.

- 1A4cii Off road vehicles and other machinery: appears as a key source for NMVOC, due to emission changes in other sectors.

- 1B1b Solid fuel transformation: disappears as a key source for SOx because the activities of the Brussels and Flemish coke ovens have been terminated respectively in 1993 and 1996.

- 1B2avi Oil (other): the disappearance of 1B2avi as key sector for CO emissions in 2007 can be attributed to a different level of data handling in 1990 compared to 2007 in Flanders. In 1990 emissions are allocated to the correspondent IPCC code based on the SNAP code (installation level), while in 2007 emissions are attributed to the IPCC code at sector level (NACE code). Emissions of the catalytic cracking process in one of Flanders’s refineries (with SNAP code 040102) is allocated in 1990 in the sector 1B2avi, while the emissions of this process in 2007 are allocated in the sector 1A1b. Moreover absolute CO emissions of this refinery decreased significantly because of a modified operation mode in 1997 of the catalytic cracking unit (regeneration mode from partial to complete combustion).

- 2A1 Cement production: disappears as key source for As, Cr, Se, PM2.5, PM10 and TSP. A significant emission reduction was obtained due to a new dust purification system of one plant.  The sector becomes key source for Pb and dioxin in 2007 compared to 1990.  It can be attributed to emission changes in the other sectors

- 2A2 Lime production: appears as key source for NOx,, NMVOC and Hg and disappears for PM2.5 and PM10 . The change in relative importance of the sector for PM2.5, PM10 and Hg emissions can be attributed to emission changes in the other sectors.

- 2B2 Nitric acid production: disappears as a key sector for NOx, which can be attributed to a different level of data handling in 1990 compared to 2007 in Flanders. In 1990 emissions are allocated to the correspondent IPCC code based on the SNAP code (installation level), while in 2007 emissions are attributed to the IPCC code at sector level (NACE code). Emissions of the nitric acid production (with SNAP code 040402) are allocated in 1990 in the sector 2B2, while the emissions of this process in 2007 could not be reported separately and are allocated in the sector 2B5.

- 2B5 Other (chemical industry): disappears as a key source for Cd and Zn. The emissions in 1990 originate from a number of sulphuric acid production installations in a (Flemish) non-ferro plant. These installations have been shut down in 1992.

- 2C1 Iron and steel production: disappears as a key sector for NOx and SOx in 2007 compared to 1990 because of a different emission estimation method before and after 1993 (obligation from 1993 for Flemish plants to report their emissions as described in Vlarem II). Emissions before 1993 were estimated based on production numbers for NOx multiplied by an emission factor or S-content for SOx, while after 1993 measurements became available. 2C1 appears as a key sector for NMVOC.  There are some COV emissions non estimated in 1990 in Wallonia and now estimated in 2005.

- 2C5e Other metal production: appears as a key source for SOx, CO, Hg and Cr, which can be attributed to a different level of data handling in 1990 compared to 2007 in Flanders. In 1990 emissions are allocated to the correspondent IPCC code based on the SNAP code (installation level), while in 2007 emissions are attributed to the IPCC code at sector level (NACE code). Emissions of metal production are allocated in 1990 in the detailed sectors 2C1 to 2C5d, while all process emissions of the metal sector in 2007 are allocated in 2C5e. 2C5e disappears as key source for dioxins because of an emission reduction in the steel sector (emission reduction programme and the installation of an air purification installation) in the late 90’ies. Emissions of dioxins by the non-ferro sector are from 1999 on taken from the yearly reports provided by the companies.

- 2D1 Pulp and paper: appears for NMVOC.  It can be attributed to emission changes in the other sectors

- 2G Other: appears as a key sector for SOx, which can be attributed to a different level of data handling in 1990 compared to 2007 in Flanders. In 1990 emissions are allocated to the correspondent IPCC code based on the SNAP code (installation level), while in 2007 emissions are attributed to the IPCC code at sector level (NACE code). Process emissions that cannot be allocated to a specific IPCC code as in 1990 due to detail shortage are allocated in 2G.

- 4G appears for NMVOC Flanders: since in the new NFR templates 4D1 includes the emissions from N-fertiliser and 5E is not available anymore, NMVOC emissions from grassland are from the submission 2009 on allocated in 4G (Agriculture other), wherease NMVOC emissions from forests are allocated in the memo item 7B.

- 6A Solid waste disposal on land: appears as a key source for PM10 in 2007 compared to 2000 due to emission changes in other sectors.

- 6Ca Clinical waste incineration: is key sector in 1990 but not anymore in 2007. Dioxin emissions of hazardous waste are allocated in this sector. Because of a (structural) reorganisation of the sector in 1994, these emissions have strongly declined.

- 6Cb Industrial waste incineration: appears as a key sector in 2007 compared to 1990, although the absolute emission value has declined.

- 6Cc Municipal waste incineration: for dioxins and a number of heavy metals (Cd/Hg/As/Pb/Cr/Cu/Ni/Zn), waste incineration disappears as key source in 2007 compared to 1990. This can be attributed to the (structural) reorganisation of the sector in 1994, which included also air purification measures. Moreover, the majority of the waste incinerators recuperate their energy nowadays. As a consequence their emissions are reported under the sector 1A1a.

It can be assumed that most categories with a notation key NE will not bring big differences in the ranking of the key sources if they would be estimated, since most emissions are relatively low or even not existing.

The emissions of the categories that are IE (included elsewhere) are explained in the Excel Footnotes Table IV F3.

## QA/QC and Verification methods

In the framework of the European and international obligations with respect to the greenhouse gas emission inventory, Belgium has developed a QA/QC-plan.

Although this plan is focused on greenhouse gas emissions, a lot of these issues are also appropriate for the air pollutants.

Information about the developed QA/QC-plan in Belgium and all procedures involved can be find in the NIR (National Inventory Report) of March 2010, more specifically in chapter 1.6. ‘Information on the QA/QC plan including verification and treatment of confidentiality issues where relevant’.

The three regions have their own QA/QC procedures. The regional inventories are compiled by the national focal point (IRCEL-CELINE). Then the national inventory is checked by each region before the submission.

## General assessment of completeness

Notice 1: the industrial wastewater (category 6B1) is not considered because of a lack of data. Only domestic wastewater emissions are calculated in Belgium for the time being.

Notice 2: the emissions of PAH’s are reported only as a total of PAH’s.

### Sources Not Estimated (NE)

Cfr. Excel footnotes Table IV 1 F2.

### Sources Included Elsewhere (IE)

Cfr. Excel footnotes Table IV 1 F3.

### An account of sub-sources included in reporting codes ‘Other’

Cfr. Excel footnotes Table IV 1 F4.

# Explanation of key sources

The absolute changes in emission values of the key sources for the NEC pollutants and PM10 pollutant over the period 1990-2007 are shown in the following figures.

Trends/time series inconsistencies

- In 1993 environmental legislation (Vlarem II) came into force in Flanders. This included a reporting obligation for class 1 industrial plants, which induced in some cases a difference in methodology to calculate/estimate emissions before and after 1993.  In 1995 Vlarem II was extended with class 2 industrial plants and thresholds per pollutant. The modification of some thresholds results in the incomparability of emission data from 2004 on compared to the period before 2004 for e.g. some heavy metals.  In Wallonia, IPPC plants have had to report their emissions since 2001 and it’s sometimes difficult to make a recalculation before 2001 because of the lack of data.

- In Flanders, there is a different level of data handling in some years (1990, 1991, 1992, 1993, 1995, 1996, 1998, 2000, 2001) compared to the other years (1994, 1997, 1999, from 2001 to 2007). In the former years emissions are allocated to the correspondent IPCC code based on the SNAP code (installation level), whereas in the latter years emissions are attributed to the IPCC code at sector level (NACE code).

- Public electricity and heat production: technical measures and changeover to natural gas and use of fossil fuels with less sulphur. Emissions of waste incineration with energy recuperation and emissions of CHP installations are allocated to the electricity sector.

- Petroleum refining: decrease of SOx and NOx to meet the bubble emission thresholds for 2010 as imposed by the Flemish Government.

- Manufacture of solid fuels: decreasing emissions due to the closure of coke ovens in Brussels and Flanders, resp. in 1993 and 1996 and closure of the last Flemish mines in 1992.

- Stationary combustion in manufacturing industries: in general decreasing emission trends between 1990 and 2007 due to important efforts to reduce emissions.

- Residential sector: emissions are highly climate related. Fluctuations in emissions can also be attributed to a shift towards natural gas, the increasing number of households (with fewer persons per household), the limited isolation degree of the houses and the low compactness.

- Road transport: decrease of emissions of SOx due to the use of fuels with low sulphur content (from 2003), a significant decline in Pb emissions due to the use of unleaded petrol (from 2000), due to the enhanced application of catalytic converters NOx emissions decrease, but NH3 emissions increase.  For the same reason, PAH emissions become relatively more important due to the increase of naphthalene emissions. More stringent emission standards for diesel cars from 2005 induced lower emissions of particulate matter. In Flanders the model COPERT IV is used for the complete time series. In Wallonia, the model used before 2000 is not COPERT III but COPERT II and Myrtille.  For 2007 and 2008, Wallonia has used COPERT IV.  In Brussels COPERT III is used to calculate the emissions from road transport. These different models can lead to time series inconsistencies.  Because of the lack of data and resources, there isn’t any recalculation so far (see also 3.5.1).

- Railways: decreasing emissions due to the gradual change of diesel trains towards less polluting alternatives.

- Maritime navigation: gradual increase of emissions of most pollutants due to the expansion of the merchant fleet (number of services and magnitude of ships).

- National fishing: decreasing emissions due to the scaling down of the sector.

- Manure management: significant decreases of NH3 emissions in 1991 (Flemish Manure Decree of 23/1/1991), 2000 (MAP 2bis), 2003 (more stringent legislation) and 2007 (MAP 3, particular influence on emissions from cattle). Decrease of NH3 emissions of poultry in 2003 due to the brake-out of bird flue and the subsequent extermination of poultry by the authorities.

- Fuel combustion in agriculture: decreasing emissions (in particular emissions of heavy metals) due to the switchover towards less polluting fuels.

- Road paving with asphalt: PAH emissions decrease significantly due to a ban on the use of tar containing asphalt.

- Dioxin emissions of the metallurgical sector have decreased significantly due to emission reduction measures.

- PAH emissions of wood preservation (sector 2G) have decreased significantly due to emission reduction measures in the sector.

- Waste incineration: global emissions have decreased significantly due to the (structural) reorganisation of the sector in 1994, which included also air purification measures. Moreover, in Flanders and Brussels, the emissions of waste incinerators with energy recuperation are reported under the sector 1A1a.

# Energy (NFR sector 1)

## General methodology

To prepare the Belgian inventory for the section energy, the regional energy balances of Flanders, Wallonia and Brussels are the prime source of activity data.

The main source of information on the industrial emissions is also obtained from the annual industrial reports.

To have a total picture of all emissions by industrial activities, also activities with emissions below the threshold have to be taken into account. These emissions are estimated in a collective way. The collective estimation of the emissions due to combustion processes is done by multiplying the energy data with default emission factors.

## Default Emission factors

### SO2 and NOx

Generally in the combustion processes, the SO2 emissions are mainly based on the sulphur content of the fuel and the NOx emissions vary with the fuel and the sector.

The following table gives the default emission factor used in each region.

Table 1 : Emission factors for SO2 and NOx

|  |  |  | Flanders | Wallonia | Brussels | Flanders | Wallonia | Brussels |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel | Sector/ boiler | Unit | SO2 |  |  | NOx |  |  |
| Coal | commercial | g/GJ | 600 | 600 | 600 | 130 | 130 | 130 |
|  | residential | g/GJ | 600 | 600 | 600 | 75 | 75 | 75 |
|  | industrial | g/GJ | 546 | 600 | 600 | 110-171 | 300 | 180 |
|  | Agriculture heating | g/GJ | 648 |  |  | 300 |  |  |
| Coke oven coke | industrial | g/GJ |  | 540 |  |  |  |  |
| coke |  | g/GJnes | 552 |  |  | 110-171 |  |  |
| Natural gas | commercial | g/GJ | 0 | 0 | 0 | 50 | 50 | 50 |
|  | residential | g/GJ | 0 | 0 | 0 | 50 | 50 | 50 |
|  | industrial | g/GJ | 0 | 0 | 0 | 41-199 | 150 | 100 |
|  | Agriculture heating | g/GJ | 0 |  |  | 64 |  |  |
| blast furnace-gas |  | g/GJ |  |  |  |  |  |  |
| Coke oven gas | industrial | g/GJ | 150 |  |  | 130 |  |  |
| Fuel | industrial | g/GJ | 492 | 495 | 467 | 123-369 | 200 | 180 |
| Heavy fuel | commercial | g/GJ | 480 |  | 480 | 200 |  | 180 |
|  | Agriculture heating | g/GJ | 480 |  |  | 125 |  |  |
| Diesel oil | commercial | g/GJ | 95 | 95 | 95 | 60 | 60 | 60 |
|  | residential | g/GJ | 95 | 95 | 95 | 50 | 50 | 50 |
|  | Agriculture heating | g/GJ | 95 | 95 |  | 50 | 50 |  |
|  | Farming vehicles | g/GJ | 92 | 95 |  | 1200 |  |  |
|  | industrial | g/GJ | 94 | 95 | 95 | 49 | 70 | 180 |
| Diesel oil | Since 2008 | g/GJ | 48 | 48 | 48 |  |  |  |
| Lamp oil | commercial | g/GJ | / |  |  | 60 |  |  |
| Propane/butane/LPG | Commercial | g/GJ | 0 |  |  | 40 |  |  |
|  | Residential | g/GJ | 0 |  |  | 40 |  |  |
|  | Agriculture heating | g/GJ | 0 |  |  | 40 |  |  |
| Biogas | industrial | g/GJ |  | 288 |  |  | 88 |  |
| Renewable fuels | commercial | g/GJ | / |  |  | 45 |  |  |
| Waste gas |  | g/GJ |  |  |  |  |  |  |
| Waste | commercial | g/GJ | 130 |  |  | 150 |  |  |
| Industrial waste | industrial | g/GJ |  |  |  |  |  |  |
| Wood | residential | g/GJ | 6 | 6 | 0 | 100 | 100 | 100 |

### NH3

The following table gives the default emission factor used in each region.

Table 2 : Emission factors for NH3.

|  |  | Flanders | Wallonia | Brussels |
| --- | --- | --- | --- | --- |
| Coal | g/GJ | 0,3 | 0,4 |  |
| Coke oven gas | g/GJ |  | 0,87 |  |
| Natural gas | g/GJ |  | 0,6 |  |
| Gasoil | g/GJ |  | 0,1 |  |
| Fuel oil | g/GJ |  | 0,1 |  |
| biogas | g/GJ |  | 15 |  |
| wood | g/GJ | 3,8 | 7 |  |

### Other pollutants

The following tables give the default emission factor used in each region for CO, PM 10, PM2,5 and NMVOC.

Table  3 : Emission factors for CO, PM10, PM2,5 and NMVOC.

| Brussels EF 2007 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sector | Fuel | Units | CO | PM 10 | PM 2,5 | NMVOC |
| 1A1a | waste | g/tonne | 41,71 | 2,67 (since 2006) | 2,67 (since 2006) | 20 |
| 1A1a | Natural gas | g/GJ | 20 | 0 | 0 | 2,5 |
| 1A3c | diesel oil | g/GJ | 255 | 115,85 | 110,06 | 140 |
| 1A3dii | diesel oil | g/GJ | 255 | 92,68 | 88,04 | 100 |
| 1A4ai | coal | g/GJ | 2000 | 30 | 15 | 200 |
| 1A4ai | heavy fuel | g/GJ | 15 | 50 | 40 | 3 |
| 1A4ai | other | g/GJ | 20 |  |  | 2,5 |
| 1A4ai | diesel oil | g/GJ | 20 | 5 | 5 | 3 |
| 1A4ai | natural gas | g/GJ | 48 | 0 | 0 | 5 |
| 1A4bi | coal | g/GJ | 2000 | 30 | 15 | 200 |
| 1A4bi | wood | g/GJ | 7000 | 128,5 | 121 | 500 |
| 1A4bi | diesel oil | g/GJ | 20 | 5 | 5 | 3 |
| 1A4bi | natural gas | g/GJ | 48 | 0 | 0 | 5 |
| 1A4bi | but/prop | g/GJ | 48 | 0 | 0 | 5 |

| Flanders EF 2007 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sector | Fuel | Units | CO | PM10 | PM2,5 | NMVOC | Pb | Cd | Hg |
| 1A1a | natural gas | g/GJ | - | - | - | 1 | - | - | - |
| 1A1a | fuel A | g/GJ | - | - | - | 2,3 | - | - | - |
| 1A1a | gasoil | g/GJ | - | - | - | 7,5 | - | - | - |
| 1A1a | coal | g/GJ | - | - | - | 0,5 | - | - | - |
| 1A1a | fuel D | g/GJ | - | - | - | 2,3 | - | - | - |
| 1A1a | Bipol | g/GJ | - | - | - | 2,3 | - | - | - |
| 1A1a | HO | g/GJ | - | - | - | 0 | - | - | - |
| 1A1a | H2 | g/GJ | - | - | - | 0 | - | - | - |
| 1A1a | bisphenol | g/GJ | - | - | - | 2,3 | - | - | - |
| 1A1a | fuel C | g/GJ | - | - | - | 2,3 | - | - | - |
| 1A1a | blast furnace gas | g/GJ | - | - | - | 0 | - | - | - |
| 1A1a | coke oven gas | g/GJ | - | - | - | 0 | - | - | - |
| 1A1a | refinery gas | g/GJ | - | - | - | 8 | - | - | - |
| 1A1a | sludge | g/GJ | - | - | - | 10 | - | - | - |
| 1A1a | olive stones | g/GJ | - | - | - | 10 | - | - | - |
| 1A1a | wood dust | g/GJ | - | - | - | 10 | - | - | - |
| 1A1a | pellets | g/GJ | - | - | - | 10 | - | - | - |
| 1A1a | coffe grounds | g/GJ | - | - | - | 10 | - | - | - |
| 1A1a | wood chips | g/GJ | - | - | - | 10 | - | - | - |
| 1A1a | biofuel | g/GJ | - | - | - | 2,3 | - | - | - |
| 1A1a | bio dust | g/GJ | - | - | - | 10 | - | - | - |
| 1A3c |  | g/GJ | 324,6 | 27,6 | 26,2 | 56,8 | - | 0,0002 | - |
| 1A3dii |  | g/GJ | 215,0 | 36,6 | 34,7 | 45,5 |  | 0,0002 |  |
| 1A4ai | coal | g/GJ | 4600 | 30 | 15 | 200 | 0,130 | 0,0015 | 0,0051 |
| 1A4ai | other | g/GJ |  | - | - | 20 | - | - | - |
| 1A4ai | renewables | g/GJ | 25 | - | - | 500 | 0,000984 | 0,000515 | 0,000234 |
| 1A4ai | fuel | g/GJ | 40 | 5 | 5 | 3 | 0,0041 | 0,0014 | 0,0014 |
| 1A4ai | heavy fuel | g/GJ | 40 | 49,8 | 40,2 | 3 | 0,016 | 0,0003 | 0,0001 |
| 1A4ai | natural gas | g/GJ | 25 | 0 | 0 | 2 | 0,000984 | 0,000515 | 0,000234 |
| 1A4ai | LPG | g/GJ | 25 | 0,2 | 0,2 | 2 | 0,000984 | 0,000515 | 0,000234 |
| 1A4ai | lamp petrol | g/GJ | 40 | 5 | 5 | 3 | 0,0041 | 0,0014 | 0,0014 |
| 1A4bi | coal | g/GJ | 4600 | 30 | 15 | 200 | 0,13 | 0,0015 | 0,0051 |
| 1A4bi | biomass/renewable | g/GJ | 5300 | 128,25 | 121,5 | 345/1145* | 0,04 | 0,0014 | 0,0005 |
| 1A4bi | fuel | g/GJ | 46 | 5 | 5 | 3 | 0,0041 | 0,0014 | 0,0014 |
| 1A4bi | natural gas | g/GJ | 31 | 0 | 0 | 2 | 0,000984 | 0,000515 | 0,000234 |
| 1A4bi | LPG | g/GJ | 31 | 0,2 | 0,2 | 2 | 0,000984 | 0,000515 | 0,000234 |

* 345: stoves; 1145 fireplaces

| Wallonia EF 2007 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CO | PM10 | PM2.5 | NMVOC |
| 1A1a | STEAM COAL | 15 | Plant specific |  | 5 |
| 1A1a | RESIDUAL OIL | 15 | 3 | 2.5 | 10 |
| 1A1a | GAS OIL | 10-15 | 2-5 | 2-5 | 5-15 |
| 1A1a | NATURAL GAS | 12-20 | 0,1 | 0,1 | 2.5-5 |
| 1A1a | BIOGAS | 13 | 0 | 0 | 2,5 |
| 1A4ai | STEAM COAL | 2000 | 30 | 15 | 200 |
| 1A4ai | WOOD AND SIMILAR WOOD WASTES | 5000 | 95 | 90 | 600 |
| 1A4ai | GAS OIL | 20 | 5 | 5 | 10 |
| 1A4ai | NATURAL GAS | 50 | 0 | 0 | 5 |
| 1A4ai | LIQUEFIED PETROLEUM GASES (LPG) | 50 | 0 | 0 | 5 |
| 1A4ai | BIOGAS | 13 | 0 | 0 | 2,5 |
| 1A4bi | STEAM COAL | 2000 | 30 | 15 | 200 |
| 1A4bi | BROWN COAL BRIQUETTES | 2000 | 30 | 15 | 200 |
| 1A4bi | COKE OVEN COKE FROM HARD COAL | 2000 | 30 | 15 | 200 |
| 1A4bi | WOOD AND SIMILAR WOOD WASTES | 5000 | 128,5 | 121 | 600 |
| 1A4bi | GAS OIL | 20 | 5 | 5 | 15 |
| 1A4bi | NATURAL GAS | 50 | 0 | 0 | 2,5 |
| 1A4bi | LIQUEFIED PETROLEUM GASES (LPG) | 50 | 0 | 0 | 5 |

## Energy industries (category 1.A.1)

### Public electricity and heat plants (category 1.A.1.a)

This category contains the power installations for the production of electricity and heat and the combined heat-power installations (in joint venture with the electricity producers). These installations are located in different sectors in Belgium (industry, agriculture and service sector). Also included in this sector are the waste incineration installations with energy recuperation for Flanders and Brussels (waste incineration installations without energy recuperation are allocated in the sector 6C waste incineration). For Wallonia, these emissions are still included in the sector 6C but it will be changed for the next submission.

#### SO2 and NOx

The number of companies is limited and well known. For the large power plants in the public electricity sector, the SO2 and NOx emissions that are reported are directly given by the power plants and based on continuous analyses in the chimneys.

#### NH3

In the Walloon region, the NH3 emissions are estimated by using the default emission factors (table 2).

### Petroleum refining (category 1.A.1 b)

The activity data of the petroleum refining are taken over from the Flemish energy balance as petroleum refining only occurs in Flanders.

SO2 and NOx are directly reported by the refineries. SO2 emissions are calculated based on the sulphur content of the fuel. NOx emissions are calculated with emission factors provided by the refineries or emissions are calculated based on measurements.

### Manufacture of solid fuels and other energy industries  (category 1.A.1.c)

The emissions originating from category 1.A.1.c ‘Manufacture of Solid Fuels and Other Energy Industries’ are the emissions coming form the combustion in the coke ovens.

In  Wallonia, in the category 1.A.1.c the emission factors for SO2 are based on the sulphur content of the fuel.  The emission factor for NOx for the coke gas is given directly by the power plant. The non diffuse NH3 emissions are estimated by using the default emission factors (table 2).

There was a coke plant in the Brussels region until 1993. The emission factors used, are the same as the ones used in Wallonia .

In Flanders the last coke plant has closed in 1996. The emission factors for SO2 are based on the sulphur content of the fuel. The emission factor for NOx is based on literature data.

## Manufacturing industry and construction (category 1.A.2)

### General

Pollutant emissions are mostly reported directly by the individual large companies on the basis of analysis. For most sectors the remainder of the emissions is calculated on the basis of the remaining fuel consumption (estimated as the difference between energy consumption reported in the regional energy statistics for the whole sector and the fraction reported by the large companies) and standard emission factors listed in tables above.

The energy consumption data originate from the regional energy balances in the 3 regions, supplemented with specific information from the companies themselves, for example activity data from iron and steel industry.

In the Walloon region, the NH3 emissions are estimated by using the default emission factors (table 2).

### Iron and steel sector

The SO2 and NOx emissions from the iron and steel sector are partly put in category 1.A.2.a (energetic part / except for the emissions from the cokes ovens which are allocated in the category 1.A.1.c) and partly in category 2.C.1 (process part).

## Transport (category 1.A.3 and 1.A.5.b)

### Road transport

Emissions of NH3, SO2 and NOx are calculated by compiling the emissions of each region based on the use of the specific models used in the 3 regions (based on COPERT IV in the Walloon and the Flemish regions and based on COPERT III in the Brussels region for the year 2007).

Before 2007, the Walloon region had used different models to estimate these emissions (myrtille, COPERT III).  There weren’t recalculations because of the lack of time and available data.  A recalculation will be made for the next submission to calculate road transport emissions with COPERT IV from 1990 to 2008.

In Flemish Region MIMOSA 4 (a COPERT IV based model) is used to calculate the emissions for the complete - time series 1990-2008.

The Brussel-Capital region uses COPERT III to estimate the emissions for the complete time series. Recalculations for some key years are planned for future submissions.

### Air transport

In the two regions (Flemish and Walloon region) where air transport is relevant, a slightly different approach was taken in estimate the emissions from air transport.

In Flanders, emissions of civil aviation are calculated for Landing and Take-Off cycles (LTO) based on flight data and emission factors.  These emissions are calculated for 4 airports for civil aviation (Antwerp, Ostend, the international airport of Brussels-National and from 2008 on also Kortrijk-Wevelgem) and for 5 airports for military aviation (Kleine Brogel, Brasschaat, Koksijde, Sint-Truiden and Goetsenhove). The airport of origin and airport of destination are known, a division in domestic/international traffic can be made.

In Wallonia, the emissions from civil aviation (category 1.A.3.a) are estimated on the basis of the number of LTO in two airports (Liège and Charleroi), following a very simple methodology described in the EMEP/EEA guidebook. Some information on the country's total number of LTO is available. Unfortunately, there is no general knowledge about the aircraft types carrying out the aviation activities. 58 % of the LTO are supposed to be domestic flights. A distinction is also made between emissions from domestic and international LTO and cruise activities. Emissions factors used to estimate emissions from domestic and international traffic are based on the table 8.2 in the EMEP/EEA guidebook. The emissions from domestic LTO and cruise activities are reported under the category 1.A.3.a (civil aviation), while emissions from international LTO and cruise activities are reported under "international bunkers : aviation". Kerosene used in international air transport is assigned to the bunker fuels. Data on LTO activities and fuel consumption in the airports of Charleroi and Liège come from the statistics of these airports.

### Railways

The railway traffic is not estimated in the same way in the 3 regions.

Flemish emissions of railway traffic are estimated for the first time (submission 2009) by the EMMOSS model. The basis for the calculations is gross ton kilometers driven by trains. The formula for calculation is : emission (g) = gross ton kilometers (ton km) x specific end-energy use (kWh/ton km) x emission factor (g/kWh). Emission factors are derived from ISO 8178/F test cycles for CO, fuel consumption, NOx and TSP. Emissions for C6H6, NH3, N2O PAK en CH4 were taken over from Klein, 2006 (The Netherlands).  SO2 and CO2 are fuel-specific (SO2 calculated dependent on content of S in fuel). Emissions for shunting trains are also calculated.

The emissions from railways are estimated in the Walloon region and in the Brussels region by multiplying the train’s fuel consumption by the fuel specific emission factors.  The SO2 emission factors are the factors in table 1. The NOx emission factors are those described in table 8.1 of the EMEP/EEA guidebook.

In Wallonia and Brussel, the data from the National Society of the Belgian Railways (NMBS) are used to calculate the energy consumption for the train services in Belgium. These data are available for the transport of persons and goods and for electricity and gasoil driving. The total consumption of gasoil in the Walloon region is based on the Belgian data of gasoil consumption and the regional information on driven train- and ton-kilometers of persons and goods. The emissions are estimated by multiplying the train’s fuel consumption by the fuel specific emission factors.

### Navigation

For navigation, fuel consumption is taken from the regional energy balances.

In Flanders, emissions from maritime navigation are calculated (for the first time in 2007)  with the emission model EMMOSS;

The emissions originating from inland shipping are estimated for the first time (submission 2009) by the EMMOSS model. As a consequence the whole time series is adapted compared to previous submissions. In the two other regions, emissions are calculated using emission factors in table 1 for SO2 and emission factors of the EMEP/EEA handbook for NOx.

## Other sectors (category 1.A.4)

The fuel consumption of the service sector is based on general statistics of natural gas, supplemented with results from surveys for solid and liquid fuels. Agricultural fuel consumption is estimated from statistical information concerning area used, etc., combined with specific energy consumption from literature. The energy consumption of these sectors is published in the regional energy balances.

In Flanders, emissions by heating systems of buildings are calculated on a collective basis. The database consists of emissions due to residential combustion (heating by households) and tertiary combustion (heating by hotels/restaurants, medical services, education, offices and administrative activities, trade, other services and combined heat-power installations (CHP)). Emissions are calculated by multiplying the energy use and emission factors.

Data on energy used can be found in the Energy Balance for Flanders. A provisional energy balance is made yearly for year (x-1), whereas a final energy balance is made for year (x-2). For households energy data on electricity use, natural gas, fuel, solid fuels (coal), propane/butane/LPG, other (mainly wood) and renewable fuels are included. The tertiary sector contains energy data on fuel, heavy fuel, solid fuels (coal), propane/butane/LPG, electricity, other (mainly waste) and renewable fuels, for some years also lamp petrol. A distinction is made between autoproducers and non-autoproducers. For the CHP installations in the tertiary sector energy information on natural gas, fuel and other fuels (biogas) is included. For your information the worksheets, used to calculate the emissions of the households, the tertiary sector and the CHP installations are included (1 sheet per year). The origin of the emission factors used is clarified in the sheets. The emission factors to calculate particulate matter, Pb, Cd, Hg and NMVOC originate from studies, SOx  emission calculations are based on the S-content of the fuels (for SOx and NOx, the emission factors are harmonised between the regions), other emission factors (CO and NH3) are taken from the EMEP/EEA Emission Inventory Guidebook. In 2009 emission factors are re-examined as a result of the release of the revised Guidebook. The emission factors are only adapted when expert analysis reveals that better factors are available or when tuning with the other Belgian regions occurs.

In Wallonia, the main data source for this sector  is the energy balance delivered yearly by the Energy and Sustainable Building Department. The energy balance describes the quantities of energy imported, produced, transformed and consumed in the Walloon Region in a given year. The energy consumption of the household sector is calculated on the basis of regional data on the amount of natural gas and electricity sold in this sector (CWaPE), on the basis of national data (liquid fuels and solid fuels), on the basis of the socio-economic survey of 2001 (size, isolation,…) and on the basis of weather data (degree-days). The energy consumption in the service sector is calculated using the energy data of different sources (regional data on the amount of natural gas and electricity sold in this sector (CWaPE), annual survey carried out by ICEDD for all consumers “high voltage” (4800 establishments with a respond of 58 %).

In Brussels, the consumption of the tertiary is based on two approaches : one for the “high voltage customers” and the other for the “low voltage customers”. For “high-voltage customers”, the energy consumption is calculated using the energy data based on a survey and direct contacts with “high voltage” consumers  and big international public organisms. For “low-voltage customers”, the energy consumption is calculated by the “top down” method and the consumption of oil-products is estimated from the fuel/naturel gas ratio and the Belgian consumptions.

Emission factors for SO2 and NOx are listed in table 1 and emission factors for NH3 are listed in table 2.

In Wallonia, emissions from farming vehicles are calculated using emission factors in table 1 for SO2 and emission factors of the EMEP/EEA handbook for NOx.

## Other (category 1.A.5.a and 1.A.5.b)

In this section the emissions originating from the military transport (domestic air transport) are included (category 1.A.5.b).

In Wallonia, the emission factors are those described in table 8.8 of the EMEP/EEA guidebook by using the Netherlands emission factors (nature of flight : average).

In Flanders, the emissions of military aviation are estimated based on plane movements and emission factors.

No activities under category 1.A.5.a take place in Belgium.

## Fugitive emissions from fuels (category 1.B.1 and 1.B.2)

### Solid fuel transformation (category 1.B.1.b)

Emissions during the coke production are caused by the loading of the coal into the ovens, the oven/door leakage during the coking period and by extracting the coke from the ovens.

Activity data (tons of cokes) are delivered by the corresponding industry.

In Wallonia and in Brussels (closed plant), the emission factor is 21 g SO2/t and 480 g NOx/t. In Flanders no fugitive SO2 and NOx emissions are estimated.

In Wallonia, the NH3 emission factor is 138 g/t (EPA).

### Petroleum refineries (category 1.B.2.a)

Estimation of the emissions of SO2 and NOx from the sector petroleum refining is generally provided by the companies based on monitoring results or emission factors.

## International bunkers (category Memo Items – International Bunkers)

Marine bunkering comes from the Belgian energy statistics.

For international air transport, bunkers fuel consumption of the international air transport is given directly by the two Walloon airports. For the airports in Flanders, the reported kerosene fuel is assigned to the bunker fuels and all gasoline for air transport is allocated to domestic air transport.

# Industrial processes (NFR sector 2)

## Overview

### Description of the sector

The structure of the industrial sector has undergone profound changes over recent decades. The mining industries have practically disappeared with the closure of the last coalmines. The metallurgy and textile sectors have been relatively stable, after several waves of closures and restructuring. The metallurgy industry nevertheless remains one of the key sectors of Belgian industry, both in terms of employment and turnover. The two other key sectors of industrial activity are the chemical industry and the food processing industry, which contribute respectively 3.8% and 2.5% to the GDP.

In this sector of industrial processes the emissions of industrial activities which are not related to the combustion of fossil fuels are included.

### Allocation of emissions

The industrial processes in Belgium are covered by

- categories 2.A.1 (cement production) and 2.A.2 (lime production), activities which take place only in the Walloon region and category 2.A.7 (production of glass and ceramics), activities which take place in the Flemish and the Walloon region;

- categories 2.B.1 (ammonia production), 2.B.2 (nitric acid production), these activities take place both in the Flemish and the Walloon regions and category 2.B.5 other industrial in the chemical industry in the Walloon region i.e. the production of maleic anhydride and in the Flemish region i.e. the production of caprolactam and other process emissions reported by the chemical industry (e.g. the production of ethylene oxide, acrylic acid, …);

- category 2.C.1 (metal production i.e. iron and steel industry), these activities take place both in the Flemish and the Walloon regions.

## Methodological issues

The main process emissions of NH3, SO2 and NOx are calculated in Belgium by using production figures, mainly directly originating from the industrial plant, combined with emission factors presented in reference works like CITEPA, EMEP/EEA handbook, IPCC Guidelines or other specific bibliographies or calculated via measurements carried out by the industrial companies.

### Mineral products (category 2.A)

The mineral industry is one of the most important sectors of industrial process emissions in Belgium.

In Belgium, cement production (category 2.A.1) only takes place in Wallonia. The activity data is the clinker production collected directly from individual plants.

The emissions are estimated by using a plant-specific emission factor (NH3, SO2 and NOx). The emissions are the sum of combustion and process emissions.

An average emission factor by plant has been estimated in 2002 and is applied on the all time-series 1990-2001.  Since 2002, the emission factor varies each year and has been calculated directly by the plant.

Production of lime also occurs only in the Walloon region of Belgium.

These emissions of lime production (category 2.A.2) are estimated by using a plant-specific emission factor for SO2 and NOx .  The NH3 emission factor is 5,1g/t (National Pollutant Inventory in Australia).

The activity data is the lime and dolomite lime production and is collected directly from individual plants.  The emission factors are also collected directly from individual plants. The emissions are the sum of combustion and process emissions.

The production of glass (category 2.A.7)  in Belgium takes place in the Flemish and in the Walloon regions.

Both in the Flemish and the Walloon region, all the emissions are given by the plant and are in the combustion part.

### Chemical industry (category 2.B)

The chemical industry is the most important sector in industrial processes in Belgium.

#### Ammonia production (category 2.B.1)

Nowadays there is ammonia production in 2 companies in Belgium.

In Flanders the emissions of SO2 and NOx originating from the production of ammonia are obtained by monitoring results.

In the Walloon region, the producer of ammonia provides the NOx emissions based on their production and on monitoring.

#### Nitric acid production (category 2.B.2)

In Flanders the emissions of SO2 and NOx originating from the production of nitric acid are obtained by monitoring results. Since 2000 only one plant is still involved in this sector.

The producer of nitric acid in the Walloon region provides the NOx emissions based on their production and on monitoring.  There are three installations on the plant. There is only one installation with an abatement technology (SCR) installed in 1996 which lead also in a strongly increasing of the production in 1996.

Although the closure of 2 plants in the Flemish region, in 1995 and in 2000 respectively, the production of nitric acid still increases. NOs emissions are also provided by the plant involved and based on measurements.

#### Other production (category 2.B.5)

In Wallonia and Flanders the emissions of NH3 originating from chemical production are obtained by monitoring results. The plants involved are producers of polyvinylchloride, ammonium nitrate, ammonium phosphate and NPK fertilisers.

### Metal production (category 2.C)

Metal production, more specific the iron and steel production (category 2.C.1.) is the third most important sector of industrial process emissions in Belgium. These activities are situated in the Flemish and the Walloon regions .

In Flanders, the process emissions from iron and steel production are based on monitoring results provided by the companies.

In the Walloon region, iron is produced through the reduction of iron oxides (ore) with metallurgical coke (as the reducing agent) in a blast furnace to produce pig iron. Steel is made from pig iron and/or scrap steel using electric arc or basic oxygen.

The emission estimates in this sub-sector include also emissions from the production of steel in basic oxygen type furnaces but not emissions from the combustion of the fuel.

The emissions from electric arc furnaces include all the emissions (combustion and process).

Table 4 : Emission factors used in the iron and steel sector in the Walloon region (Source: plant specific /ULG)

|  | SO2 (g/t) | NOx (g/t) |
| --- | --- | --- |
| Pig iron tapping | 63 |  |
| Basic oxygen furnace steel plant | 100 | 50 |
| Electric furnace steel plant | 180 | 100 |

In the sinter plants, the emissions calculated involve combustion and process emissions and are reported in the energy sector.

# Solvent and other product use (NFR sector 3)

In Belgium the emissions of NMVOC in the source category ‘Solvent and other product use’ include paint application (building industry & households except for Brussels), production of medicines, paints, inks and glues, domestic use of other products (incl. glues and adhesives), coating processes in general (incl. assembly of automobiles), printing industry, wood conservation, treatment of rubber, storage and handling of products, recuperation of solvents, extraction of oil, cleaning, degreasing and dry cleaning.

The regions in Belgium are using comparable methodologies to estimate the emissions of solvent and other product use in their region.

The emissions of NMVOC in Flanders are estimated by using the results of a study started by the University of Gent in 1998 and continued by the Flemish Environment Agency (VMM).

In Wallonia, the calculation is based on a methodology established by Econotec.

In the Brussels region, the emissions are calculated by using different sources : average emission factors, surveys and information collected from the sector.

# Agriculture (NFR sector 4)

## Data sources

The main activity data are the livestock figures and agricultural land area. The National Institute of Statistics (NIS) [26] publishes these numbers yearly. 75% of all agricultural businesses have to fill in a form each year about the situation on the 1st of May of that year and send it to the National Institute of Statistics (NIS).The remaining 25% has been asses and is asked to fill in a form the next year. Like this every two years 100% of the agricultural businesses is surveyed. Further details on the agricultural census methodology and QA/QC issues can be found on the NIS website: www.statbel.fgov.be.

With an average temperature of 10.9°C in 2008 (www.meteo.be), Belgium as a whole has a "cool" climate.

In Flanders, since 2009 a new model is used to calculate the NH3-emissions from animal manure for each stable in Flanders. This model (Emission model ammonia Flanders (EMAV)) is developed by the Institute for Agricultural and Fisheries Research (the so-called ILVO). The model takes into account four emission stages: stable, outdoor/indoor storage of manure, manure application to land and emissions from grazing animals. From 2000 on input data as animal number, N-production, type of emission poor stable, manure transport, agricultural zone, … are obtained by the Manure Bank of the Flemish Land Agency (VLM; www.vlm.be/algemeen/diensten/Mestbank). This information is available on the level of the stable. The factors used in this model are updated yearly if necessary. For the period 1990 to 1999 this detailed information was not available with the Manure Bank, therefore the data from the NIS is used. This information is available on the level of the municipality.

The nitrogen excretion factors in Flanders of most of the animal categories are used as described in the Manure Action Plans (MAP2bis till 2006 and from 2007 on MAP3; http://www.vlm.be/SiteCollectionDocuments/Publicaties/mestbank/Normen_2009.pdf). For the N-excretion factors of swine and poultry, a farmer can choose to use the standard excretion factors. Or they can choose (or in some cases are obliged) to use the other systems (regression, animal feed covenant, input-output balance).

Since September 2003 all new built stables for poultry and swine need to be constructed in an ammonia emission-poor way. The licensed stables are inventoried by the manure bank of the Flemish Land Agency. Based on the amount of ammonia emission-poor stables, the stable emission factors for swine and poultry are yearly adjusted.

In Wallonia, the emissions are calculated using a model developed by a consultant agency Siterem. Different data are used as input in the model which then calculates CH4, N2O and NH3 emissions. Four emissions sources were pointed: animal husbandry, the excreta of agricultural animals deposited in buildings and collected as either liquid slurry or solid manure, application of animal manure to land and mineral fertilisers.

In Wallonia nitrogen excretion  factors were first determined for the implementation of the CE Nitrates Directive 91/676 (see annexes of the decree downloadable  on http://www.nitrawal.be/pdf/arretenitrates_mb2.pdf, but were representing the nitrogen after deduction of the atmospheric losses, so new factors were calculated on this basis for the purposes of estimating atmospheric emissions.

The data on sludge spreading on agricultural soils are available on the website of DGRNE (http://www.environnement.wallonie.be/).

The allocation of animals to animal waste management system (AWMS) comes from the NIS agricultural census of 1992 and 1996, where those data were collected by animal type. Those data are not collected on a yearly basis by the NIS given their slow pace of change. However, an update of the 1996 data would likely be useful in the near future. So far we have no information about the NIS planning regarding this update.

7.3.2. Livestock (category 4B)

The NH3 emission estimation from livestock is based on the amount of gross nitrogen excreted by each animal category, estimated through local production factors (see 7.3.1). The calculation takes into account the different stable types, the number of days in pasture, the different manure management systems, the manure application on land.

The N-excretion factors, as well as the allocation of animals to AWMS in Wallonia are presented in table 5.

|  | Population Size | Nex (kg/head.yr) | Solid storage | liquid storage | % stable vs pasture |
| --- | --- | --- | --- | --- | --- |
| Bovines under 6 months | 188283 | 13,4 | 87% | 13% | 100% |
| Bovines between 6 months and 1 year: male | 46715 | 37,5 | 90% | 10% | 50% |
| Bovines between 6 months and 1 year: female | 94715 | 30,8 | 87% | 13% | 50% |
| Bovines more than 1 year for fattening: male | 62015 | 97,8 | 87% | 13% | 50% |
| Bovines more than 1 year for reproduction: male | 18083 | 84,4 | 77% | 23% | 50% |
| Bovines more than 1 year: female | 352882 | 58,9 | 77% | 23% | 50% |
| Dairy cows | 217948 | 120,5 | 56% | 44% | 56% |
| Brood cows | 329196 | 97,8 | 91% | 9% | 50% |
| Piglet under 20 kg | 57335 | 4,7 | 25% | 75% | 100% |
| Piglet between 20 and 50 kg | 85428 | 10,4 | 25% | 75% | 100% |
| Fattening pigs more than 50 kg | 214166 | 16,1 | 25% | 75% | 100% |
| Sows | 16150 | 37,5 | 42% | 58% | 100% |
| Breeding males | 4460 | 42,9 | 43% | 57% | 100% |
| Lambs | 6759 | 4,4 | 100% | 0% | 100% |
| Sheep <1 year | 13268 | 4,4 | 100% | 0% | 50% |
| Sheep >1year | 29085 | 8,8 | 100% | 0% | 50% |
| Goats <1year | 2617 | 4,4 | 100% | 0% | 50% |
| Goat > 1 year | 8127 | 8,8 | 100% | 0% | 50% |
| Horses | 13487 | 75,0 | 100% | 0% | 50% |
| Broilers | 3596008 | 0,4 | 89% | 11% | 100% |
| Laying hens | 1491276 | 0,8 | 6% | 94% | 100% |
| Other poultry | 211698 | 0,6 | 26% | 74% | 100% |

Table 5.: Animal number, nitrogen excretion factors and allocation of animals to AWMS for each category in Wallonia (2008).

As the EMAV-model in Flanders uses input data, as animal number en N-production, on the level of the farm, these data for year X can only be obtained on this level of detail at the end of the year X+1. Only the total animal number in Flanders for 2008 was available on time for this reporting. Therefore in table 6 an overview is given of the animal number and derived nitrogen excretion factors as used in Flanders for the calculation of the NH3-emission in 2007. NH3-emission in 2008 is taken the same as in 2007.

| Category | Population Size | Nex (kg N/head.yr) |
| --- | --- | --- |
| Bovine |  |  |
| Slaughter calves | 169463 | 10,5 |
| #### Bovine under 1 year | 103004 | 23 |
| Bovine under 1yr for replacement | 173819 | 33 |
| Bovine from 1 to 2 year | 96413 | 61 |
| Bovine from 1 to 2yr for replacement | 175450 | 56 |
| Bovine more than 2 year | 150456 | 77 |
| Dairy cows | 255260 | 97 |
| Brood cows | 175413 | 97 |
| Sheep |  |  |
| Sheep under 1 year | 24612 | 4,36 |
| Sheep more than 1 year and goats | 38239 | 10,5 |
| Swine |  |  |
| Piglet from 7 to 20 kg | 1511821 | 2,46 |
| Fattening pigs from 20 to 110 kg | 3763271 | 11.39 |
| Fattening pigs more than 110 kg | 78745 | 21.36 |
| Boars | 7233 | 21,54 |
| Sows including piglets less than 7 kg | 470614 | 21.52 |
| Horses |  |  |
| Horses and pony less than 200 kg | 6815 | 35 |
| Horses and pony from 200 to 600 kg | 26370 | 50 |
| Horses more than 600 kg | 6345 | 65 |
| Rabbit motheranimal | 19235 | 8.64 |
| poultry |  |  |
| Broilers (for breeding) | 612937 | 0.46 |
| Broilers (for fattening) | 15262549 | 0.62 |
| Laying hens (for breeding) | 2283180 | 0,36 |
| Laying hens | 7238618 | 0.67 |
| Ostriches | 1593 | 10.61 |
| Turkeys | 169663 | 2.01 |

Table 6.: Animal number and nitrogen excretion factors for each animal category in Flanders (2007).

For information, there is a significant decrease in NH3 emissions from 4.B.1a, 4.B.1b and 4.B.8 between 1999 and 2000 due to the implementation of the Manure Action Plans in Flanders.

In 1991 there was the first Manure Decree. One of the items of this Decree was the reduction of the period (months) in which manure can be applied to land. This had a minor impact on the NH3 emissions.

The Manure Action Plan2bis in 2000 focussed on the reduction of the manure surplus. Among others this was done by reducing the amount manure that can be applied to land and furthermore the obligation to apply manure in an emission poor way. Also the regulation concerning manure processing had an impact on the NH3-emission.

In 2003 all new stables for swine and poultry have to constructed in an emission poor way.

The NH3-emission trend shown in the figure, reflects the implementation of these successive MAP’s (1991, 2000, 2003).

## Direct soil emission (category 4D)

In Wallonia, the NH3 emissions from mineral fertilisers account for the nitrogen volatilised as NH3 and N2O. The model uses a volatilisation rate (FracGASF) from mineral nitrogen to NH3 and N2O. In Wallonia the average rate of 2.3 % is based on the default values recommended by IIASA for different types of fertilisers

In Flanders, the NH3-emission from fertiliser use is calculated using the same EMAV-model in which the area of a certain agricultural zone is multiplied with the respective fertiliser use (kg N/ha) and emission coefficient. Depending of the type of mineral fertiliser, a different emission coefficient is used. The different types of mineral fertiliser used in Belgium originate from the International Fertilizer Industry Association (IFA). The emission factor (%) in table 7 is the average of all emission coefficients.

For the amount of fertilizer use the Department Agriculture and Fishery and the Institute for Agricultural and Fisheries Research conducts surveys on a representative sample of the different types of agricultural businesses and produces yearly weighted average values on the fertiliser use, taking into account the manure pressure [50].

|  | Synthetic fertilizer use (kg N /ha) | Synthetic fertilizer use (ton N) | Emission Factor (%) |
| --- | --- | --- | --- |
| Wallonia | 96 | 71865 | 2.3 |
| Flanders |  |  |  |
| Dunes | 127.48 | 266 | 4.06 |
| Kempen area | 79.92 | 9633 | 4.06 |
| Sandy area | 107.63 | 16429 | 4.06 |
| Sand Loam area | 104.25 | 21152 | 4.06 |
| Loamy area | 134.24 | 5482 | 4.06 |
| Polders | 127.48 | 6966 | 4.06 |
| Luikse Meadow area | 97.16 | 318 | 4.06 |

Table 7. : The amount (kg N) of synthetic fertilizer use (per ha) (2008) and the emission factor used in each Region.

In both regions, the ammonia emission from grazing is estimated, taking into account the number of days in pasture, the nitrogen excreted by each animal category, and the EEA emission factor of 8%. In table 8 an overview is given of the different factors used in both regions and for the different grazing animal categories.

|  | Days in pasture (% of year) | Nitrogen excreted on pasture (ton) | Emission factor (%) |
| --- | --- | --- | --- |
| Flanders |  |  |  |
| Female bovine | ± 50 | 27541 | 8 |
| Sheep | 80 | 408 | 8 |
| Horses | 50 | 986 | 8 |
| Wallonia |  |  |  |
| Bovine | 50 | 45488 | 8 |
| sheep | 50 | 158 | 8 |
| horses | 50 | 507 | 8 |

Table 8 : The days in pasture (%/yr), nitrogen excreted on pasture (ton) and the emission factor used for each grazing category in 2008.

# Waste (NFR sector 6)

## Solid waste disposal on land (category 6A)

The NMVOC and PM&TSP emissions from land disposal of solid waste are calculated by the Flemish region.

The methodology applied to estimate NMVOC emissions is the same methodology as the one used to calculate the CH4-emissions (see description in NIR of March 2010, Chapter 8.2.). Just one factor is different: biogas contains 2420 ppm NMVOC (reference: D’Haene V. et al., 2002, a study performed by the University of Gent by order of the Flemish Environment Agency].

The emissions of PM2.5 and PM10 are due to smoking of tobacco in Flanders. It should be considered however for the following submission to allocate these emissions in a more appropriate sector.  Emissions are calculated by multiplying the tobacco consumption in Flanders (calculated from the Belgian tobacco consumption, taking into account the number of households and the average spending, www.statbel.fgov.be) and an emission factor of 40 kg/ton for TSP, PM10 and PM2.5 (TNO, 2001). The methodology is described in detail in a study performed by VITO under the authority of VMM (Schrooten, L., Van Rompaey, H. Berghmans, P., Vanderreydt, I. & Bleux, N. (2002). Deel A: Emissie-inventaris fijn stof Vlaanderen voor 1995 en 2000. Deel B: Monitoring fijn stof emissies voor industriële bronnen. 2002/IMS/R/200).

No waste disposal sites are located in the Brussels-Capital region in Belgium.

## Wastewater treatment (category 6B)

In Belgium, emissions originating from sceptic tanks are estimated by multiplying the emission factor (an NH3 emission factor of 750 g/person is used) by the number of inhabitants not connected to a municipal wastewater treatment plant.

.

## Waste incineration (category 6C)

Waste incinerators

In Wallonia, the total emissions from the waste incinerators are based on data collected through environmental reporting by the individual plants. The only hospital waste incinerator has closed since 2005.

Since 2000, more precise data have been reported by the waste incineration plants. The four municipal waste incineration plants produce electricity.  Regarding dioxins, a continuous measurement has been put in place for the 4 incinerators : http://environnement.wallonie.be/data/air/dioxines/menu/menu.htm

All these waste incinerators in Wallonia produce electricity, consequently the allocation of these emissions should be the category 1A1a and will be corrected in the next submission.

In Flanders (since 2006), all the municipal waste incineration plants produce electricity.

The plants are obliged to report their emissions yearly in an emission report. These data are integrated  in the emission inventory. The emissions are allocated in the category 1A1a.

In Brussels, the last waste incinerator without recuperation of energy has closed in 1998.

Cremation

For Flanders : the Activity data is 29877 cremations in 2007 (derived from the yearly statistics of crematoria), an emission factor of  0,0504 µg TEQ/cremation is used (Results of measurements made by the Flemish government – Department Leefmilieu/Afdeling Milieu-inspectie).

For Wallonia: the emission factor from the Netherlands of 4µg TEQ/corpse, reported in EMEP (Emission Inventory Guidebook December 2006 B991-9) is applied . The number of corpses is available here :  http://www.crematorium.be/statistiek-2/?lang=

For Brussels, the emission factor for dioxins is 7.9 µgTEQ/cremation. The number of cremations is directly collected in the sector.

Open combustion of waste.

Only Flanders estimate these emissons. To make the calculcation, it is assumed that 5% of the average amount of municipal waste is burnt in open barrels (Van Rompaey, H., De Fré, R. De Spiegeleer, E., Polders, C., Vanderstraeten, P. & Wevers, M. (2001). Emissies van dioxins en PAK’s door gebouwenverwarming met vaste brandstoffen, AMINAL, VITO). The amount of municipal waste per household can be found on the website of the Public Flemish Waste Company (www.ovam.be). The number of households can be found on www.statbel.fgov.be.  The emission factor of TSP is taken from Wevers, M. (2002). (Meetcampagnes dioxines en PAK’s: emissies door open vuren, open tonnetjes, kachels en allesbranders en analyses van roetstalen uit schouwen. AMINAL/Aminabel, VITO). Emission factors for PM10 and PM2.5 are taken from TNO (2001) (for reference see 6A 3°).  Dioxin and PAH’s emissions of combustion in open barrels are also estimated, but are allocated in 1A4bi. Other combustion emissions are not estimated.

## Others (category 6D)

NH3 emissions from compost production are estimated using regional activity data combined with a default emission factor of  0,24 kg NH3/ton compost.

# Recalculations and Improvements

## Recalculations

As was mentioned in the previous Informative Inventory Report, the regions continue their effort to actualise and optimize the time series from the year 1990 on. Emissions of the years 1990 to 2007 were actualised and submitted to the secretariat of the UNECE, additionally to the obligatory submission of 2008.

This actualisation was necessary because of changes in methodologies over the years for all regions.

The year 2007 has been completely recalculated as the energy balances were revised. For the year 2008, the 2010 submission is still provisional.

The emissions are reported in the updated templates of the NFR-tables IV 1 for the main pollutants (SO2, NOx, NH3, NMVOC and CO), particulate matter (TSP, PM10 and PM2.5), heavy metals and persistent organic pollutants. Emissions of the main pollutants and of heavy metals and POP’s are reported from 1990 on, emissions of particulate matter (PM) are reported from 2000 on (first year of reporting obligation for PM is 2000).

All emissions are reported in the table IV 1 as sectoral emissions.

- Gridded data and LPS data

- Gridded data for 2000 (national totals) were supplied by May 15th 2006. As already mentioned in the previous Informative Inventory Report, gridded data were also supplied in 2007 for emissions of 2005. No gridded data were supplied in 2009. Currently only national totals are reported since the (Flemish) database does not allow the emission distribution over the EMEP grids at sectoral level.

- The emissions of large point sources for 2005 were revised by the regions in 2007 and submitted to the UNECE secretariat on 30 May 2007.

## Allocation changes of emissions

### Flemish Region

- The emissions of Combined Heat-Power (CHP) installations in the sectors of public electricity, petroleum refining, manufacturing industries and construction (in joint venture with the electricity sector), agriculture and tertiary sector are allocated to the transformation sector 1A1a ‘Public electricity and Heat production’.

- The emissions of the autoproducers are allocated to the sector where the installations are located.

- Emissions of waste incineration plants with energy recuperation are reported under the transformation sector 1A1a ‘Public electricity and Heat production’, whereas the emissions of plants without energy recuperation are allocated to the sector 6C (waste incineration). This is in line with the emissions reported in the CRF tables to UNFCCC, according to the IPCC Good Practice Guidance. From emission year 2007 on all waste incineration plants recuperate the energy. The subdivision between emissions of waste incineration plants with and without energy recuperation could be done for the first time for NMVOC in 2007 for the complete time series. The new NFR template demands the distinction between clinical, industrial, municipal, cremation and small scale waste incineration. Most waste incineration emissions are due to industrial and municipal waste incineration. Based on information by OVAM (Public Waste Agency of Flanders) a distribution of 73 % municipal waste versus 27 % industrial waste is used. Due to a lack of detailed data, this subdivision was not possible for dioxins nor for PAH’s. Due to a limited number of detailed data from 1991 to 1998, all dioxin emissions of municipal, industrial, hazardous and sludge incineration, as well as of crematoria are allocated in the sector 6Cb.

- The emissions from the chemical sector cover for all contaminants the emissions from the basic chemistry and other chemistry. For all contaminants except for NMVOC the chemical sector does not include the emissions caused by ‘production of synthetic fibres (like silk) and other synthetic materials’. The NMVOC emissions of the latter sector are included in the IPCC category 1A2c (combustion) or 2B (process).

- The combustion emissions of manufacture of steel tubes, wire drawing and cold rolling mills are added to the combustion emissions of the ferro sector (1A2a).

- The emissions of the waste recuperation sector, estimated on a collective basis, is allocated in 6D.

- The SNAP item 040413 (Chlorine), which is not allocated in the IPCC classification, is in the (Flemish) reporting allocated in 2B5 (Chemical industry – Other).

- Emissions of particulate matter, originating from road wear by agricultural vehicles were until submission 2006 reported in the IPCC category 1A5b. From a study made by VITO in cooperation with Ghent University, it appeared that most particulate matter emissions originate from land tilling (with a minimal share of road wear by agricultural vehicles). For this reason, it was decided to report these emissions in the category 1A4cii from submission 2007 on.

- NMVOC emissions of different forest types and of grassland used to be reported under IPCC code 5B. In consultation with the other regions, it was decided to report (from the submission 2008 on) NMVOC emissions from grassland under IPCC code 4D1, whereas emissions from forests are reported under the IPCC code 5E. However, since in the new NFR templates 4D1 includes the emissions from N-fertiliser and 5E is not available anymore, NMVOC emissions from grassland are from the submission 2009 on allocated in 4G (Agriculture other), whereas NMVOC emissions from forests are allocated in the memo item 7B.

- The emissions reported in the IPCC category 1A3dii used to include the inland navigation on Flemish waterways and the emissions by the shipping traffic between two Belgian (Flemish) North Sea harbours. As was mentioned in 4.2, shipping emissions are reported from 2008 on in a more extended and detailed way, distinguishing international maritime emissions, international inland waterways and inland navigation.

### Walloon Region

In Wallonia, no changes in allocation of emissions took place during the 2010 submission.

The category ‘Public Electricity and Heat production (1A1a)’ includes fuel combustion emissions associated with the generation of electricity for commercial or public sale and includes emissions of Combined Heat-Power (CHP) installations when there is a joint venture between an industry and a supplier of public electricity.  In this case, all heat is delivered to the industrial plant and the electricity is sold to the energy sector.

The auto-generators category is mapped out in the category 1A2 ‘Manufacturing Industries and Construction’. The sub-category depends on the type of the sector or industry where the energy is used.

The emissions of the cement plants and the lime and limestone plants are included in the category 2A.  The emissions are the sum of combustion and process emissions.

### Brussels Capital Region

In Brussels, no changes in allocation of emissions took place during the 2010 submission.

## Optimisation and actualisation of emission reporting

The regions will continue their work to get a better tuning and harmonization of their methodologies (emission factors, allocations…).

In what follows, a short overview of the most important changes in methodology to calculate the emissions during the 2010 submission and some points of attention for the future emission estimates are given for the three regions in Belgium.

### Flemish Region

- The MIMOSA model, developed by VITO under the authority of VMM and the department of Environment, Nature and Energy, calculates the emissions of (road) traffic in Flanders. This model simulates the emissions of sixteen air pollutants (CO, NOx, fine dust, SO2, CO2, N2O, CH4, Pb, Cu, Cr, Ni, Se, Zn, Cd, volatile organic substances and NH3). Emissions are calculated based on countings on roads in the Flemish Region, which allows a geographic reproduction of the emissions. VMM authorized VITO to adapt the MIMOSA model (MIMOSA 4.0) to actualise, refine and optimize the model and to develop scenario’s. The MIMOSA model is based on the COPERT IV methodology.

- Emissions of heavy metals,CO and NH3 originating from tertiary and residential sectors are estimated with EMEP/EEA emission factors.

- NH3 emissions originating from manure management and fertilizer use are calculated by a model called EMAV, developed by ILVO. This model has been used for the first time during this 2010 submission for the entire time series. The model uses from 2000 on an extended dataset which is on the time of this submission only available until the year 2007. Data for 2008 are now taken the same as 2007. For the submission 2011 this will be updated. From 2000 on the model foresees to take into account the manure  transport between farmers. However because of a technical problem with the transcript of this item in the  model this is not yet taken into account. This will be the case in the next submission.

- Some points of attention mainly with regard to the emissions of NMVOC are:

- Emissions of CS2: these emissions are not included in total emissions of NMVOC. Question is if this approach is right when we take into account the different existing definitions of NMVOC e.g. with regard to the vapour pressure. Contacts with neighbouring countries didn’t give a clear answer so far. Further investigation on this point is necessary. Most important emitters in Flanders of CS2 are situated in the sector production of synthetic fibres (like silk) and other synthetic materials.

- For the CHP installations and the autoproducers of the sector 1A4ai the same emission factors are used. Some of them are different from the default emission factors:

| g/Gj | CO | NOx | NMVOS | Pb | Cd | Hg |
| --- | --- | --- | --- | --- | --- | --- |
| natural gas | 407 | 158 | 47 | 0,000234 | 0,000515 | 0,0001 |
| biogas | 407 | 165 | 50 | 0,000234 | 0,000515 | 0,0001 |
| fuel | 385 | 307 | 46 | 0,00407 | 0,00136 | 0,00136 |

- Dewaxing of automobiles: more investigation work is necessary to estimate the emissions of NMVOC in this sector.

- Coating (coating of metal, paper, plastic; gluing of metal, plastic; production of glue), dry cleaning, degreasing: results of a study performed by the University of Ghent under the authority of VMM became available in June 2009: “Optimization NMVOC emission inventory of the sectors coating, dry cleaning and degreasing”. The emissions of NMVOC of the sectors dry cleaning and decreasing have been optimized. For the next submission also emissions of the coating sectors will be optimized.

### Walloon Region

- The HCB emissions were recalculated for the complete time series.

- The mineral N was revised in Wallonia for the recent years, according to recent data made available (years 200-2008).

- In Wallonia,the allocation between liquid slurry and solid storage were revised for swine manure, as a slight overestimation of the percentage of solid storage was detected in the previous submissions

### Brussels Capital Region

- The Brussels Capital Region is strictly limited to an urban area, consequently these emissions are only available for a limited number of relevant sectors.

- NH3 is added for ‘1A1a Public electricity and heat production’

- Corrections of errors in ‘1A2f i Stationary combustion in manufacturing industries and construction: Other’ for SOx, COV, TSP, PM10 and PM2.5.

- Addition of a vector in the total emissions of sector ‘1A4a i Commercial / institutional: Stationary’

- Corrections of errors in ‘1A4bi  Residential: Stationary plants’ for PAH and Hg.

- Revision of emissions from ‘2C5f Storage, handling and transport of metal products’ with the availability of reports of annual measurement campaigns for NOx, SOx, TSP and CO.

- Revision of NMVOC emissions from ‘3A2 - Industrial Coating Application’.

- Revision of NMVOC emissions from ‘3D1 – Printing’.

- Modification of NMVOC emission factor from ‘3D2 - Domestic solvent use including fungicides’.

Vanherle, K, Van Zeebroeck, B. & Hulskotte, J. (2007). Emissiemodel voor spoorverkeer en scheepvaart in Vlaanderen: EMMOSS. Study performed by Transport and Mobility Leuven (TML) under the authority of VMM. 100 p.  URL: http://www.tmleuven.be/project/emmoss/index.htm

OVAM (2008). Tarieven en capaciteiten voor storten en verbranden - Actualisatie tot 2006, evolutie en prognose. D/2007/5024/79. 69 p.

Bogman, P., Cornelis, W. & Gabriëls, D. (2006). Opwaaiend stof ten gevolge van het bewerken van landbouwgronden. Study performed by Ghent University under the authority of Aminal, Aminal/AMN/BVO/TWOL2004/OL200300249. 60 pp.