|  | REPUBLIC OF BULGARIA |
| --- | --- |
|  | MINISTRY OF ENVIRONMENT AND WATER EXECUTIVE ENVIRONMENT AGENCY |

Bulgarian Informative Inventory Report

National Emissions inventory for year 2008

Submission under the UNECE Convention on Long-Range Transboundary Pollution (CLRTAP/EMEP)

Sofia, March 2010

| Title of Report | Bulgarian Informative Inventory Report for year 2008 |
| --- | --- |
| Contact Name | Mr. Ivan Angelov Head of Delegation to the EB to CLRTAP Mrs. Eksevia Petrusenko Expert in Air Protection Directorate |
| Organization | Ministry of Environment and Water |
| Address | 67, W. Gladstone Str. Sofia 1000 BULGARIA |
| Fax | + 359 2 981 09 54 |
| Phone | + 359 2 940 63 05 + 359 2 940 63 71 |
| E - mail | angelov@moew.government.bg e.petrusenko@moew.government.bg |

| Title of Report | Bulgarian Informative Inventory Report for year 2008 |
| --- | --- |
| Contact Name | Mrs. Evelina Nikolova NRC Air Emissions BG |
| Organization | Executive Environmental Agency |
| Address | 136 "Tzar Boris III" Blvd., P.O.Box 251, 16 18 Sofia, BULGARIA |
| Fax | + 359 2 955 90 15 |
| Phone | + 359 2 940 64 87 |
| E - mail | Serafimov@nfp-bg.eionet.eu.int |

Executive Summary (in English − to be updated yearly)

The National emission inventory for year 2008 is prepared following the recommendations for improvement, set in the Report for the Stage 3 in-depth review of emission inventories submitted under the UNECE LRTAP Convention and EU National Emissions Ceilings Directive.

According to the long term plan, approved by the EMEP Steering Body Bulgaria was one of the countries, selected for stage 3 in-depth reviews of the UNECE LRTAP Convention and EU NEC Directive inventories in 2009.  The review was coordinated by the EMEP emission centre CEIP acting as review secretariat. The review took place from 22nd June 2009 to 25th June 2009 in Copenhagen Denmark and was hosted by the European Environment Agency (EEA).

Explanation of differences between reported national totals presented in Annex IV Table 1

There are differences in reported national totals between emissions inventories under CLRTAP (2010 submission) and NEC Directive (2009 submission). The reasons may be explained as follows:

- The activity data for Waste (NFR sector 6) were not yet updated and available in the end of 2009. In order to keep the deadline for submission under NEC Directive (31st of December for year X-1) the activity data for the same sector from the previous year’s submission were used. Till the deadline for submission under the CLRTAP (15th of February for year X-2) all activity data necessary for preparation of the national emission inventory were available;

- The recommendations of the expert review team (ERT) in the frame of the Stage 3 in-depth review were more precisely implemented in the inventory under CLRTAP. In order to improve inventory on the base of the recomendations of the ERT, for some not well developed in the national methodology categories/pollutants, the EFs from  EMEP/CORINAIR Emission Inventory Guidebook 2009 were applied. Changes were made in the following sources: 2A1, 2A2, 2A4, 2A6, 2B1, 2B5a, 2C2, 2D1, 3C, 4B and it is presented in sectoral chapters.

The same reporting table which has been submitted to CLRTAP/EMEP, will be sent in the end of 2010 as a final inventory under the NEC Directive for year X-2.

Clarification of the reason for differences in reported national totals with UNFCCC reports

There are differences between the reported national totals and UNFCCC reports for the entire territory. For assessment of the emissions in the air under UNFCCC were used mainly emission factors which are set in the IPCC (default EFs) and country-specific EFs as well. Generally, the results obtained by the IPCC methods differ from the results of the national inventory, which is carried out in compliance with the CORINAIR methodology. The reasons for the difference have both methodological and structural origin. There are also certain differences in the quantity of the input data, used for calculating the emissions. The activity data from the national material and energy balances are the base for calculating the emissions from sectors Energy and Industrial Processes under UNFCCC. The activity data, which are used for UNECE/CLRTAP and NECD inventory are collected directly from the emission stationary sources.

According to Bulgarian Clean Air Act (CAA), article 25 (6) the Minister of Environment and Water in co-ordination with the interested ministers approves a Methodology for the calculation, using balance methods, of the emissions of harmful substances (pollutants), emitted in the ambient air. The national Methodology (approved with Order RD 77 from 03.02.2006 of MOEW) is harmonized with CORINAIR methodology for calculation of the emissions according to the CLRTAP.

During 2007, there was a project for development of a Common methodology for emissions inventory under CLRTAR and UNFCCC, i.e. to update the present Methodology under article 25 (6) CAA. (approved with Order RD 40 from 22.01.2008 of MEW)

The aim of the project was harmonization of the national Methodology with IPCC, including the three main greenhouse gases – CO2, CH4 and N2O (plus relevant ODS and SF6).

Chapter 1. Introduction

## 1.1 National Inventory Background

The UNECE Geneva Convention on Long-range Transboundary Air Polution was developed to assess and control transport of pollutants between countries.

Bulgaria ratified the Convention in 1981 as well as its related Protocols:

- The 1985 Helsinki Protocol on the Reduction of Sulphur Emissions or their Transboundary Fluxes by at least 30%;

- The 1988 Sofia Protocol concerning the Control of Emissions of Nitrogen Oxides or their Transboundary Fluxes;

- The 1991 Geneva Protocol on the Control of Emissions of Volatile Organic Compounds or their Transboundary Fluxes;

- The 1994 Oslo protocol on Further Reduction of Sulphur Emissions;

- The 1998 Aarhus Protocol on Heavy Metals;

- The 1998 Aarhus Protocol on Persistent Organic Pollutants.

- The 1999 Gothenburg Protocol to Abate Acidification, Eutrophication and Ground-level Ozone

- The 1984 Geneva Protocol on Long-term Financing of the Cooperative Programme for Monitoring and Evaluation of the Long-range Transmission of Air Pollutants in Europe (EMEP)

## 1.2 Institutional arrangements 
(to be updated when there have been major changes since the last report)

The national reporting obligations to the UNFCCC, UNECE and EC are administered by the Ministry of Environment and Water (MOEW).

The administrative structure responsible for the preparation of Emissions inventories under UNFCCC, UNECE and EC is the Executive Environmental Agency (ExEA).

Legal basis of the Bulgarian National Inventory System (NIS)

The legal basis for the Bulgarian NIS is provided by the Environmental Protection Act /EPA/ (State Gazette №91/2002), in particular by the provisions of it’s Chapter 8, which establishes the National Environmental Monitoring System and lists all tasks.

EPA establishes the national ExEA and its responsibilities for monitoring of the environment as well as the responsibility for preparation of emission inventories.

To ensure the effective and timely functioning of NIS, as well as complete reporting under the United Nations Framework Convention on Climate Change (UNFCCC) and the Convention of Long-Range Transboundary Air Pollution (CLRTAP), the Order № RD-54/25.01.2007, issued by the Minister of Environment and Water, defines the organisation of the activities concerning the preparation and presentation of the national emissions inventories and their supporting reports under the both conventions.

In conformity with the requirements of the UNFCCC and the CLRTAP, the above mentioned Order defines the following:

- Procedures for inventory of air emissions, including the anthropogenic GHGs emissions in the atmosphere;

- Responsible institutions and their obligations, including the main sources of input data

- QA/QC Procedures;

- Preparation of the national reports for the results of the inventories (NIR and IIR);

- The order and requirements for archiving of the information.

In addition, an order (Order № RD-377/08.06.2007 by the Minister of Environment and Water) is issued on the basis of Article 4 from the Council of Ministers Regulation on the organization of activities with regard to preparation and presentation of reports to the European Commission on the implementation of the legislative acts, which are part from the European Community legislation in the field of environmental protection (State Gazette №43/2007), as well in accordance with Chapter III.2 of the above mentioned Order № RD-54/25.01.2007. This Order RD-377/2007 determines:

- Procedures and requirements for reporting to the European Commission and the European Environment Agency;

- Timely performance of all activities concerning the preparation of national inventories and relevant national reports.

Besides the EPA, there are other legal and institutional arrangements and agreements which are the basis for the NIS:

National Statistics Act - according to it the National Statistical Institute (NSI) has to prepare annually national material and energy balances. NSI has its internal regulations, which determine the responsibilities of its departments, including their responsibilities with regard to activity data provision and data quality control.

Till the end of 2009 the collaboration between MOEW and NSI regarding the provision and exchange of the statistical and environmental information was regulated by an official agreement (RD21-25/30.01.2003). In order to strengthen the institutional arrangements and to fulfill the required general and specific functions of NIS a new agreement between MOEW and National Statistical Institute was signed (RD21-35/12.02.2010). There is also a new agreement between MOEW and the Ministry of Agriculture and Food (MAF) and its institutions. These agreements ensure the support from these organisations regarding the choice of the activity data and EFs and methods, in the compilation of emission estimates and QA/QC of these estimates.

As a consequence of the above mentioned orders, all institutional and procedural options of Bulgarian NIS are determined, in order to ensure the effective functioning.

- In the end of 2009 two projects were approved by the MOEW, which objectives are to improve the Party’s current estimation methodologies, the technical competence of the staff within the national system, and the management and archiving of data. Both projects are already in progress:

- Project 1 – “Recalculations of the previously submitted estimates of emissions under UNFCCC and the European Monitoring and Evaluation Programme/Convention on Long-range Transboundary Air Pollution (EMEP/CLRTAP) according to the new Common Methodology from the base year up to the year in which recalculations are made and cover all inventory data”.  This project started on the 1st of December 2009 and will be completed by the 1st of February 2011. The result of this project will be incorporated into the 2011 submission.

- Project 2 – “Development of software tool and automatic preparation of national inventories under UNFCCC and UNECE/CLRTAP”. This project is to commence on 28 December 2009 and will be completed by 28 December 2010. The result of this project will be incorporated into the 2011 submission.

- Bulgaria will use these projects to improve the capacity of the national system in planning, preparing and managing its annual submissions, to improve the technical competence of the staff, and to fulfil the requirements for Recalculations of the previously submitted estimates.

## 1.3 Inventory preparation process 
(to be updated when there have been major changes since the last report)

All activities regarding the preparation of emissions inventories are coordinated and managed on the state level by the Ministry of Environment and Water.

The ExEA is the institution in charge with the overall responsibility for Management of the inventory process (Order No 54/25.01.2007 of the Minister of Environment and Water) for GHG emissions (under UNFCCC) and emissions that are defined in the CLRTAP.

The preparation of the inventory includes three main stages: planning, preparation and management. When planning the particular inventory the following documents are taken into account with regard to potential inventory improvements:

- - Inventory Improvement Plan for the relevant reported year;

- - Recommendations from reviews (stage 3 in depth review);

- - Other recommendations;

- - Research;

- - Plan operators and industrial associations recommendations and/or remarks.

Inventory improvements are implemented before the inventory is compiled.

The institutions involved at national and local (sub-national) levels in the emissions inventory are Ministry of Environment and Water (MOEW) respectively Executive Environmental Agency (ExEA), Regional Environment Inspectorates (REI), and National Statistical Institute (NSI) with its regional bodies.

As it is written above there is an official agreement between MEW and NSI for collaboration in the process of preparation of the national emissions inventory.  ExEA is an official member of the NSI for exchange of statistical information. According to the agreement the collection and provision of the necessary activity data by source categories is responsibility of the NSI.

NSI uses up-to-date statistical methods and procedures for data collection, summarizing and structuring which are harmonized with EUROSTAT. NSI has two level hierarchical structure - National office and Regional offices. The primary statistical questionnaires are collected at the regional statistical offices, examined for consistency of the data and processed. 
The National office receives the primary information and the processed information from the regional offices and develops the National totals and balances.

Two parallel emission inventory programs are conducted in Bulgaria. The first one covers 150 large point sources and it is conducted by REI. The second one which covers more then 2000 point sources is conducted by the NSI. Both are under the guidance of the Ministry of Environment and Water. The data collected are for air pollution control facilities and their efficiency, technological and production data, data for fuels used and fines imposed.

The National Statistic Institute is the responsible organization for assessment of the following air emissions sources:

- Combustion in energy production and energy transformation;

- Combustion in commercial, institutional sectors and agriculture, forestry, fishing;

- Combustion in industry;

- Production processes;

- Extraction and distribution of fossil fuels;

- Agriculture.

The Executive Environmental Agency is the organization responsible for assessment of the following air emissions sources:

- Combustion in residential sector;

- Road transport;

- Other mobile sources and machinery;

- Waste treatment and disposal.

Data from the emission inventory are stored at local and national levels.

The national data base for air emissions including archiving of data is located in ExEA.

Data Documentation

Documentation of the inventory is sufficiently detailed and clear as to allow to an independent but knowledgeable analyst to obtain and review the references used and to reproduce the emission estimates.

Maintaining Files for whole inventory

The Inventory Team Leader (ITL) maintaines a complete and separate file for the whole inventory. The intent is that this file includes all the materials and the analyst needs to develop the inventory for the relevant year. The most important is that the files should be maintained in a transparent manner. The ITL has considerable discretion over the form and content of the file (files).

A file is maintained for the current inventory and includes all relevant information. The file contains a list of the names and locations of all “working” spreadsheets, with explanations of links among them, and any recent electronic back-up copies of working drafts of the spreadsheets. It also contains results of check/review with all necessary contact information, copies of reference materials or data. Copies of hand calculations or notes made by the inventory staff, appropriately documented, are placed in the file. In addition, copies of the checklist completed for QA/QC purposes, together with the attendant file, are placed in the file.

The QA/QC Manager checks the files for completeness as part of Tier 1 quality control activities. Completed checks are reported on the QA/QC checklist.

Documenting the Inventory Spreadsheets and the Inventory Report

The inventory staff ensured that the documentation associated with the inventory is sufficient for an independent analyst to determine the reference source for each piece of data used to calculate emissions, and to locate the data in the inventory archives. The documentation also provided complete information on any changes that are made to data sources or methodological changes that occur in a given inventory. Both the inventory spreadsheets and the Inventory Report itself are subject to scrutiny to determine if the references are complete, accurate, and consistent. Both the inventory staff and especially QA/QC Manager are familiar with the following procedures, which are designed to maintain high quality.

In the spreadsheets, every primary data element (activity data, emission factor, etc.) must have a reference for the source of the data. No non-calculated values should appear in the spreadsheets that are not referenced, with the exception of standard unit conversion factors or similar information.

Everything - supporting documentation, comments, and especially all printouts made from spreadsheets - is dated.

The inventory team and QA/QC Manager perform various checks to verify the adequacy of the documentation of the spreadsheet.

General documentation of references

To the extent feasible, effort is made to ensure that the documentation follows a uniform format across the inventory spreadsheets.

Individuals, agencies, institutions, or companies providing information are identified by full name, association, phone and fax numbers.

## 1.4 Methods and data sources 
(to be updated when there have been major changes since the last report)

National Inventory Methodology

According to Clean Air Act (CAA), article 25 (6) The Minister of Environment and Water in co-ordination with the interested ministers approves a Methodology for the calculation, with balance methods, of the emissions of harmful substances (pollutants), emitted in the ambient air. This national Methodology (approved with Order RD 77/03.02.2006 of MOEW) is harmonized with the CORINAIR methodology for calculation of the emissions according to the CLRTAP.

In 2007 MOEW/ExEA ran a project for development of a Common methodology for emissions inventory under CLRTAR and UNFCCC, i.e. to update the present Methodology under article 25 (6) CAA. (Approved with Order RD 40 from 22.01.2008 of MOEW).

- The aim of the project was the harmonization of the national Methodology with IPCC, including the three main greenhouse gases – CO2, CH4 and N2O (plus relevant ODS and SF6). The results of this project for the inventory of NOx, NMVOC, CO and SOx under UNFCCC will be incorporated into the 2011 submission.

The second part of the project was amended in order to include additional requirements for establishment of Data Quality Control System, i.e. to include procedures for QA/QC at all stages of data collection and calculation of emissions.

The first project was finalized in 2007, the second one – in 2008.

The official QA/QC Plan for National emissions inventories was approved by the Ministry of Environment and Water in 2009.

## 1.5 Key Categories (to be updated each year)

Methodological approach to identify key categories

It is good practice for each country to identify its national key categories in a systematic and objective way. This can be achieved by a quantitative analysis of the relationship between the magnitude of emissions in any one year (level) and the change in the emissions year to year (trend) of each category’s emissions compared to the total national emissions.

Key category analysis is prepared based on the methodology set in the EMEP/EEA air pollutant emission inventory guidebook - 2009.

The methodology follows the IPCC approach to produce pollutant-specific key categories and covers Approaches 1 and 2 for both level and trend assessments. The two approaches developed by the IPCC and described below, for performing the key category analysis can be used to identify the key categories for each pollutant. Both approaches identify key categories in terms of their contribution to the absolute level of the national emissions and to the trend of emissions.

- In Approach 1:	 the key categories are identified using a predetermined cumulative emissions threshold. Key categories are those which, when summed together in descending order of magnitude, cumulatively add up to 80% of the total level.

- In Approach 2:	 the key categories can be derived by inventory compilers if category uncertainties or parameter uncertainties are available. Under Approach 2 the categories are sorted according to their contribution to uncertainty.

For preparation of the key category analysis and based on the inventory data available Approach 1 has been selected.

Level assessment

The contribution of each source category to the total national inventory level is calculated according to equation (1) (level assessment (Approach 1)):

Key category level assessment = source category estimate / total contribution

Lx,t = Ex,t /  ΣEt			(1)

Where:

Lx,t = level assessment for source x in latest inventory year (year t)

Ex,t = value of emission estimate of source category x in year t

ΣEt = total contribution, which is the sum of the emissions in year t, calculated using the aggregation level chosen by the country for key category analysis

The next tables present key category analysis (Approach 1) for each pollutant, based on the latest inventory year. The source categories are presented in the tables, sorted by largest contribution to national total.

Key categories are those that, when summed together in descending order of magnitude, add up to 80 % of the total in column Comulative total. For each pollutant key categories are coloured in grey.

Spreadsheet for the Approach 1 analysis — level assessment of NOx

| NFR category code | NFR category | Latest year estimate NOx, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A1a | 1 A 1 a Public electricity and heat production | 56.82 | 0.30 | 30 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 48.85 | 0.25 | 25 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 37.92 | 0.20 | 20 |
| 2B2 | 2 B 2 Nitric acid production | 25.20 | 0.13 | 13 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 5.80 | 0.03 | 3 |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 5.68 | 0.03 | 3 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 4.92 | 0.03 | 3 |
| 4F | 4 F Field burning of agricultural wastes | 3.42 | 0.02 | 2 |
| 1 A 3 c | 1 A 3 c Railways | 1.28 | 0.01 | 1 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.42 | 0.00 | 0 |
| 1B2c | 1 B 2 c Venting and flaring | 0.38 | 0.00 | 0 |
| 1 A 3 a i (i) | 1 A 3 a i (i) International aviation (LTO) | 0.25 | 0.00 | 0 |
| 2C1 | 2 C 1 Iron and steel production | 0.23 | 0.00 | 0 |
| 2D1 | 2 D 1 Pulp and paper | 0.21 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.18 | 0.00 | 0 |
| 2A6 | 2 A 6 Road paving with asphalt | 0.17 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.12 | 0.00 | 0 |
| 1A3aii(i) | 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | 0.11 | 0.00 | 0 |
| 2 C 5 a | 2 C 5 a Copper production | 0.09 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.09 | 0.00 | 0 |
| 1A2d | 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | 0.07 | 0.00 | 0 |
| 2 C 5 b | 2 C 5 b Lead production | 0.04 | 0.00 | 0 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.03 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.01 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.00 | 0.00 | 0 |
| 1B2a iv | 1 B 2 a iv Refining / storage | 0.00 | 0.00 | 0 |
| 2C2 | 2 C 2 Ferroalloys production | 0.00 | 0.00 | 0 |
| TOTAL 2008 |  | 192.28 | 1 | 100 |

The main sources of NOx emissions are NFR 1A1 Public electricity and heat production, 1A3b Road transport (national total),  1A5b Other, Mobile (national total) and 2B2 Nitric acid production, which contribut respectively 30%, 25%, 20% and 13% to national total.

Spreadsheet for the Approach 1 analysis — level assessment of NMVOC

| NFR category code | NFR category | Latest year estimate NMVOC, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 39.444 | 0.32 | 32 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 34.215 | 0.28 | 28 |
| 4D1a | 4 D 1 a Synthetic N-fertilizers | 19.850 | 0.16 | 16 |
| 4F | 4 F Field burning of agricultural wastes | 6.832 | 0.06 | 6 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 6.068 | 0.05 | 5 |
| 3D3 | 3 D 3 Other product use | 2.896 | 0.02 | 2 |
| 2D2 | 2 D 2 Food and drink | 2.390 | 0.02 | 2 |
| 3A2 | 3 A 2 Industrial coating application | 2.161 | 0.02 | 2 |
| 1B2a v | 1 B 2 a v Distribution of oil products | 2.104 | 0.02 | 2 |
| 2B1 | 2 B 1 Ammonia production | 2.039 | 0.02 | 2 |
| 2B5a | 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 1.226 | 0.01 | 1 |
| 3D1 | 3 D 1 Printing | 0.861 | 0.01 | 1 |
| 3A3 | 3 A 3 Other coating application (Please specify the sources included/excluded in the notes column to the right) | 0.484 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.269 | 0.00 | 0 |
| 3A1 | 3 A 1 Decorative coating application | 0.242 | 0.00 | 0 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.203 | 0.00 | 0 |
| 2C1 | 2 C 1 Iron and steel production | 0.186 | 0.00 | 0 |
| 3C | 3 C Chemical products | 0.178 | 0.00 | 0 |
| 1B2a iv | 1 B 2 a iv Refining / storage | 0.174 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.150 | 0.00 | 0 |
| 2D1 | 2 D 1 Pulp and paper | 0.136 | 0.00 | 0 |
| 3B1 | 3 B 1 Degreasing | 0.090 | 0.00 | 0 |
| 6A | 6 A Solid waste disposal on land | 0.085 | 0.00 | 0 |
| 1A1a | 1 A 1 a Public electricity and heat production | 0.073 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.061 | 0.00 | 0 |
| 2A6 | 2 A 6 Road paving with asphalt | 0.055 | 0.00 | 0 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.022 | 0.00 | 0 |
| 1 A 3 a i (i) | 1 A 3 a i (i) International aviation (LTO) | 0.015 | 0.00 | 0 |
| 2C2 | 2 C 2 Ferroalloys production | 0.012 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.009 | 0.00 | 0 |
| 2A5 | 2 A 5 Asphalt roofing | 0.008 | 0.00 | 0 |
| 1A3aii(i) | 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | 0.006 | 0.00 | 0 |
| 1A2d | 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | 0.003 | 0.00 | 0 |
| 3B2 | 3 B 2 Dry cleaning | 0.003 | 0.00 | 0 |
| 2 C 5 a | 2 C 5 a Copper production | 0.003 | 0.00 | 0 |
| 1B2a i | 1 B 2 a i Exploration, production, transport | 0.002 | 0.00 | 0 |
| 2 C 5 b | 2 C 5 b Lead production | 0.001 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.000 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.000 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.000 | 0.00 | 0 |
| 1B2b | 1 B 2 b Natural gas | 0.000 | 0.00 | 0 |
| TOTAL 2008 |  | 122.559 | 1 | 100 |

The main sources of NMVOC emissions are NFR 1A4bi  Residential: Stationary plants and 1A3b   Road transport, which contribut respectively 32% and 28% to the national total.

The biggest part of NMVOC emissions are due to burning of woods for heating in residential sector. The next table presents disaggregated sector NFR 1A4bi to main fuel types, related to NMVOC emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutant | EF, g/GJ | Emissions (Gg) |
| Woods | 2321050.1 | NMVOC | 925 | 38.645 |
| Coal | 91303 | NMVOC | 300 | 0.630 |
| Briquettes | 74501 | NMVOC | 200 | 0.149 |
| Gaseous Fuels | 89350 | NMVOC | 5 | 0.019 |
| Naphtha | 842 | NMVOC | 15 | 0.001 |
| Total NFR 1A4bi |  | NMVOC |  | 39.444 |

Spreadsheet for the Approach 1 analysis — level assessment of SOx

| NFR category code | NFR category | Latest year estimate SOx, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A1a | 1 A 1 a Public electricity and heat production | 663.223 | 0.90 | 90 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 30.338 | 0.04 | 4 |
| 2B5a | 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 15.991 | 0.02 | 2 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 5.950 | 0.01 | 1 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 5.830 | 0.01 | 1 |
| 2D1 | 2 D 1 Pulp and paper | 4.389 | 0.01 | 1 |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 3.650 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 2.691 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 1.112 | 0.00 | 0 |
| 1B2c | 1 B 2 c Venting and flaring | 0.549 | 0.00 | 0 |
| 2 C 5 b | 2 C 5 b Lead production | 0.444 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.299 | 0.00 | 0 |
| 2 C 5 a | 2 C 5 a Copper production | 0.255 | 0.00 | 0 |
| 2A6 | 2 A 6 Road paving with asphalt | 0.210 | 0.00 | 0 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 0.150 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.060 | 0.00 | 0 |
| 1 A 3 a i (i) | 1 A 3 a i (i) International aviation (LTO) | 0.024 | 0.00 | 0 |
| 1A2d | 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | 0.018 | 0.00 | 0 |
| 2B4 | 2 B 4 Carbide production | 0.014 | 0.00 | 0 |
| 1A3aii(i) | 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | 0.010 | 0.00 | 0 |
| 2C1 | 2 C 1 Iron and steel production | 0.004 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.003 | 0.00 | 0 |
| 1B2a iv | 1 B 2 a iv Refining / storage | 0.002 | 0.00 | 0 |
| 2B1 | 2 B 1 Ammonia production | 0.001 | 0.00 | 0 |
| 2C2 | 2 C 2 Ferroalloys production | 0.00025 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.00009 | 0.00 | 0 |
| 1B2b | 1 B 2 b Natural gas | 0.00003 | 0.00 | 0 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.00003 | 0.00 | 0 |
| TOTAL 2008 |  | 735.218 | 1 | 100 |

The main source of SOx emissions is 1A1a Public electricity and heat production, which contributs 90% to the national total. The main part of these emissions are due to Combustion plants ≥ 50 < 300 MWth and Combustion plants ≥ 300 MWth (boilers).

The next table presents disaggregated sector NFR 1A1a to SNAP subcategories related to SOx emissions:

| Disaggregation of NFR 1A1a to SNAP subcategories |  |  |
| --- | --- | --- |
| SNAP | Category | Latest year estimate SOx, Gg |
| 010102 | Combustion plants ≥50<300MW (boilers) | 436.890 |
| 010101 | Combustion plants≥300MW (boilers) | 207.628 |
| 010202 | Combustion plants ≥50<300MW(boilers) | 17.926 |
| 010203 | Combustion plants < 50 MW (boilers) | 0.776 |
| 010105 | Stationary engines | 0.002 |
| 010103 | Combustion plants < 50 MW (boilers) | 0.001 |
| Total | 1A1a | 663.223 |

The biggest source of SOx emissions is Energy complex Maritza East with 3 TPP: Brikel - factory for producing of briquettes, which includes TPP Maritza East 1 (capacity of 1020 MWth), TPP Maritza East 2 (capacity 4312 MWth) and TPP Maritza Iztok 3 (capacity 2420 MWth).

In 2008 the Energy complex Maritza East contributes 56% to the national total emissions of SOx. The Energy complex Maritza East use lignite coals with high contain of sulfur (between 2.0% and 3.7%). The contain of ash in the coals is between 15.7% and 29.8%.

From 2002 the power plants in the complex started a process of equipment of the boilers with desulphurization facilities, thus reducing the SOx emissions. In 2008 the desulphurization facilities reduced SO2 emissions with 305 105 Gg.

Spreadsheet for the Approach 1 analysis — level assessment of NH3

| NFR category code | NFR category | Latest year estimate NH3, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 4B1 a | 4 B 1 a Cattle dairy | 14.015 | 0.24 | 24 |
| 6A | 6 A Solid waste disposal on land | 6.823 | 0.12 | 12 |
| 4D1a | 4 D 1 a Synthetic N-fertilizers | 6.102 | 0.10 | 10 |
| 4B1 b | 4 B 1 b Cattle non-dairy | 5.949 | 0.10 | 10 |
| 4B8 | 4B8 Swine | 5.512 | 0.09 | 9 |
| 4B9d | 4B9d Other Poultry | 5.415 | 0.09 | 9 |
| 4B3 | 4B3 Sheep | 4.884 | 0.08 | 8 |
| 4B9a | 4B9 a Laying hense | 3.225 | 0.06 | 6 |
| 4B6 | 4B6 Horses | 2.778 | 0.05 | 5 |
| 2B5a | 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 1.538 | 0.03 | 3 |
| 4F | 4 F Field burning of agricultural wastes | 1.025 | 0.02 | 2 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 0.691 | 0.01 | 1 |
| 6B | 6 B Waste-water handling | 0.183 | 0.00 | 0 |
| 2A4 | 2 A 4 Soda ash production and use | 0.019 | 0.00 | 0 |
| 2B2 | 2 B 2 Nitric acid production | 0.007 | 0.00 | 0 |
| 2B1 | 2 B 1 Ammonia production | 0.004 | 0.00 | 0 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 0.004 | 0.00 | 0 |
| 6D | 6 D Other waste(e) | 0.002 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.0002 | 0.00 | 0 |
| 1A2d | 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | 0.0002 | 0.00 | 0 |
| TOTAL 2008 |  | 58.175 | 1 | 100 |

The main sources of NH3 emissions are NFR 4B Manure management (72%), 6A Solid waste disposal on land (12%) and 4D1 Synthetic N-fertilizers (10%).

Spreadsheet for the Approach 1 analysis — level assessment of PM2.5

| NFR category code | NFR category | Latest year estimate PM2.5, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 20.61500 | 0.87 | 87 |
| 2A6 | 2 A 6 Road paving with asphalt | 1.37390 | 0.06 | 6 |
| 1A1a | 1 A 1 a Public electricity and heat production | 0.80608 | 0.03 | 3 |
| 2A1 | 2 A 1 Cement production | 0.44950 | 0.02 | 2 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.17310 | 0.01 | 1 |
| 2D1 | 2 D 1 Pulp and paper | 0.06945 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.04420 | 0.00 | 0 |
| 2B5a | 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 0.02370 | 0.00 | 0 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.01484 | 0.00 | 0 |
| 2A2 | 2 A 2 Lime production | 0.01300 | 0.00 | 0 |
| 1 A 3 a i (i) | 1 A 3 a i (i) International aviation (LTO) | 0.00030 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.00022 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.00020 | 0.00 | 0 |
| 1A3aii(i) | 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | 0.00013 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.00001 | 0.00 | 0 |
| TOTAL 2008 |  | 23.58364 | 1 | 100 |

The main source of PM2.5 emissions is NFR 1A4bi  Residential: Stationary plants, which contribut 87% to the national total. The biggest part of PM2.5 emissions are due to burning of woods for heating in residential sector. The next table presents disaggregated sector NFR 1A4bi  to main fuel types, related to PM2.5 emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutant | EF, g/GJ | Emissions (Gg) |
| Woods | 2321050.1 | PM2.5 | 475 | 19.845 |
| Coal | 91303 | PM2.5 | 330 | 0.693 |
| Briquettes | 74501 | PM2.5 | 100 | 0.075 |
| Gaseous Fuels | 89350 | PM2.5 | 0.5 | 0.002 |
| Naphtha | 842 | PM2.5 | 16.5 | 0.001 |
| Total NFR 1A4bi |  | PM2.5 |  | 20.615 |

Spreadsheet for the Approach 1 analysis — level assessment of PM10

| NFR category code | NFR category | Latest year estimate PM10, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 29.61750 | 0.50 | 50 |
| 2A6 | 2 A 6 Road paving with asphalt | 10.30450 | 0.18 | 18 |
| 1A1a | 1 A 1 a Public electricity and heat production | 9.78596 | 0.17 | 17 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 3.15200 | 0.05 | 5 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 3.03380 | 0.05 | 5 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 1.37252 | 0.02 | 2 |
| 2A1 | 2 A 1 Cement production | 0.81700 | 0.01 | 1 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.47602 | 0.01 | 1 |
| 2D1 | 2 D 1 Pulp and paper | 0.10231 | 0.00 | 0 |
| 2A2 | 2 A 2 Lime production | 0.06300 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.04650 | 0.00 | 0 |
| 2B5a | 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 0.03160 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.00446 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.00308 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.00036 | 0.00 | 0 |
| TOTAL 2008 |  | 58.81061 | 1 | 100 |

The main sources of PM10 emissions are  1A4bi  Residential: Stationary plants (50%), 2A6 Road paving with asphalt (18%) and 1A1a Public electricity and heat production (17%).  The next table presents disaggregated sector NFR 1A4bi  to main fuel types, related to PM10 emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutant | EF, g/GJ | Emissions (Gg) |
| Woods | 2321050.1 | PM10 | 695 | 29.036 |
| Coal | 91303 | PM10 | 240 | 0.504 |
| Briquettes | 74501 | PM10 | 100 | 0.075 |
| Gaseous Fuels | 89350 | PM10 | 0.5 | 0.002 |
| Naphtha | 842 | PM10 | 21.5 | 0.001 |
| Total NFR 1A4bi |  | PM10 |  | 29.618 |

Spreadsheet for the Approach 1 analysis — level assessment of TSP

| NFR category code | NFR category | Latest year estimate TSP, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2A6 | 2 A 6 Road paving with asphalt | 48.09 | 0.35 | 35 |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 31.33 | 0.23 | 23 |
| 1A1a | 1 A 1 a Public electricity and heat production | 30.73 | 0.23 | 23 |
| 2B5a | 2 B 5 a Other chemical industry (Please specify the sources included/excluded in the notes column to the right) | 15.42 | 0.11 | 11 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 5.44 | 0.04 | 4 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 1.61 | 0.01 | 1 |
| 2C1 | 2 C 1 Iron and steel production | 1.20 | 0.01 | 1 |
| 2A1 | 2 A 1 Cement production | 0.90 | 0.01 | 1 |
| 2D1 | 2 D 1 Pulp and paper | 0.20 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.19 | 0.00 | 0 |
| 2A2 | 2 A 2 Lime production | 0.15 | 0.00 | 0 |
| 2 C 5 a | 2 C 5 a Copper production | 0.15 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.15 | 0.00 | 0 |
| 2 C 3 | 2 C 3 Aluminum production | 0.14 | 0.00 | 0 |
| 2 C 5 b | 2 C 5 b Lead production | 0.07 | 0.00 | 0 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.07 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.01 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.01 | 0.00 | 0 |
| 2C2 | 2 C 2 Ferroalloys production | 0.01 | 0.00 | 0 |
| 2A4 | 2 A 4 Soda ash production and use | 0.00220 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.00196 | 0.00 | 0 |
| 3C | 3 C Chemical products | 0.00137 | 0.00 | 0 |
| 1B2a iv | 1 B 2 a iv Refining / storage | 0.00129 | 0.00 | 0 |
| 2A5 | 2 A 5 Asphalt roofing | 0.00001 | 0.00 | 0 |
| TOTAL 2008 |  | 135.87 | 1 | 100 |

The main sources of TSP emissions are 2A6 Road paving with asphalt (35%), 1A4bi  Residential Stationary plants (23%), 1A1a Public electricity and heat production (23%).

Based on the recomendation of the ERT’s report from stage 3 in-depth review for some categories/pollutants, which are not well developed in the national methodology, EFs from  EMEP/CORINAIR Emission Inventory Guidebook 2009 were applied in order to improve inventory. Such improvements are made in NFR 2A6 regarding the dust emissions.

|  | NFR 2A6 SNAP 040611 |  |  |
| --- | --- | --- | --- |
| Component | Activity data | EF | Emissions |
|  | Mg | kg/Mg | Gg |
| TSP | 3434834.757 | 14 | 48.0876866 |
| PM10 | 3434834.757 | 3 | 10.3045043 |
| PM2.5 | 3434834.757 | 0.4 | 1.3739339 |

The next table presents disaggregated sector NFR 1A4bi to main fuel types, related to TSP emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutant | EF, g/GJ | Emissions (Gg) |
| Woods | 2321050.1 | TSP | 730 | 30.499 |
| Coal | 91303 | TSP | 350 | 0.735 |
| Briquettes | 74501 | TSP | 120 | 0.089 |
| Gaseous Fuels | 89350 | TSP | 0.5 | 0.002 |
| Naphtha | 842 | TSP | 27.5 | 0.001 |
| Total NFR 1A4bi |  | TSP |  | 31.326 |

Spreadsheet for the Approach 1 analysis — level assessment of CO

| NFR category code | NFR category | Latest year estimate CO, Gg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 177.847 | 0.43 | 43 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 176.844 | 0.43 | 43 |
| 2C1 | 2 C 1 Iron and steel production | 28.409 | 0.07 | 7 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 15.169 | 0.04 | 4 |
| 2B1 | 2 B 1 Ammonia production | 3.427 | 0.01 | 1 |
| 4F | 4 F Field burning of agricultural wastes | 1.776 | 0.00 | 0 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 1.342 | 0.00 | 0 |
| 1A1a | 1 A 1 a Public electricity and heat production | 1.158 | 0.00 | 0 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.597 | 0.00 | 0 |
| 1 A 3 a i (i) | 1 A 3 a i (i) International aviation (LTO) | 0.358 | 0.00 | 0 |
| 2A6 | 2 A 6 Road paving with asphalt | 0.352 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.346 | 0.00 | 0 |
| 2D1 | 2 D 1 Pulp and paper | 0.305 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.244 | 0.00 | 0 |
| 2A4 | 2 A 4 Soda ash production and use | 0.195 | 0.00 | 0 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.190 | 0.00 | 0 |
| 1A3aii(i) | 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | 0.152 | 0.00 | 0 |
| 2 C 5 b | 2 C 5 b Lead production | 0.087 | 0.00 | 0 |
| 1B2c | 1 B 2 c Venting and flaring | 0.085 | 0.00 | 0 |
| 1B2a iv | 1 B 2 a iv Refining / storage | 0.073 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.021 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.013 | 0.00 | 0 |
| 1A2d | 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | 0.011 | 0.00 | 0 |
| 2C2 | 2 C 2 Ferroalloys production | 0.009 | 0.00 | 0 |
| 2 C 5 a | 2 C 5 a Copper production | 0.007 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.005 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.005 | 0.00 | 0 |
| 2A5 | 2 A 5 Asphalt roofing | 0.00017 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.00016 | 0.00 | 0 |
| TOTAL 2008 |  | 409.026 | 1 | 100 |

The main sources of CO emissions are 1A4bi  Residential Stationary plants (43%) and 1A3b   Road transport (43%).

The next table presents disaggregated sector NFR 1A4bi  to main fuel types, related to CO emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutant | EF, g/GJ | Emissions (Gg) |
| Woods | 2321050.1 | CO | 4000 | 167.116 |
| Coal | 91303 | CO | 4000 | 8.400 |
| Briquettes | 74501 | CO | 3000 | 2.235 |
| Gaseous Fuels | 89350 | CO | 25 | 0.094 |
| naphtha | 842 | CO | 70 | 0.003 |
| Total NFR 1A4bi |  | CO |  | 177.847 |

Spreadsheet for the Approach 1 analysis — level assessment of Pb

| NFR category code | NFR category | Latest year estimate Pb, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2 C 5 b | 2 C 5 b Lead production | 185.374 | 0.62 | 62 |
| 2 C 5 a | 2 C 5 a Copper production | 45.690 | 0.15 | 15 |
| 1 A 3 a i (i) | 1 A 3 a i (i) International aviation (LTO) | 18.200 | 0.06 | 6 |
| 2C1 | 2 C 1 Iron and steel production | 15.829 | 0.05 | 5 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 8.647 | 0.03 | 3 |
| 1A3aii(i) | 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | 7.741 | 0.03 | 3 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 5.282 | 0.02 | 2 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 2.990 | 0.01 | 1 |
| 1A1a | 1 A 1 a Public electricity and heat production | 2.782 | 0.01 | 1 |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 1.650 | 0.01 | 1 |
| 2 C 5 d | 2 C 5 d Zinc production | 1.331 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 1.272 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.107 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.093 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.045 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.006 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.001 | 0.00 | 0 |
| TOTAL 2008 |  | 297.039 | 1 | 100 |

The main sources of Pb emissions are 2C5b Lead production and 2C5a Copper production, which contribute respectively 62% and 15% to national total.

Spreadsheet for the Approach 1 analysis — level assessment of Cd

| NFR category code | NFR category | Latest year estimate Cd, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2 C 5 b | 2 C 5 b Lead production | 1.3095 | 0.36 | 36 |
| 2 C 5 a | 2 C 5 a Copper production | 0.9270 | 0.25 | 25 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.3096 | 0.08 | 8 |
| 2C1 | 2 C 1 Iron and steel production | 0.2662 | 0.07 | 7 |
| 1A1a | 1 A 1 a Public electricity and heat production | 0.2341 | 0.06 | 6 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.1600 | 0.04 | 4 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.1153 | 0.03 | 3 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.1092 | 0.03 | 3 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.1090 | 0.03 | 3 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.0587 | 0.02 | 2 |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 0.0430 | 0.01 | 1 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 0.0150 | 0.00 | 0 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 0.0076 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.0038 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.0034 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.0005 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.0003 | 0.00 | 0 |
| TOTAL 2008 |  | 3.6722 | 1 | 100 |

The main sources of Cd emissions are 2C5b Lead production, 2C5a Copper production, 2C5d Zinc production and 2C1 Iron and steel production, which contribute respectively 36%, 25%, 8% and 7% to national total.

Spreadsheet for the Approach 1 analysis — level assessment of Hg

| NFR category code | NFR category | Latest year estimate Hg, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other | 0.4916 | 0.32 | 32 |
| 1A1a | 1 A 1 a Public electricity and heat production | 0.3680 | 0.24 | 24 |
| 2 C 5 b | 2 C 5 b Lead production | 0.2496 | 0.16 | 16 |
| 2C1 | 2 C 1 Iron and steel production | 0.1706 | 0.11 | 11 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.1090 | 0.07 | 7 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.0604 | 0.04 | 4 |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 0.0500 | 0.03 | 3 |
| 2 C 5 a | 2 C 5 a Copper production | 0.0265 | 0.02 | 2 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.0188 | 0.01 | 1 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.0060 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.0038 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.0013 | 0.00 | 0 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.0002 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.0002 | 0.00 | 0 |
| TOTAL 2008 |  | 1.556 | 1 | 100 |

The main sources of Hg emissions are 1A2fi Stationary combustion in manufacturing industries and construction (32%), 1A1a Public electricity and heat production (24%), 2C5b Lead production (16%) and 2C1 Iron and steel production (11%).

Spreadsheet for the Approach 1 analysis — level assessment of As

| NFR category code | NFR category | Latest year estimate As, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2 C 5 a | 2 C 5 a Copper production | 8.876 | 0.60 | 60 |
| 1A1a | 1 A 1 a Public electricity and heat production | 3.462 | 0.23 | 23 |
| 2 C 5 b | 2 C 5 b Lead production | 1.029 | 0.07 | 7 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.943 | 0.06 | 6 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.214 | 0.01 | 1 |
| 2C1 | 2 C 1 Iron and steel production | 0.113 | 0.01 | 1 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.087 | 0.01 | 1 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.063 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.007 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.004 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.002 | 0.00 | 0 |
| 3C | 3 C Chemical products | 0.000002 | 0.00 | 0 |
| TOTAL 2008 |  | 14.80 | 1 | 100 |

The main sources of As emissions are 2C5a Copper production (60%) and 1A1a Public electricity and heat production (23%).

Spreadsheet for the Approach 1 analysis — level assessment of Cr

| NFR category code | NFR category | Latest year estimate Cr, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 6.26674 | 0.70 | 70 |
| 2C1 | 2 C 1 Iron and steel production | 1.06679 | 0.12 | 12 |
| 1A1a | 1 A 1 a Public electricity and heat production | 0.75072 | 0.08 | 8 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.55041 | 0.06 | 6 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.16037 | 0.02 | 2 |
| 2 C 5 a | 2 C 5 a Copper production | 0.13248 | 0.01 | 1 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.07226 | 0.01 | 1 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.00933 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.00200 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.00124 | 0.00 | 0 |
| 3C | 3 C Chemical products | 0.00002 | 0.00 | 0 |
| TOTAL 2008 |  | 9.01235 | 1 | 100 |

Spreadsheet for the Approach 1 analysis — level assessment of Cu

| NFR category code | NFR category | Latest year estimate Cu, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2 C 5 a | 2 C 5 a Copper production | 71.855 | 0.89 | 89 |
| 1A1a | 1 A 1 a Public electricity and heat production | 4.799 | 0.06 | 6 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 2.119 | 0.03 | 3 |
| 2C1 | 2 C 1 Iron and steel production | 0.858 | 0.01 | 1 |
| 2 C 5 b | 2 C 5 b Lead production | 0.857 | 0.01 | 1 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.540 | 0.01 | 1 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.057 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.055 | 0.00 | 0 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.019 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.004 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.001 | 0.00 | 0 |
| TOTAL 2008 |  | 81.164 | 1 | 100 |

Spreadsheet for the Approach 1 analysis — level assessment of Ni

| NFR category code | NFR category | Latest year estimate Ni, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2 C 5 a | 2 C 5 a Copper production | 14.6387 | 0.70 | 70 |
| 1A1a | 1 A 1 a Public electricity and heat production | 3.5363 | 0.17 | 17 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 1.9842 | 0.10 | 10 |
| 2C1 | 2 C 1 Iron and steel production | 0.2837 | 0.01 | 1 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.2464 | 0.01 | 1 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.0348 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.0289 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.0179 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.0029 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.0020 | 0.00 | 0 |
| 3C | 3 C Chemical products | 0.0002 | 0.00 | 0 |
| TOTAL 2008 |  | 20.7759 | 1 | 100 |

Spreadsheet for the Approach 1 analysis — level assessment of Se

| NFR category code | NFR category | Latest year estimate Se, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 15.5229 | 0.78 | 78 |
| 2 C 5 a | 2 C 5 a Copper production | 3.9744 | 0.20 | 20 |
| 1A1a | 1 A 1 a Public electricity and heat production | 0.2045 | 0.01 | 1 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.0602 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.0536 | 0.00 | 0 |
| 2C1 | 2 C 1 Iron and steel production | 0.0522 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.0032 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.0004 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.0003 | 0.00 | 0 |
| 3C | 3 C Chemical products | 0.000002 | 0.00 | 0 |
| TOTAL 2008 |  | 19.8715 | 1 | 100 |

Spreadsheet for the Approach 1 analysis — level assessment of Zn

| NFR category code | NFR category | Latest year estimate Zn, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2 C 5 a | 2 C 5 a Copper production | 64.5191 | 0.37 | 37 |
| 2C1 | 2 C 1 Iron and steel production | 53.8946 | 0.31 | 31 |
| 2 C 5 d | 2 C 5 d Zinc production | 17.9711 | 0.10 | 10 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 16.4120 | 0.09 | 9 |
| 2 C 5 b | 2 C 5 b Lead production | 15.7173 | 0.09 | 9 |
| 1A1a | 1 A 1 a Public electricity and heat production | 4.3979 | 0.03 | 3 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 1.0967 | 0.01 | 1 |
| 1A1c | 1 A 1 c Manufacture of solid fuels and other energy industries | 0.1091 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.0320 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.0115 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.0016 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.0004 | 0.00 | 0 |
| TOTAL 2008 |  | 174.1633 | 1 | 100 |

Spreadsheet for the Approach 1 analysis — level assessment of PAH

| NFR category code | NFR category | Latest year estimate PAH, Mg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 16.6 | 0.86 | 86 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 2.52 | 0.13 | 13 |
| 1B2a iv | 1 B 2 a iv Refining / storage | 0.162 | 0.01 | 1 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 0.060 | 0.00 | 0 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.013 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.002 | 0.00 | 0 |
| 1 A 3 c | 1 A 3 c Railways | 0.001 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.001 | 0.00 | 0 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.00012 | 0.00 | 0 |
| 2A5 | 2 A 5 Asphalt roofing | 0.00009 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.00003 | 0.00 | 0 |
| 2C1 | 2 C 1 Iron and steel production | 0.0000002 | 0.00 | 0 |
| TOTAL 2008 |  | 19.3594 | 1 | 100 |

The main sources of PAH emissions are 1A4bi  Residential Stationary plants, which contribute 86% to national total.

The next table presents disaggregated sector NFR 1A4bi  to main fuel types, related to PAH emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutant | EF, g/Mg | Emissions (Mg) |
| Woods | 2321050.1 | PAHs | 5.64 | 13.09072256 |
| Black Coal | 237856 | PAHs | 12 | 2.854272 |
| Brown coal | 91303 | PAHs | 5.57 | 0.50855771 |
| Briquettes | 74501 | PAHs | 2.26 | 0.16837226 |
| Naphtha | 842 | PAHs | 3.4 | 0.0028628 |
| Total NFR 1A4bi |  | PAHs |  | 16.62478733 |

Spreadsheet for the Approach 1 analysis — level assessment of HCB

| NFR category code | NFR category | Latest year estimate HCB, kg | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 2C1 | 2 C 1 Iron and steel production | 26.089 | 0.99 | 99 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.13 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.073 | 0.00 | 0 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.048 | 0.00 | 0 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.038 | 0.00 | 0 |
| TOTAL 2008 |  | 26.377 | 1 | 100 |

Spreadsheet for the Approach 1 analysis — level assessment of DIOX

| NFR category code | NFR category | Latest year estimate DIOX, g | Level Assessment Lx,t | Comulative total [%] |
| --- | --- | --- | --- | --- |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 14.400 | 0.27 | 27 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 11.700 | 0.22 | 22 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 9.415 | 0.18 | 18 |
| 2C1 | 2 C 1 Iron and steel production | 7.717 | 0.15 | 15 |
| 2 C 5 a | 2 C 5 a Copper production | 2.915 | 0.06 | 6 |
| 1A1a | 1 A 1 a Public electricity and heat production | 2.259 | 0.04 | 4 |
| 6Cb | 6 C b Industrial waste incineration (d) | 1.090 | 0.02 | 2 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 0.865 | 0.02 | 2 |
| 2 C 5 d | 2 C 5 d Zinc production | 0.539 | 0.01 | 1 |
| 2 C 5 b | 2 C 5 b Lead production | 0.493 | 0.01 | 1 |
| 1 A 3 c | 1 A 3 c Railways | 0.352 | 0.01 | 1 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 0.230 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.192 | 0.00 | 0 |
| 2 C 3 | 2 C 3 Aluminum production | 0.146 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.050 | 0.00 | 0 |
| 2D1 | 2 D 1 Pulp and paper | 0.005 | 0.00 | 0 |
| 1A4ai | 1 A 4 a i Commercial / institutional: Stationary | 0.003 | 0.00 | 0 |
| 1A4ci | 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.002 | 0.00 | 0 |
| TOTAL 2008 |  | 52.374 | 1 | 100 |

The main sources of DIOX emissions are 1A4bi  Residential Stationary plants (27%), 1A5b Other, Mobile (22%), 1A2a Stationary combustion in manufacturing industries and construction: Iron and steel (18%) and  2C1 Iron and steel production 15%.

The next table presents disaggregated sector NFR 1A4bi  to main fuel types, related to DIOX emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutant | EF, µg/Mg | Emissions (g) |
| Woods | 2321050.1 | DIOX | 4.7 | 10.91 |
| Black Coal | 237856 | DIOX | 11.8 | 2.81 |
| Brown coal | 91303 | DIOX | 5.46 | 0.50 |
| Briquettes | 74501 | DIOX | 3 | 0.22 |
| Naphtha | 842 | DIOX | 0.45 | 0.0004 |
| Total NFR 1A4bi |  | DIOX |  | 14.44 |

Spreadsheet for the Approach 1 analysis — level assessment of PCBs

| NFR category code | NFR category | Latest year estimate PCBs, kg | Level Assessment Lx,t | Comulative total |
| --- | --- | --- | --- | --- |
| 1A4bi | 1 A 4 b i Residential: Stationary plants | 162.7 | 0.74 | 74 |
| 1A3bi | 1 A 3 b i Road transport: Passenger cars | 45.5 | 0.21 | 21 |
| 1A5b | 1 A 5 b Other, Mobile (including military, land based and recreational boats) | 11.7 | 0.05 | 5 |
| 1A2fi | 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 1.0173 | 0.00 | 0 |
| 1A2a | 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 0.2293 | 0.00 | 0 |
| 6Cb | 6 C b Industrial waste incineration (d) | 0.0180 | 0.00 | 0 |
| 1A1b | 1 A 1 b Petroleum refining | 0.0016 | 0.00 | 0 |
| 6Ca | 6 C a Clinical wasteincineration (d) | 0.0006 | 0.00 | 0 |
| TOTAL 2008 |  | 221.167 | 1 | 100 |

The main sources of PCBs emissions are 1A4bi  Residential Stationary plants (73.6%) and 1A3b Road transport (20.6%).

The next table presents disaggregated sector NFR 1A4bi  to main fuel types, related to PCBs emissions:

| Disaggregation of NFR 1A4bi to main fuel types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel types | Activity data, Mg | Pollutants | EF, mg/Mg | Emissions (kg) |
| Woods | 2321050.1 | PCBs | 59.4 | 137.87 |
| Brown coal | 91303 | PCBs | 183.2 | 16.7267 |
| Black Coal | 237856 | PCBs | 31.6 | 7.5162 |
| Briquettes | 74501 | PCBs | 8.1 | 0.60346 |
| Naphtha | 842 | PCBs | 0.6 | 0.0005 |
| Total NFR 1A4bi |  | PCBs |  | 162.7173 |

## 1.6 QA/QC and Verification methods (to be updated every 5 years)

Quality assurance and quality control system

The  Executive Environment Agency is responsible for the preparation of the National Emissions Inventories and the relevant National Inventory Reports under UNFCCC and CLARTAP.

The ExEA is also responsible for coordinating QA/QC activities for the national inventory.

The QA/QC plan is a basic element of the QA/QC system. The plan outlines QA/QC activities that are implemented and includes the scheduled time frame for inventory preparation from its initial development through the final reporting in any year. It contains an outline of the processes and schedule to review of all source categories.

The QA/QC plan is an internal document to organise, plan and implement QA/QC activities. Once developed, it is referenced and used in subsequent inventory preparation, or modified as appropriate.

The official QA/QC Plan for National emissions inventories was approved by the Ministry of Environment and Water in 2009. It was created on the basis of Quality Management System developed on the basis of an National Study of Bulgarian Academy of Science, Geophysical Institute.

The QA/QC plan is provided to all institutions, which are engaged in the process of preparation of emissions inventories under UNFCCC and UNECE/CLRTAP for implementation.

QA/QC Plan includes following elements:

- Responsible institutions;

- QA/QC Procedures;

- Timetable with the relevant deadlines.

The cycle of QA/QC activity for inventory consists of 6 steps:

Step 1. QA/QC Manager develops QA/QC Plan. Inventory Team Leader approves it. QA/QC Manager reviews the draft attendant file (AF) for the previous inventory and prepares preliminary AF for current inventory.

Step 2. According to QA/QC Plan the inventory (or part of inventory) is sent for check and review.

Step 3. QA/QC Manager obtains the results of check and review.

Step 4. QA/QC Manager registers findings in the checklist (CL) and AF, as well as processes this information and marks decision about corrective actions.

Step 5. The person who is responsible for corrective actions carries out this work and reports to QA/QC Manager for updating checklist attendant file.

Step 6.  QA/QC Manager prepares the QA/QC chapter to the Inventory Report and a draft AF for the next inventory..

Quality Control (QC) is a system of routine technical activities to assess and maintain the quality of the inventory as it is being compiled. It is performed by personnel compiling the inventory. The QC system is designed to:

- Provide routine and consistent checks to ensure data integrity, correctness, and completeness;

- Identify and address errors and omissions;

- Document and archive inventory material and record all QC activities.

General (QC) procedures that all source categories follow when gathering, maintaining, handling, documenting, checking and archiving the data, supporting documents, and files (both text documents and spreadsheets) are associated with the inventory.

General (QC) procedures are described in Checklists that is part of QA/QC Plan.

Quality Assurance (QA) is a planned system of review procedures conducted by personnel not directly involved in the inventory compilation/development process. The quality assurance process includes expert review was conducted in two stages: a review of the initial set of emission estimates and, a review of the estimates and text of the Inventory Report.

The expert peer review present opportunity to uncover technical issues related to the application of methodologies, selection of activity data, or the development and choice of emission factors. The comments received during these processes are reviewed and, as appropriate, incorporated into the Inventory Report or reflected in the inventory estimates.

Internal Quality Audits - an audit system will be developed to provide additional QA measures.

QA/QC Manager maintains and archives all documentation for every inventory produced. It includes all final and worksheet files that are created during the inventory preparation. The archived documents are stored on server and in inventory archive (paper).

## 1.7 General uncertainty evaluation (to be updated every 5 years)

The overall uncertainty is closely related to the emission sources data uncertainty (fuels, activities, processes, etc.) and to the emission factor uncertainty.

The uncertainty of the emission sources can be defined during data collection and processing, and it is a part of procedures, applied by the NSI and its regional structures. Different criteria for uncertainty assessment are used, for example as statistical subtraction, on basis on differences between the production, import, export and consumption of fuels, through expert assessments, etc. The uncertainty of the emission factors depends on the origin of the factors applied.

## 1.8 General Assessment of Completeness (to be updated each year):

### i) Sources Not Estimated (NE)

The emissions of air pollutants for 2008 are calculated based on the activity data and emission factors. The emission factors are set in the national methodology for emissions inventory under UNECE/CLRTAR and UNFCCC (see point 1.4). Collecting of the necessary activity data is responsibility of the NSI.

The number of not estimated sources/pollutants in the Emissions inventory 2008 are reduced compared to the previous year submission. Based on the recomendations of the ERT set in the report from stage 3 in-depth review for some categories/pollutants, which are not well developed in the national methodology, EFs from EMEP/CORINAIR Emission Inventory Guidebook 2009 were applied in order to improve the inventory. Such improvements are made in the following sources: 2A1, 2A2, 2A4, 2A6, 2B1, 2B5a, 2C2, 2D1, 3C, 4B which is presented in the sectoral chapters.

The next table presents the NFR source categories, which continue to be problematic for assessment, because of the data gaps (lack of activity data and/or EFs).

| NFR09 code | Substance(s) | Reason for not estimated |
| --- | --- | --- |
| 1 A 3 b vi | All | Lack of activity data and/or EFs |
| 1 A 3 b vii | All | Lack of activity data and/or EFs |
| 1 A 3 d i (ii) | All | Lack of activity data and/or EFs |
| 1 A 3 e | All | Lack of activity data and/or EFs |
| 1 A 3 b v | All | Lack of activity data and/or EFs |
| 1A 4 c iii | All | Lack of activity data and/or EFs |
| 1 B 1 c | All | Lack of activity data and/or EFs |
| 1 B 3 | All | Lack of activity data and/or EFs |
| 2 A 7 a | All | Lack of activity data and/or EFs |
| 2 A 7 b | All | Lack of activity data and/or EFs |
| 2 A 7 c | All | Lack of activity data and/or EFs |
| 2 C 5 e | All | Lack of activity data and/or EFs |
| 2 C 5 f | All | Lack of activity data and/or EFs |
| 2 D 3 | All | Lack of activity data and/or EFs |
| 2 E | All | Lack of activity data and/or EFs |
| 2 F | All | Lack of activity data and/or EFs |
| 2 G | All | Lack of activity data and/or EFs |
| 3 D 2 | All | Lack of activity data and/or EFs |
| 4 B 13 | All | Lack of activity data and/or EFs |
| 4 D 2 a | All | Lack of activity data and/or EFs |
| 4 D 2 b | All | Lack of activity data and/or EFs |
| 4 D 2 c | All | Lack of activity data and/or EFs |
| 4 G | All | Lack of activity data and/or EFs |
| 6 C d | All | Lack of activity data and/or EFs |
| 6 C e | All | Lack of activity data and/or EFs |

### ii) Sources Included Elsewhere (IE)

The next table presents the NFR source categories, which are included elsewhere in the reporting table. The main reason for such aggregation is lack of representative activity data for separate assessment.

| NFR09 code | Substance(s) | Included in NFR code |
| --- | --- | --- |
| 1 A 2 b | All | 2C |
| 1 A 2 c | All | 1A2a |
| 1 A 2 e | All | 1A2a |
| 1 A 2 f ii | All | 1A5b National total - Other mobile source and machinery |
| 1 A 3 b ii | All | 1A3bi National total Road Transport |
| 1 A 3 b iii | All | 1A3bi National total Road Transport |
| 1 A 3 b iv | All | 1A3bi National total Road Transport |
| 1 A 3 d ii | All | 1A5b National total - Other mobile source and machinery |
| 1 A 4 a ii | All | 1A5b National total - Other mobile source and machinery |
| 1 A 4 b ii | All | 1A5b National total - Other mobile source and machinery |
| 1 A 4 c ii | All | 1A5b National total - Other mobile source and machinery |
| 1 A 5 a | All | 1A4a |
| 1 B 1 b | All | 1A1c |
| 4B2 | NH3 | 4B1b |
| 4B4 | NH3 | 4B3 |
| 4B7 | NH3 | 4B6 |
| 4 B 9 b | NH3 | 4 B 9 d |
| 4 B 9 c | NH3 | 4 B 9 d |

### iii) Other notation keys

Sub-sources accounted for in reporting codes "other"

| NFR09 code | Substance(s) reported | Sub-source description |
| --- | --- | --- |
| 1 A 2 fi | All | 030204 Plaster furnaces 030311 Cement 030312 Lime (include. Iron and steel and paper pulp industry) 030313 Asphalt concrete plants 030314 Flat glass 030315 Container glass 030317 Other glass 030319 Bricks and tiles 030320 Fine ceramic materials |
| 1 A 5 a | All | Included in 1A4a |
| 1 A 5 b | All | 08 Other mobile source and machinery National total |
| 1 B 1 c | All | Not estimated |
| 1 B 3 | All | Not estimated |
| 2 A 7 d | All | 040613 Glass (decarbonizing) |
| 2 B 5 a | All | 040401 Sulfuric acid 040404 Ammonium sulphate 040405 Ammonium nitrate 040406 Ammonium phosphate 040407 NPK fertilisers 040413 Chlorine production 040414 Phosphate fertilizers 040415 Storage and handing of inorganic chemical products 040501 Ethylene 040502 Propylene 040507 Polyethylene – High Density 040509 Polypropylene 040511 Polystyrene 040514 Styrene-butadiene rubber (SBR) 040515 Acrylonitrile butadiene styrene (ABS) resins 040517 Formaldehyde 040519 Phtalic anhydride 040520 Acrylonitrile |
| 2 C 5 e | All | Not estimated |
| 2 G | All | Not estimated |
| 3 A 3 | All | 060102 060107 060109 060110 |
| 3 D 3 | All | 060404 060405 060406 |
| 4 B 13 | All | Not estimated |
| 4 G | All | Not estimated |
| 6 D | All | 091003 Sludge spreading |
| 7 A | All | Not estimated |

Chapter 2: Explanation of key trends (to be updated each year)

There are changes in the time trend for key categories and pollutants and the reasons for these changes are as follows:

- Till inventory year 2006 the emissions of air pollutants were estimated based on the CORINAIR 94 methodology. Changes in the time trend for key categories due mainly to the implementation of new national common methodology for emissions inventory under UNECE/CLRTAR and UNFCCC (see point 1.4).

- The other reason for some of the changes is the implementation of the recomendation of ERT after stage 3 in-depth review in 2008 inventory.

- Bulgaria has not submitted yet Recalculations of previously submitted estimates of emissions.

- In the end of 2009 the MEW approved a schedule for two projects, whose defined objectives are to improve the Party’s current estimation methodologies, the technical competence of the staff within the national system, and the management and archiving of data. Both projects are already in progress:

- Project 1 – “Recalculations of the previously submitted estimates of emissions under UNFCCC and the European Monitoring and Evaluation Programme/Convention on Long-range Transboundary Air Pollution (EMEP/CLRTAP) according to the new Common Methodology from the base year up to the year in which recalculations are made and cover all inventory data”.  This project started on the 1st of December 2009 and will be completed by the 1st of February 2011. The result of this project will be incorporated into the 2011 submission.

- Project 2 – “Development of software tool and automatic preparation of national inventories under UNFCCC and UNECE/CLRTAP”. This project is to commence on 28 December 2009 and will be completed by 28 December 2010. The result of this project will be incorporated into the 2011 submission.

- Bulgaria will use these projects to improve the capacity of the national system in planning, preparing and managing its annual submissions, to improve the technical competence of the staff, and to fulfil the requirements for Recalculations of the previously submitted estimates.

- Because of the above mentioned reasons the key trends are prepared for 2007 and 2008 only.

There are no significant changes in the time trend for key categories of NO2, SO2, CO, NMVOC, PCBs, DIOX, PAH, HCB in 2008 inventory compared to the previous one.

Main sectors (sorted by largest contribution to total for 2007 and 2008):

| Year | Component | Main sectors (sorted by largest contribution to total) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2007 | NO2 | 1A1a | 1A3b | 1A5b | 2B2 | 1A4b i | 1A2a | 1A2f |
| 2008 |  | 1A1a | 1A3b | 1A5b | 2B2 | 1A2a | 1A4bi |  |
| 2007 | CO | 1A4b i | 1A3b | 2C | 1A5b | 2B1 |  |  |
| 2008 |  | 1A4bi | 1A3b | 2C1 | 1A5b | 2B1 |  |  |
| 2007 | NMVOC | 1A4b i | 1A3b | 4D1 | 4F | 1A5b | 3D | 3A |
| 2008 |  | 1A4bi | 1A3b | 4D1a | 4F | 1A5b | 3D3 | 2D2 |
| 2007 | SO2 | 1A1a | 1A5b | 2B5 | 1A1b | 1A1c |  |  |
| 2008 |  | 1A1a | 1A5b | 2B5a | 1A2fi | 1A2a |  |  |
| 2007 | NH3 | 6A | 4B1a | 4B3 | 4D1 | 4B8 | 4B1b | 4B6 |
| 2008 |  | 4B1a | 6A | 4D1a | 4B1b | 4B8 | 4B9d | 4B3 |
| 2007 | Pb | 1A2b | 1A3aii(ii) | 1A3aii(i) | 2C | 1A2a | 1A2f | 1A1a |
| 2008 |  | 2 C 5 b | 2 C 5 a | 1 A 3 a i (i) | 2C1 | 1A2fi | 1A3aii(i) | 1A2a |
| 2007 | Cd | 1A2b | 2C | 1A1a | 1A2a | 1A1c | 1A2f | 6C |
| 2008 |  | 2 C 5 b | 2 C 5 a | 2 C 5 d | 2C1 | 1A1a | 1A2fi | 1A2a |
| 2007 | Hg | 1A1a | 1A2f | 1A2b | 2C | 1A2a | 6C | 1A4b i |
| 2008 |  | 1A2fi | 1A1a | 2 C 5 b | 2C1 | 6Cb | 1A2a | 1A4bi |
| 2007 | PCBs | 1A4b i | 1A3b | 1A5b |  |  |  |  |
| 2008 |  | 1A4bi | 1A3b | 1A5b |  |  |  |  |
| 2007 | DIOX | 1A2a | 1A4b i | 1A5b | 2C | 1A2b | 1A1a | 6C |
| 2008 |  | 1A4bi | 1A5b | 1A2a | 2C1 | 2 C 5 a | 1A1a | 6Cb |
| 2007 | PAH | 1A4b i | 1A5b | 1B2a iv |  |  |  |  |
| 2008 |  | 1A4bi | 1A5b | 1B2a iv |  |  |  |  |
| 2007 | НСВ | 2C |  |  |  |  |  |  |
| 2008 |  | 2C1 |  |  |  |  |  |  |

The decrease of NH3 emissions from NFR 6A Solid waste disposal on land with almost 50% is due to implementation of the requirements of National program for implementation of Directive  1999/31/ЕС in the 2008 inventory.

Based on the recommendations of ERT in 2008 inventory the emissions of HM are presented more precisely in NFR 2C5. These emissions were included in 1A2b in the previous inventories.

The decrease of DIOX emissions from NFR 1A2a is due mainly to the significant decrease of the production in SNAP 030301 Sinter plants. The largest plant for the production of ferrous metals - plant MK “Kremikovci”, was in process of closing at that time.

Chapters 3 – 9 Sectoral Methodologies:

Chapter 3: ENERGY (NFR sector 1)

## (to be updated when there have been major changes since the last report)

Methodology

The emissions of air pollutants in Energy (NFR sector 1) for 2008 are calculated based on the activity data and emission factors. The latters are set in the national methodology for emissions inventory under UNECE/CLRTAR and UNFCCC (see point 1.4). This methodology is harmonized with EMEP/CORINAIR Emission Inventory Guidebook.

Collecting of the necessary activity data is responsibility of NSI. NSI uses up-to-date statistical methods and procedures for data collection, summarizing and structuring that are harmonized with EUROSTAT. The activity data for Energy (NFR sector 1) are presented in the reporting table.

The emissions from combustion processes are divided to the following sub-sectors:

- Energy Industries (1A1);

- Manufacturing Industries and Construction (1A2);

- Transport (1A3);

- Other sectors - Commercial, Households, Agriculture and Forestry (1A4);

- Other (1A5).

The fugitive emissions are:

- Coal Mining (1B1);

- Extraction, Transportation and Distribution of Petrol Products and Natural Gas (1B2).

Sub-sector Energy Industries included the groups:

- Public Electricity and Heat Production (1A1a);

- Petroleum Refining (1A1b);

- Manufacture of Solid fuels Production and Other Energy Industries (1A1c).

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 1A1a | 1A1a Public electricity and heat production | 010101 010102 010103 010105 010202 010203 010204 010205 |
| 1A1b | 1A1b Petroleum refining | 010302 010303 010306 |
| 1A1c | 1A1c Manufacture of solid fuels and other energy industry | 010406 |

Sub-sector Manufacturing Industries and Construction included the groups:

- Ferrous Metallurgy (1A2a);

- Non-ferrous Metallurgy (1A2b);

- Chemical Industry (1A2c);

- Pulp and Paper Production and Printing Industry (1A2d);

- Food Industry (1A2e);

- Other (1A2f).

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 1A2a | 1A2a Iron and steel | 030101 030102 030103 030104 030105 030203 030301 030302 030303 |
| 1A2b | 1A2b Non-ferrous metals | 030304 030305 030306 030307 030308 030309 030310 |
| 1A2c | 1A2c Chemicals | Included in 1A2a |
| 1A2d | 1A2d Pulp, paper and print | 030321 |
| 1A2e | 1A2e Food processing, beverages and tobacco | Included in 1A2a |
| 1A2fi | 1A2f Other | 030204 030311 030312 030313 030314 030315 030317 030319 030320 |
| 1 A 2 f ii | 1 A 2 f ii Mobile Combustion in Manufacturing Industries and Construction: | Included in 1A5b |

Sub-sector Transport includes the following groups: air, sea, road, inland waterway transport and other kinds of transport. The group Other transport includes emission sources form agriculture and construction, such as: agriculture machinery for land cultivation, wood processing machinery, construction machinery, etc. The latter are so-called off-road machines. This type of machines is using basically diesel fuel. The aggregation level in transport is by fuel type.

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 1A3aii(i) | 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | 080501 |
| 1A3ai (i) | 1 A 3 a i (i) International aviation (LTO) | 080502 |
| 1A3bi | 1A3b i R.T., passenger cars | 07 Road Transport National total |
| 1A3b ii | 1A3b ii R.T., light duty vehicles | Included in 1A3bi |
| 1A3b iii | 1A3b iii R.T., heavy duty vehicles | Included in 1A3bi |
| 1A3b iv | 1A3b iv R.T., Mopeds and motorcycles | Included in 1A3bi |
| 1A3bv | 1A3bv R.T., Gasoline evaporation | Included in 1A3bi |
| 1A3b vi | 1A3b vi R.T., Automobile tyre and brake wear | NE |
| 1A3b vii | 1A3b vii R.T., Automobile road abrasion | NE |
| 1A3c | 1A3c Railways | 0802 |
| 1A3d ii | 1A3d ii National navigation | Included in 1A5b |
| 1A3e i | 1A3e i Pipeline compressors | NE |
| 1A4ai | 1A4ai Commercial / Institutional: Stationary | 020103 020105 |
| 1A4a ii | 1A4a ii Commercial / Institutional: Mobile | Included in 1A5b |
| 1A4b i | 1A4b i Residential plants | 020201 020202 020203 020204 020205 |
| 1A4b ii | 1A4b ii Household and gardening (mobile) | Included in 1A5b |
| 1A4c i | 1A4c i Stationary | 020302 020304 |
| 1A4c ii | 1A4c ii Off-road vehicles and other machinery | Included in 1A5b |
| 1A4c iii | 1A4c iii National fishing | NE |
| 1A5a | 1A5a Other, Stationary (including military) | Included in 1A4a |
| 1A5b | 1A5b Other, Mobile (including military) | 08 Other mobile source and machinery National total |

NFR 1 A 3 a ii (i) Civil aviation (Domestic, LTO)

The next table presents calculation of emissions from NFR 1A3aii Civil aviation (domestic, LTO).

| NFR 1A3aii Civil aviation (domestic, LTO) |  |  |  |
| --- | --- | --- | --- |
| Component | Number of LTO | National methodology | Emissions |
|  |  |  | Mg |
|  |  | Ef t/LTO |  |
| SOx (SO2) | 12902 | 0.0008 | 10.322 |
| NMVOC | 12902 | 0.0005 | 6.451 |
| CO | 12902 | 0.0118 | 152.244 |
| NOx | 12902 | 0.0083 | 107.087 |
| PM2.5 | 12902 | 0.00001 | 0.129 |
| Pb | 12902 | 0.0006 | 7.741 |
| PAH | 12902 | 1.2E-10 | 0.000 |

NFR 1 A 3 a i (i) International aviation (LTO)

The next table presents calculation of emissions from NFR 1A3ai(i) International aviation (LTO).

| Component | Number of LTO | National methodology | Emissions |
| --- | --- | --- | --- |
|  |  |  | Mg |
|  |  | Ef t/LTO |  |
| SOx | 30294 | 0.0008 | 24.235 |
| NMVOC | 30294 | 0.0005 | 15.147 |
| CO | 30294 | 0.0118 | 357.469 |
| NOx | 30294 | 0.0083 | 251.440 |
| PM2.5 | 30294 | 0.00001 | 0.303 |
| Pb | 30294 | 0.0006 | 18.176 |
| PAH | 30294 | 1.2E-10 | 0.000 |

NFR 1A3b

The next table presents the EFs used for assessment of air emissions in sector NFR 1A3b National total Road transport.

| NFR 1A3b |  | National methodology |
| --- | --- | --- |
| Component | Type of Fuel | EFs Mg/Mg |
| SOx | Benzin | 0.0001 |
|  | Diesel | 0.0001 |
|  | LPG | - |
| NOx | Benzin | 0.01609 |
|  | Diesel | 0.02503 |
|  | LPG | 0.0483 |
| NMVOC | Benzin | 0.04039 |
|  | Diesel | 0.00409 |
|  | LPG | 0.0185 |
| CO | Benzin | 0.2378 |
|  | Diesel | 0.0138 |
|  | LPG | 0.0643 |
| PM10 | Benzin | - |
|  | Diesel | 0.00348 |
|  |  | EFs g/Mg |
| Hg | Benzin | - |
|  | Diesel | - |
| Cd | Benzin | 0.01 |
|  | Diesel | 0.01 |
| Pb | Benzin | 4.99 |
|  | Diesel | - |
| PAHs | Benzin | 0.01 |
|  | Diesel | 0.055 |
|  |  | EF, µg/Mg |
| DIOX | Benzin | 0.36 |
|  | Diesel | 0.02 |
|  |  | EF, mg/Mg |
| PCBs | Benzin | 52.7 |
|  | Diesel | 15.4 |

The next table presents fuel consumptions in sector NFR 1A3b for 2006, 2007 and 2008.

| NFR 1A3b | Fuel consumption Road Transport 2006 | Fuel consumption Road Transport 2007 | Fuel consumption Road Transport 2008 |
| --- | --- | --- | --- |
| Type of Fuel | Mg | Mg | Mg |
| Benzin | 605175 | 596772 | 598461 |
| Diesel | 975422 | 837741 | 905738 |
| LPG | 297519 | 262994 | 342628 |

NFR 1A3c

Till inventory year 2007 the emissions from NFR 1A3c Railways were presented in national total of Other mobile source and machinery. In 2008 this emission source is estimated separately. The next table presents the EFs used for assessment of air emissions in sector NFR 1A3c Railways. In 2008 the consumption of fuel (diesel) in NFR 1A3c was 32294 Mg.

| NFR 1A3c | Type of Fuel | National methodology |
| --- | --- | --- |
| Component |  | Ef kg/Mg |
| SOx | Diesel | 0 |
| NMVOC | Diesel | 4.65 |
| NOx | Diesel | 39.6 |
| CO | Diesel | 10.7 |
| NH3 | Diesel | 0.007 |
| TSP | Diesel | 4.58 |
| PM10 | Diesel | 1.44 |
| PM2.5 | Diesel | 1.37 |
|  |  | Ef µg/Mg |
| DIOX | Diesel | 10.9 |
|  |  | Ef mg/kg |
| Cd | Diesel | 0.01 |
| Cu | Diesel | 1.7 |
| Cr | Diesel | 0.05 |
| Ni | Diesel | 0.07 |
| Se | Diesel | 0.01 |
| Zn | Diesel | 1 |
|  |  | Ef g/Mg |
| PAH | Diesel | 0.03 |

NFR 1A5b

The next table presents the EFs used for assessment of air emissions in sector NFR 1A5b Other mobile source and machinery - national total. In 2008 the consumption of fuel diesel in NFR 1A5b was 758456 Mg.

| NFR 1A5b |  | National methodology |
| --- | --- | --- |
| Component | Type of Fuel | EFs Mg/Mg |
| SOx (SO2) | Diesel | 0.04 |
| NMVOC | Diesel | 0.008 |
| NOx | Diesel | 0.05 |
| CO | Diesel | 0.02 |
| NH3 | Diesel | 0.000005 |
| PM 10 | Diesel | 0.004 |

Other Sectors (NFR 1A4)

Sub-sector Other sectors includes the groups:

- Commercial/Institutional (1A4a);

- Residential (1A4b);

- Agriculture/Forestry/Fisheries (1A4c).

These groups include only stationary sources, as the aggregation level is the type of the fuel and the combustion technology in the corresponding group (services, households, agriculture).

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 1A4ai | 1A4ai Commercial / Institutional: Stationary | 020103 020105 |
| 1A4a ii | 1A4a ii Commercial / Institutional: Mobile | Included in 1A5b |
| 1A4b i | 1A4b i Residential plants | 020201 020202 020203 02 02 04 020205 |
| 1A4b ii | 1A4b ii Household and gardening (mobile) | Included in 1A5b |
| 1A4c i | 1A4c i Stationary | 020302 020304 |
| 1A4c ii | 1A4c ii Off-road vehicles and other machinery | Included in 1A5b |
| 1A4c iii | 1A4c iii National fishing | NE |

The next table presents the EFs used for assessment of air emissions in sector NFR 1A4bi Residential plants.

| NFR 1A4bi Residential plants |  |  |
| --- | --- | --- |
| Type of Fuel | Component | National methodology |
|  |  | EF, g/GJ |
| Brown coal | SOx | 840 |
|  | NOx | 160 |
|  | NMVOC | 300 |
|  | TSP | 350 |
|  | CO | 4000 |
|  | PM10 | 240 |
|  | PM2.5 | 330 |
|  |  | EF, g/Mg |
|  | Hg | 0.055 |
|  | Cd | 0.02 |
|  | Pb | 1.47 |
|  | PAHs | 5.57 |
|  |  | EF, µg/Mg |
|  | DIOX | 5.46 |
|  |  | EF, mg/Mg |
|  | PCBs | 183.2 |
|  |  | EF, g/GJ |
| Briquettes | SOx | 840 |
|  | NOx | 150 |
|  | NMVOC | 200 |
|  | TSP | 120 |
|  | CO | 3000 |
|  | PM10 | 100 |
|  | PM2.5 | 100 |
|  |  | EF, g/Mg |
|  | Hg | 0.045 |
|  | Cd | 0.01 |
|  | Pb | 1.5 |
|  | PAHs | 2.26 |
|  |  | EF, µg/Mg |
|  | DIOX | 3 |
|  |  | EF, mg/Mg |
|  | PCBs | 8.1 |
|  |  | EF, g/GJ |
| Woods | SOx | 30 |
|  | NOx | 120 |
|  | NMVOC | 925 |
|  | TSP | 730 |
|  | CO | 4000 |
|  | PM10 | 695 |
|  | PM2.5 | 475 |
|  |  | EF, g/Mg |
|  | Hg | 0.005 |
|  | Cd | 0.013 |
|  | Pb | 0.28 |
|  | PAHs | 5.64 |
|  |  | EF, µg/Mg |
|  | DIOX | 4.7 |
|  |  | EF, mg/Mg |
|  | PCBs | 59.4 |
|  |  | EF, g/GJ |
| Naphta | SOx | 140 |
|  | NOx | 70 |
|  | NMVOC | 15 |
|  | TSP | 27.5 |
|  | CO | 70 |
|  | PM10 | 21.5 |
|  | PM2.5 | 16.5 |
|  |  | EF, g/Mg |
|  | Hg | 0.001 |
|  | Cd | 0.07 |
|  | Pb | 0.72 |
|  | PAHs | 3.4 |
|  |  | EF, µg/Mg |
|  | DIOX | 0.45 |
|  |  | EF, mg/Mg |
|  | PCBs | 0.6 |
|  |  | EF, g/GJ |
| LPG | SOx | 0.5 |
|  | NOx | 57 |
|  | NMVOC | 5 |
|  | TSP | 0.5 |
|  | CO | 25 |
|  | PM10 | 0.5 |
|  | PM2.5 | 0.5 |
| Black coals |  | EF, g/Mg |
|  | Hg | 0.118 |
|  | Cd | 0.042 |
|  | Pb | 3.18 |
|  | PAHs | 12 |
|  |  | EF, µg/Mg |
|  | DIOX | 11.8 |
|  |  | EF, mg/Mg |
|  | PCBs | 31.6 |

Manufacturing Industries and Construction (NFR 1A2)

Sub-sector Manufacturing Industries and Construction included the groups:

- Ferrous Metallurgy (1A2a);

- Non-ferrous Metallurgy (1A2b);

- Chemical Industry (1A2c);

- Pulp and Paper Production and Printing Industry (1A2d);

- Food processing, beverages and tobacco (1A2e);

- Other (1A2f).

The group Other included machinery construction, electrical engineering, light industry and auto-generating plants for combined production of electrical and thermal energy. The aggregation level of sub-sector Manufacturing industries and construction is mainly the fuel type, as in some cases the type of combustion technology.

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 1A2a | 1A2a Iron and steel | 030101 030102 030103 030104 030105 030203 030301 030302 030303 |
| 1A2b | 1A2b Non-ferrous metals | Included in 2C |
| 1A2c | 1A2c Chemicals | Included in 1A2a |
| 1A2d | 1A2d Pulp, paper and print | 030321 |
| 1A2e | 1A2e Food processing, beverages and tobacco | Included in 1A2a |
| 1A2fi | 1A2fi Other | 030204 030311 030312 030313 030314 030315 030317 030319 030320 |
| 1 A 2 f ii | 1 A 2 f ii Mobile Combustion in Manufacturing Industries and Construction: | Included in 1A5b |

The fugitive emissions:

- Coal Mining (1B1);

- Extraction, Transportation and Distribution of Petrol Products and Natural Gas (1B2).

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 1B1a | 1B1a Coal mining and handing | 050101 050102 050103 |
| 1B1b | 1B1b Solid fuel transformation | Included in 1A1c |
| 1B1c | 1B1c Other | NE |
| 1B2a i | 1B2a i Exploration production, transport | 050201 |
| 1B2a iv | 1B2a iv Refining/Storage | 040101 040102 |
| 1B2a v | 1B2a v Distribution of oil products | 050501 050502 050503 |
| 1B2a vi | 1B2a vi Geothermal energy extraction | NE |
| 1B2b | 1B2b Natural gas | 050302 050303 050601 |
| 1B2c | 1B2c Venting and flaring | 090203 |

Source specific QA/QC and verification

All activities regarding QC as described in QA/QC System have been undertaken in Energy (NFR sector 1).

Source-specific planned improvements

To improve the accuracy of the estimates in Energy (NFR sector 1).

To update the national methodology with the requirements of the last version of EMEP/CORINAIR Emission Inventory Guidebook 2009.

To improve transparency, completeness, consistency, including recalculations and time-series and comparability of national emission inventory (see Chapter 10).

Chapter 4: INDUSTRIAL PROCESSES (NFR sector 2)

## (to be updated when there have been major changes since the last report)

Methodology

The emissions of air pollutants in Industrial processes (NFR sector 2) for 2008 are calculated based on the activity data and emission factors. The latter are set in the national common methodology for emissions inventory under UNECE/CLRTAR and UNFCCC (see point 1.4). The methodology is harmonized with EMEP/CORINAIR Emission Inventory Guidebook.  The collection of the necessary activity data is responsibility of NSI. NSI uses up-to-date statistical methods and procedures for data collection, summarizing and structuring that are harmonized with EUROSTAT.

The activity data for Industrial processes (NFR sector 2) are presented in the reporting table.

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 2A1 | 2A1 Cement Production | 040612 |
| 2 A2 | 2A2 Lime Production | 040614 |
| 2A 3 | 2 A3 Limestone and Dolomite Use | 040618 |
| 2 A 4 | 2A4 Soda Ash Production and use | 040619 |
| 2A 5 | 2A5 Asphalt Roofing | 040610 |
| 2A 6 | 2A6 Road Paving with Asphalt | 040611 |
| 2A7 a | 2 A 7 a Quarrying and mining of minerals other than coal | NE |
| 2A7 b | 2 A 7 b Construction and demolition | NE |
| 2A7 c | 2A 7 c Storage, handling and transport of mineral products | NE |
| 2A7d | 2 A 7 d Other Mineral products | 040613 |
| 2B1 | 2B1 Ammonia Production | 040403 |
| 2B2 | 2B2 Nitric Acid Production | 040402 |
| 2B3 | 2B3 Adipic Acid Production | NO |
| 2B4 | 2B4 Carbide Production | 040412 |
| 2B5a | 2B5a Other chemical industry | 04 04 01 04 04 04 04 04 05 04 04 06 04 04 07 04 04 13 04 04 14 04 04 15 04 05 01 04 05 02 04 05 07 04 05 09 04 05 11 04 05 14 04 05 15 04 05 17 04 05 19 04 05 20 |
| 2B5b | 2 B 5 b Storage, handling and transport of chemical products | 040415 |
| 2C | 2C Metal Production |  |
|  | 2 C 1 Iron and Steel Production | 040202 040203 040206 040207 |
|  | 2 C 2 Ferroalloys Production | 040302 |
|  | 2 C 3 Aluminum Production | 030310 |
|  | 2 C 5 a Copper Production | 030306 030309 |
|  | 2 C 5 b Lead Production | 030304 030307 |
|  | 2 C 5 c Nickel Production | NO |
|  | 2 C 5 d Zinc Production | 030305 030308 |
|  | 2 C 5 e Other metal production (Please specify the sources included/excluded in the notes column to the right) | NE |
|  | 2 C 5 f Storage, handling and transport of metal products (Please specify the sources included/excluded in the notes column to the right) | NE |
| 2D1 | 2D1 Pulp and Paper | 040602 040603 040604 |
| 2D2 | 2D2 Food and Drink | 040605 040606 040607 040608 |
| 2D3 | 2D3 Wood processing | NE |
| 2E | 2 E Production of POPs | NE |
| 2F | 2 F Consumption of POPs and Heavy Metals (e.g. electricial and scientific equipment) | NE |
| 2G | 2G Other | NE |

Based on the recomendations of the ERT set in the report from stage 3 in-depth review for some categories/pollutants, which are not well developed in the national methodology, EFs from  EMEP/CORINAIR Emission Inventory Guidebook 2009 were applied in order to improve the inventory. Such improvements are made in the following sources in NFR sector 2: 2A1, 2A2, 2A4, 2A6, 2B1, 2B5a, 2C2, 2D1.

|  | Ef Mg/Mg |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SNAP | NMVOC | NH3 | PM2.5 | PM10 | TSP | СО |
| 040302 |  |  |  |  | 0.001 |  |
| 040403 |  | 0.00001 |  |  |  |  |
| 040404 |  |  |  |  | 0.02300 |  |
| 040405 |  | 0.002 |  |  | 0.02000 |  |
| 040406 |  |  | 0.00018 | 0.00024 | 0.00030 |  |
| 040414 |  |  | 0.00018 | 0.00024 | 0.00030 |  |
| 040602 |  |  | 0.00060 | 0.00080 | 0.00100 |  |
| 040611 | 0.000016 |  | 0.00040 | 0.00300 | 0.01400 |  |
| 040612 |  |  | 0.00011 | 0.00020 | 0.00022 |  |
| 040614 |  |  | 0.00005 | 0.00024 | 0.00059 |  |
| 040618 |  |  |  |  |  |  |
| 040619 |  | 0.0009 |  |  | 0.00010 | 0.009 |

Source specific QA/QC and verification

All activities regarding QC as described in QA/QC System have been undertaken in Industrial processes (NFR sector 2).

Source-specific planned improvements

To improve the accuracy of the estimates in Industrial processes (NFR sector 2).

To update the national methodology with the requirements of last version of EMEP/CORINAIR Emission Inventory Guidebook 2009.

To improve transparency, completeness, consistency, including recalculations and time-series and comparability of national emission inventory (see Chapter 10).

Chapter 5: SOLVENT AND OTHER PRODUCT USE (NFR sector 3)

## (to be updated when there have been major changes since the last report)

Methodology

The emissions of air pollutants in Solvent and other product use (NFR sector 3) for 2008 are calculated based on the activity data and emission factors set in the national common methodology for emissions inventory under UNECE/CLRTAR and UNFCCC (see point 1.4). The methodology is harmonized with EMEP/CORINAIR Emission Inventory Guidebook.

The collection of the necessary activity data is responsibility of NSI. NSI uses up-to-date statistical methods and procedures for data collection, summarizing and structuring that are harmonized with EUROSTAT.

The activity data for Solvent and other product use (NFR sector 3) are presented in the reporting table.

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 3A1 | 3 A 1Decorative coating application | 060103 060104 |
| 3A2 | 3 A 2 Industrial coating application | 060101 060105 060106 060108 |
| 3A3 | 3 A 3 Other coating application (Please specify the sources included/excluded in the notes column to the right) | 060102 060107 060109 060110 |
| 3B1 | 3 B 1 Degreasing | 060201 |
| 3B2 | 3 B 2 Dry cleaning | 060202 |
| 3C | 3C CEMICAL PRODUCTS, MANUFACTURE AND PROCESING | 060303 060304 060310 |
| 3 D 1 | 3 D 1 Printing | 060403 |
| 3 D 2 | 3 D 2 Domestic solvent use including fungicides | NE |
| 3 D 3 | 3 D 3 Other product use | 060404 060405 060406 |

Based on the recomendation of the ERT set in the report from stage 3 in-depth review for some categories/pollutants, which are not well developed in the national methodology, EFs from  EMEP/CORINAIR Emission Inventory Guidebook 2009 were applied in order to improve inventory. Such improvements are made in NFR 3C Chemical products, related to NMVOC emissions.

| SNAP | NMVOC |
| --- | --- |
|  | EF Mg/Mg |
| 060303 | 0.12 |
| 060304 | 0.06 |

Source specific QA/QC and verification

All activities regarding QC as described in QA/QC System have been undertaken in Solvent and other product use (NFR sector 3).

Source-specific planned improvements

To improve the accuracy of the estimates in Solvent and other product use (NFR sector 3).

To update the national methodology with the requirements of last version of EMEP/CORINAIR Emission Inventory Guidebook 2009.

To improve transparency, completeness, consistency, including recalculations and time-series and comparability of national emission inventory (see Chapter 10).

Chapter 6: AGRICULTURE (NFR sector 4)

## (to be updated when there have been major changes since the last report)

Methodology

The emissions of air pollutants in Agriculture (NFR sector 4) for 2008 are calculated based on the activity data and emission factors set in the national methodology for emissions inventory under UNECE/CLRTAR and UNFCCC (see point 1.4).

The collection of the necessary activity data is responsibility of NSI. NSI uses up-to-date statistical methods and procedures for data collection, summarizing and structuring that are harmonized with EUROSTAT. The activity data for Agriculture (NFR sector 4) are presented in the reporting table.

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 4B | 4B MANURE MANAGEMENT |  |
| 4B1 | 4B1 Cattle |  |
| 4B1 a | 4B1 a Dairy | 100501 |
| 4B1 b | 4B1 b Non- Dairy | 100502 100514 |
| 4B2 | 4B2 Buffalo | Included in 4B1b |
| 4B3 | 4B3 Sheep | 100505 100511 |
| 4B4 | 4B4 Goats | Included in 4B3 |
| 4B6 | 4B6 Horses | 100506 100512 |
| 4B7 | 4B7 Mules and Asses | Included in 4B6 |
| 4B8 | 4B8 Swine | 100503 |
| 4 B 9 a | 4 B 9 a Laying Hens | 100507 |
| 4 B 9 b | 4 B 9 b Broilers | Included in 4 B 9 d |
| 4 B 9 c | 4 B 9 c Turkeys | Included in 4 B 9 d |
| 4 B 9 d | 4 B 9 d Other Poultry | 100508 100509 |
| 4B13 | 4B13 Other | NE |
| 4 D 1 a | 4 D 1 a Synthetic N-fertilizers | 100101 100102 100104 100105 100201 100202 100204 100205 |
| 4 D 2 a | 4 D 2 a Farm-level agricultural operations including storage, handling and transport of agricultural products | NE |
| 4 D 2 b | 4 D 2 b Off-farm storage, handling and transport of bulk agricultural products | NE |
| 4 D 2 c | 4 D 2 c N-excretion on pasture range and paddock Unspesified (Please specify the sources included/excluded in the notes column to the right) | NE |
| 4F | 4F FIELD BURNING OF AGRICALTURAL WASTES | 10 03 |
| 4G | 4G Other | NE |

Based on the recomendation of the ERT set in the report from stage 3 in-depth review for Manure management EFs are verificated in order to improve inventory.

| SNAP | NH3 |
| --- | --- |
|  | EF Mg/head/y |
| 100501 | 0.0285 |
| 100502 | 0.0143 |
| 100503 | 0.00639 |
| 100505 | 0.00134 |
| 100506 | 0.008 |
| 100507 | 0.00037 |
| 100508 | 0.00028 |
| 100509 | 0.00092 |

Source specific QA/QC and verification

All activities regarding QC as described in QA/QC System have been undertaken in Agriculture (NFR sector 4).

Source-specific planned improvements

To improve the accuracy of the estimates in Agriculture (NFR sector 4).

To update the national methodology with the requirements of last version of EMEP/CORINAIR Emission Inventory Guidebook 2009.

To improve transparency, completeness, consistency, including recalculations and time-series and comparability of national emission inventory (see Chapter 10).

Chapter 7: LAND USE AND LAND-USE CHANGE (NFR sector 5)

## (to be updated when there have been major changes since the last report)

Chapter 8: WASTE (NFR sector 6)

## (to be updated when there have been major changes since the last report)

The emissions of air pollutants in Waste (NFR sector 6) for 2008 are calculated based on the activity data and emission factors are set in the national common methodology for emissions inventory under UNECE/CLRTAR and UNFCCC (see point 1.4). The methodology is harmonized with EMEP/CORINAIR Emission Inventory Guidebook.

The collection of the necessary activity data is responsibility of NSI. NSI uses up-to-date statistical methods and procedures for data collection, summarizing and structuring that are harmonized with EUROSTAT.

The activity data for Waste (NFR sector 6) are presented in the reporting table.

| NFR09 code |  | Sub-source description SNAP 97 items |
| --- | --- | --- |
| 6A | 6A Solid Waste Disposal on Land | 091004 |
| 6B | 6B Waste-Water Handling | 091001 091002 |
| 6C | 6 C a Clinical Waste Incineration (d) | 090207 |
|  | 6 C b Industrial Waste Incineration (d) | 090202 |
|  | 6 C c Municipal Waste Incineration (d) | NO |
|  | 6 C d Cremation | NE |
|  | 6 C e Small Scale Waste Burning | NE |
| 6D | 6D Other Waste | 091003 |

The decrease of NH3 emissions from NFR 6 A Solid waste disposal on land with almost 50% is due to the implementation of the requirements of the National program for implementation of Directive  1999/31/ЕС in 2008 inventory.

Based on the recomendation of the ERT set in the report from stage 3 in-depth review for some categories/pollutants, which are not well developed in the national methodology, EFs from  the EMEP/CORINAIR Emission Inventory Guidebook 2009 were applied in order to improve the inventory. Improvements are made in Waste (NFR sector 6).

The next table presents EF used for assessment of emissions in NFR 6A and NFR 6C.

| Component | 6A Solid Waste Disposal on Land | 6Ca Clinical Waste Incineration | 6Cb Industrial Waste Incineration | Dimension |
| --- | --- | --- | --- | --- |
|  | EF | EF | EF | EF |
| SOx | - | 0.00007 | 0.00007 | Mg/Mg |
| NOx | - | 0.0025 | 0.0025 | Mg/Mg |
| NMVOCs | 0.00004 | 0.0074 | 0.0074 | Mg/Mg |
| NH3 | 0.0032 | - | - | Mg/Mg |
| CO | - | 0.000125 | 0.000125 | Mg/Mg |
| TSP | - | 0.5 | 0.01 | kg/Mg |
| PM10 | - | 0 | 0.007 | kg/Mg |
| PM2.5 | - | 0 | 0.004 | kg/Mg |
| Hg | - | 3 | 3 | g/Mg |
| Cd | - | 3 | 3 | g/Mg |
| Pb | - | 35 | 35 | g/Mg |
| PAH | - | 0.02 | 0.02 | g/Mg |
| PCBs | - | 0.5 | 0.5 | mg/Mg |
| DIOX | - | 150 | 30 | µg/Mg |
| HCB | - | 0.1 | 0.002 | g/Mg |

The next table presents calculation of emissions in NFR 6B Waste-Water Handling.

| NFR 6B | EF | Activity data | Emission |
| --- | --- | --- | --- |
| Component | kg/m3 | м3 | Gg |
| NH3 | 0.0006 | 304857771.4 | 0.183 |

The next table presents calculation of emissions in NFR 6D Other Waste - SNAP 091003 Sludge spreading.

| NFR 6D | EF | Activity data | Content of NH3 | Emission |
| --- | --- | --- | --- | --- |
| SNAP 091003 | kg/Mg | Mg | Mg | Gg |
| NH3 | 50 | 316439.7472 | 31.64397472 | 0.0016 |

Source specific QA/QC and verification

All activities regarding QC as described in QA/QC System have been undertaken in Waste (NFR sector 6).

Source-specific planned improvements

To improve the accuracy of the estimates in.

To update the national methodology with the requirements of last version of EMEP/CORINAIR Emission Inventory Guidebook 2009.

To improve transparency, completeness, consistency, including recalculations and time-series and comparability of national emission inventory (see Chapter 10).

Chapter 9: Other and Natural emissions

## (to be updated when there have been major changes since the last report)

Chapter 10: Recalculations and Improvements (to be updated each year)

## 10.1 Recalculations

- Bulgaria has not submitted Recalculations of previously submitted estimates of emissions.

- In 2009 the MOEW approved two projects, which have as its objectives to improve the Party’s current estimation methodologies, the technical competence of the staff within the national system and the management and archiving of data. Both projects are already in progress:

- Project 1 – “Recalculations of the previously submitted estimates of emissions under UNFCCC and the European Monitoring and Evaluation Programme/Convention on Long-range Transboundary Air Pollution (EMEP/CLRTAP) according to the new Common Methodology from the base year up to the year in which recalculations are made and cover all inventory data”.  This project started on the 1st of December 2009 and will be completed by the 1st of February 2011. The result of this project will be incorporated into the 2011 submission.

- Project 2 – “Development of software tool and automatic preparation of national inventories under UNFCCC and UNECE/CLRTAP”. This project is to commence on 28 December 2009 and will be completed by 28 December 2010. The result of this project will be incorporated into the 2011 submission.

- Bulgaria will use the projects to improve the capacity of the national system in planning, preparation and managing its annual submission, to improve the technical competence of the staff, and to fulfil the requirements for Recalculations of the previously submitted estimates of emissions.

## 10.2 Planned improvements

- The following activities are already done to support the improvement of inventory submissions:

- A National QA/QC Plan for National emissions inventories was approved by the Ministry of Environment and Water in year 2009.

- To raise the technical competence of staff involved in the inventory development process, a training programme for Bulgarian inventory experts was updated within the Twinning project with the Federal Environment Agency of Austria. The program will cover all inventory sectors in a series of workshops planned for the period December 2009 to May 2010 .

- In order to strengthen the institutional arrangements and to fulfil the required general and specific functions a new agreement between MOEW and the National Statistic Institute was signed. There is also a new agreement between MOEW and the Ministry of Agriculture and Food (MAF) and its institutions. These agreements ensure the support from these organisations regarding the choice of the activity data and EFs and methods, in the compilation of emission estimates and QA/QC of these estimates.

- In 2009 the MOEW approved 2 projects, which objectives are to improve the Party’s current estimation methodologies, the technical competence of the staff within the national system, and the management and archiving of data. Both projects, mentioned in a p.10.1 are already in progress. The results from these projects will be incorporated into the 2011 submission.

- Bulgaria will use the projects to improve the capacity of the national system in planning, preparation and managing its annual submission, to improve the technical competence of the staff, and to fulfil the requirements for Recalculations of the previously submitted estimates of emissions.

Chapter 11: Projections

According to the art.36 of the Guidelines for reporting emission data under the CLRTAP, the

Parties shall report their latest available projections at least every five years, and provide any updated projections annually by 15th of February for the years 2010, 2015 etc. Bulgaria did not make any updated projections for this reporting round. The projections will be prepared and will be submitted not later than 2012 when reporting the inventory for year 2010.