NOTES REGARDING SPANISH RIVERS AND LAKES QUALITY DATA AND BIOLOGICAL DATA FOR WISE SoE 2010 REPORTING

CONTINENTAL SURFACE WATERS (RIVERS AND LAKES)

The templates for continental surface waters (rivers and lakes) have been fulfilled only with the data of the sampling sites belonging to the subprogramme number 1 of the surveillance monitoring of the Water Framework Directive (Surv-01), and not with the whole surveillance monitoring network. Surv-01 has been specifically designed for the assessment of the evolution of the general status of water bodies and the evaluation of long-term trends due to anthropogenic activities. Surv-01 has been statistically designed by including sampling sites located in water bodies showing the whole range of status, from high to poor. This design guarantees an unbiased view of the real situation of the Spanish water bodies when a statistical treatment of data is performed.

Due to the continuous refinement and change of the water monitoring networks, the sampling sites of this 2010 WISE-SoE report have changed from the former 2009 WISE-SoE report. This iterative process will take place continuously but the intensity of changes will decrease with time.

In 2010 WISE-SoE report, information on hazardous substances has been delivered (sheet “HazSubstRivers_Agg”) for rivers. The information provided belongs to two different data sets. The first one is the information on the substances gathered in Annex I of the Directive 2008/105/CE on environmental quality standards in the field of water policy. The second data set is built with information on the Spanish river basin specific pollutants gathered in Royal Decree 995/2000. No information on hazardous substances in lakes is provided because there are no sampling stations for these substances belonging to Surv-01 subprogramme, only operational stations are available.

This is the first year that Spain can provide sampling sites codes instead of sampling subsites in the column “NationalStationID” of the sheets “Stations” and “BasicQuality” of both templates for continental surface waters (rivers and lakes). This has been possible thanks to the intense work performed to improve the sampling networks.

DATA SETS FOR LAKES AND RIVERS

Spain sends 2 templates for continental surface water in the 2010 WISE-SoE reporting:

- Template for Rivers (Rivers.xls)

- Template for Lakes (Lakes.xls)

Also includes reservoirs, natural wetlands and lagunas.

Regarding the Validation Rules for the sheet “BasicQuality” in rivers and lakes, some considerations should be made:

- About the Chemical Checks:

    - We strongly recommend considering them with disaggregated data and not with aggregated ones (especially rules 3, 4 and 5). Although in the validation rules of 2010 WISE-SoE report a percentage of toleration has been included (ranging from 1% in rule 2 to 5% in the rest) there might be still some remaining problems.

    - It may occur that some nutrients are not determined with the same frequency in one site, i.e., we could have four determinations per year of nitrate and only two of nitrite, so the value of Total Oxidised Nitrogen (Nitrate+Nitrite) will only be calculated in two occasions per year. Then, the mean value of Total Oxidised Nitrogen will never pass the chemical check (it will never be equal to the mean value of nitrate plus the mean value of nitrite).

    - Another suggestion for future reports is that only information for measured nutrients is demanded and not for calculated ones.

    - About Chemical rule number 2 (total phosphorus > = Ortophosphate, 1% toleration): It may not be passed when any determinand is under the limit of quantification. Moreover, according to our experience, it is feasible that both parameters are in the same concentration level, so we propose to change the rule as Ortophosphate ≤ Total Phosphorus.

- About the Outliers: The raw data have been checked and they are correct.

- In the column “SampleDepth” of the table “NutrientsLakes_Agg", the QA test shows some errors in “Data types” (point 4). “SampleDepth” is meant to be in numerical format but sometimes is provided in text format to specify that it is an integrated sample of the euphotic zone (2.5 times Secchi’s depth). If we provide the numerical data regarding the final depth (2.5 times Secchi’s depth), then it might appear to be a discrete sample instead of an integrated one.

- The Q test seems to have a problem regarding the parameter “Hydrogen sulphide”. Although it is included in the table 4.3.3. of the “data dictionary” for rivers (page 48) there is not a suitable unit of measurement in the table 4.3.4. (page 49), thus the validation shows inconsistencies in the provided data set. For that reason, Spain does not provide data for this parameter.

BIOLOGICAL DATA

Spain sends one template for rivers and lakes in the 2010 WISE-SoE reporting  (BiologyRiversLakes.xls).

Spain has provided the following data sets of the Surv-01 subprogramme:

| Water category | BQE | Provided metric | Name of the field in the template | Units | Observations |
| --- | --- | --- | --- | --- | --- |
| Rivers | Benthic fauna | IBMWP (Iberian Biological Monitoring Working Party) | InvertebrateEQR_E | adimensional | - |
|  | Phytobenthos | IPS index (Indice de Pollusensibilité Spécifique) | PhytobenthosEQR_E | adimensional | - |
| Lakes/ reservoirs/ lagunas | Phytoplankton | Total biovolume of the sample | PhytoplanktonEQR_E | mm3/l | - |
|  |  | Concentration of chlorophyll-a | Chlorophyll_a | µg/l | - |
|  |  | Percentage of cyanobacteria | CyanobacteriaProportion | Proportion | Presence of cyanobacteria (excluding all the Chroococcales but genus Microcystis and Woronichinia). |
|  |  | Index of algal groups: Iga (Catalan et al, 2003.) | PhytoplanktonEQR_G | adimensional | - |
|  | Macrophytes | Number of species of macrophytes | MacrophyteEQR_G | adimensional | - |

The provided biological data are referred as face values and not as EQR’s. Currently, Spain is revising the official boundary setting for all the BQE’s, thus in a short period of time this boundaries will significantly change.