- Explanation of why UK cannot report ecological status class boundary values against Article 5 reporting typologies as required by the River Basin Management Plan reporting schema

- Surface water ecological classification

The UK (England, Wales, Scotland and Northern Ireland) is unable to report many of its classification status boundary values and supporting element standards against the reporting typologies required by the Article 13 River Basin Management Plan schema.

While developing biological element classification tools and associated supporting element environmental standards more ecologically relevant typologies were developed in contrast to the broad reporting typologies generated for the 2004 Article 5 reports.

In addition, site specific reference conditions and ecological boundaries have been developed.  It is also possible to have sites within waterbodies that fall into different ecologically relevant typologies and we have used these to produce the most relevant classification results for the waterbody. Such site specific boundaries cannot be reported against a single reporting typology per waterbody.

All the UK typologies, classification boundaries and methods have been set out in national legislation (currently draft legislation for Northern Ireland) and UK method statements. The reader is referred to these (referenced below) to gain a detailed understanding of how classification boundaries relate to typologies for categories of UK surface water.

- Intercalibration

As a consequence of the classification threshold issue highlighted above, the UK is also unable to complete the relevant links for water bodies to the intercalibration decisions.

While developing biological element classification tools the ongoing intercalibration exercise was used to inform the boundaries developed for the biological elements.

For those elements or metrics where there was an intercalibration decision these have been transferred directly into the UK legislation given below.

For those elements/metrics where there was partial intercalibration, the values for the intercalibrated metric have been used to inform the decisions on boundaries for associated metrics that were not intercalibrated.

For those elements/metrics where there was no intercalibration possible the intercalibration guidance boundary setting protocol has been used to determine boundaries and facilitate intercalibration in the future.

To gain a detailed understanding of how biological element boundaries follow the intercalibration decisions. The reader is referred to the documents referenced in Annex 1 below for the relevant schema elements.

3. Background Documents

These documents (referred to as ‘The Directions’ elsewhere) have also been uploaded to Reportnet:

England and Wales River Basin District (Surface Water Typology, Environmental Standards, Conditions Limits and Groundwater Threshold Values) Directions 2009

http://www.defra.gov.uk/environment/quality/water/wfd/documents/river-basin-typology-standards.pdf

Scotland River Basin District (Surface Water Typology, Environmental Standards, Conditions Limits and Groundwater Threshold Values) Directions 2009

http://www.scotland.gov.uk/Resource/Doc/298071/0092869.pdf

The Solway Tweed (Scotland) River Basin District (Surface Water Typology, Environmental Standards, Conditions Limits and Groundwater Threshold Values) Directions 2009

http://www.scotland.gov.uk/Publications/2009/12/14130958/0

The draft legislation in Northern Ireland (NI) – “The Water Framework Directive (Priority Substances and Classification) Regulations (Northern Ireland) 2010 will be issued for public consultation in spring 2010. The class boundaries set out in the draft legislation follow the UK approach and have been applied to the classification of water bodies in Northern Ireland.

Classification tool method statements can be found at:

http://www.wfduk.org/bio_assessment/

Joint paper by the Environment Agency (England and Wales) and the Scottish Environment Protection Agency (SEPA) for Scotland and the Department of Environment Northern Ireland (DoENI)

19 March 2010

Annex 1.

This Annex sets out the UK information for each of the schema elements which we are unable to report because of the mismatch between the reporting typologies required by WISE and the classification typologies that we have used to establish classification boundaries.

Typology codes

It has not been possible to define a single typology for differentiating water bodies of the same water category in a way that allows reliable type-specific reference conditions to be established for all quality elements. This is because reference conditions for different quality elements in the river basin district can vary according to:

- different natural characteristics; and

- different values for the same natural characteristics.

The establishment of reliable reference conditions is necessary to enable adequate confidence and precision to be achieved in classification. Where necessary to ensure such conditions can be established, we have identified quality element-specific typology systems. Individual water bodies may contain one or more quality-element specific types. In making classification decisions, the appropriate reference conditions and class boundary standards are applied depending on the location (in relation to these types) of the monitoring station within the water body.

Details of the quality-element types can be found in The Directions

All Typology Code

For a number of quality elements (including most specific pollutants), the environmental standards are the same across all types within a water category. Those standards that are applicable across all types are indicated in The Directions

River, Lake, Transitional Water, Coastal Water Intercalibration Type & National Translation

Rivers

The river intercalibration types R-N1; R-N3; R-N4; R-N5; R-C1; R-C2; R-C3; R-C4; R-C5; R-C6 are present in the UK.

The intercalibrated boundaries for benthic invertebrates and phytobenthos for all these types are the same and have been incorporated into the UK classification systems for these quality elements. The applicable boundaries are specified in The Directions and they are applied to all rivers in the Districts.

Lakes

The lake intercalibration types LA1/2; L-CB1; L-CB2, LN1, LN2a, LN3a, LN8a, LN2b, LN5, LN6a are present in the UK.

The intercalibrated boundaries for lake phytoplankton (chlorophyll a concentrations) for all these types have been incorporated into the UK phytoplankton classification system. The applicable boundaries are specified in The Directions

For non-intercalibrated lake types, phytoplankton reference conditions have been derived using the same method as used for intercalibrated lake types and the classification system boundaries have been determined using the same principles used for the most similar intercalibrated types.

The intercalibrated boundaries for lake macrophytes are the same for the intercalibration types LA1/2; L-CB1; L-CB2, 101, 102, 201, 202, 301, 302. The boundaries are specified in The Directions and they are applied to all lakes in the Districts.

Transitional & coastal waters

The transitional and coastal water intercalibration types NEA1/26a; NEA1/26b; NEA7; NEA11 are present in the UK.

The intercalibrated boundaries for benthic invertebrates are the same in types NEA1/26a; NEA1/26b; NEA7. The boundaries are specified in The Directions and are applicable to all soft sediments in UK coastal waters.

For intercalibration type NEA11, there are no intercalibrated boundaries for benthic invertebrates.

The intercalibrated boundaries for phytoplankton have been incorporated into the UK phytoplankton classification system for coastal waters. Details can be found in the UK method statement:

PHYTOPLANKTON MULTI-METRIC TOOL KIT

http://www.wfduk.org/bio_assessment/LibraryPublicDocs/phytoplankton%20v2.

The classification system applies to all UK coastal waters.

For intercalibration types NEA7 and NEA11, there are no intercalibrated boundaries for phytoplankton.

The intercalibrated boundaries for macroalgae have been incorporated into the UK macroalgae classification system for transitional and coastal waters. The boundaries are specified in The Directions

For intercalibration types NEA7 and NEA11, there are no intercalibrated boundaries for macroalgae.

The UK method for angiosperms is not yet implementable. It will incorporate the intercalibrated boundaries for angiosperms.

Units

The parameters or metric units for each hydromorphological and physicochemical quality element used in the classification scheme are specified in The Directions

Statistical expression

The statistical expression for the parameters or metric units for each hydromorphological and physicochemical quality element used in the classification scheme is specified in The Directions

Reference condition

The reference values, where established, for each physicochemical quality element and each hydromorphological quality element used in the classification scheme are equivalent to the high/good boundary values. These can be found in The Directions

For biological reference values, see below.

High/Good boundary; Good/moderate boundary; moderate poor boundary; poor/bad boundary

The relevant classification boundary values for each quality element are set out in The Directions

High Good Calibration compliant

The high/good boundaries for intercalibrated biological quality elements reflect the results of the intercalibration exercise

Good Moderate calibration compliant

The good/moderate boundaries for intercalibrated biological quality elements reflect the results of the intercalibration exercise

Reference conditions (biological quality elements) and National methods in use (biological quality elements)

Reference conditions and monitoring methods for each biological quality element are published in the following UK biological method statements or in the Directions:

UK LAKE ASSESSMENT METHODS

MACROPHYTES AND PHYTOBENTHOS,

PHYTOBENTHOS - DIATOM ASSESSMENT OF LAKE ECOLOGICAL QUALITY (DARLEQ1) http://www.wfduk.org/bio_assessment/bio_assessment/lakes_phytobenthos_darleq

MACROPHYTES (LAKE LEAFPACS)

http://www.wfduk.org/bio_assessment/bio_assessment/lakes%20leafpacs%20method%20statement

For Northern Ireland only: Macrophytes and Phytobenthos, Macrophytes (FREE INDEX)

PHYTOPLANKTON

CHLOROPHYLL A AND PERCENTAGE NUISANCE CYANOBACTERIA

http://www.wfduk.org/bio_assessment/bio_assessment/lake_phytoplankton

BENTHIC INVERTEBRATE FAUNA

CHIRONOMID PUPAL EXUVIAE TECHNIQUE (CPET)

http://www.wfduk.org/bio_assessment/bio_assessment/lakes_cpet

LAKE ACIDIFICATION MACROINVERTEBRATE METRIC (LAMM)

http://www.wfduk.org/bio_assessment/bio_assessment/lakes_invertebrates

UK RIVER ASSESSMENT METHODS

MACROPHYTES AND PHYTOBENTHOS

MACROPHYTES (RIVER LEAFPACS)

http://www.wfduk.org/bio_assessment/bio_assessment/river%20macrophytes%20nw

PHYTOBENTHOS - DIATOM ASSESSMENT for RIVER ECOLOGICAL STATUS (DARES1)

http://www.wfduk.org/bio_assessment/bio_assessment/river%20phytobenthos%20method%20statement

BENTHIC INVERTEBRATE FAUNA

RIVER INVERTEBRATE CLASSIFICATION TOOL (RICT)

http://www.wfduk.org/bio_assessment/bio_assessment/rivers_invertebrates

For Scotland only: Scottish Acid Water Indicator Community (SAWIC)

http://www.wfduk.org/bio_assessment/bio_assessment/river_acidification_nov_09

FISH FAUNA

For England and Wales: http://www.wfduk.org/bio_assessment/bio_assessment/rivers_fish

For Scotland:The Directions

UK TRANSITIONAL WATER ASSESSMENT METHODS

MACROALGAE

MACROALGAL BLOOM ASSESSMENT (OPPORTUNISTIC MACROALGAE)

http://www.wfduk.org/bio_assessment/bio_assessment/Opportunistic

FUCOID EXTENT

http://www.wfduk.org/bio_assessment/bio_assessment/Fucoid%20Extent%20ms

FISH FAUNA

TRANSITIONAL FISH CLASSIFICATION INDEX (TFCI)

http://www.wfduk.org/bio_assessment/bio_assessment/fish%20method%20statement

UK COASTAL WATER ASSESSMENT METHODS

BENTHIC INVERTEBRATE FAUNA

DOG WHELKS (Nucella lapillus) - IMPOSEX ASSESSMENT

http://www.wfduk.org/bio_assessment/bio_assessment/trac_imposex

INVERTEBRATES IN SOFT SEDIMENTS (INFAUNAL QUALITY INDEX (IQI))

http://www.wfduk.org/bio_assessment/bio_assessment/trac_iqi

MACROALGAE

MACROALGAE - ROCKY SHORE REDUCED SPECIES LIST

http://www.wfduk.org/bio_assessment/bio_assessment/macro_rockyshore

MACROALGAL BLOOM ASSESSMENT (OPPORTUNISTIC MACROALGAE)

http://www.wfduk.org/bio_assessment/bio_assessment/Opportunistic

PHYTOPLANKTON

PHYTOPLANKTON MULTI-METRIC TOOL KIT

http://www.wfduk.org/bio_assessment/LibraryPublicDocs/phytoplankton%20v2

Implementable at present

The classification system for all the quality elements listed in The Directions

is implementable at present.