Part 1

Questionnaire on the implementation of Directive 2003/87/EC

1.	Details of institutions submitting the report

1.	Name of contact person: George Nicholas Nelson

2.	Official title of contact person: Senior Adviser

3.	Name and department of organisation: Climate and Pollution Agency, 			Department of Climate and Industry

4.	Address: P.O. Box 8100 Dep, N-0032 Oslo, Norway

5.	International telephone number: +47 22 57 34 40 (direct) / +47 22 57 34 00 		(reception)

6.	International telefax number: +47 22 67 67 06

7.	E-mail: george-nicholas.nelson@klif.no

2.	Competent authorities

2.1.	Please state the name and the abbreviation of the competent authorities which are involved in the implementation of the emissions trading scheme in your country.

In answering this question, use the table below. Add further rows if necessary.

| Name | Abbreviation | Contact details |
| --- | --- | --- |
| Norwegian Pollution Control Authority | Klif | See paragraph no. 1 |
| Ministry of Environment | MD | P.O. Box 8013 Dep, N-0030 Oslo |
| Ministry of Finance | FIN | P.O. Box 8008 Dep, N-0030 Oslo |

2.2.	Please indicate which competent authority is responsible for each of the tasks listed in the table below using their abbreviations.

| Please indicate the abbreviation of the competent authority which is in charge of the following tasks: |  |
| --- | --- |
| Issuance of permits | Klif |
| Allocation of allowances | Klif |
| Issuance of allowances | Klif |
| Validation of monitoring methodology | Klif |
| Receiving and supervising verified emission reports | n.a. (Klif verifies the reports) |
| Accreditation of verifiers | n.a. (NO does not have third party verification) |
| Registry | Klif |
| Compliance and enforcement | Klif |
| Issuance of ERU as a host country | n.a. |
| Approval of the use of CERs & ERUs for compliance | Klif |
| Administration of new entrants reserve | Klif |
| Information to the public | Klif, (MD and FIN) |
| Auctioning | FIN |
| Administration of opt-ins | Klif |
| Administration of pooling | n.a. |
| Other (please specify): _____________________ |  |

3.	Coverage of activities and installations

3.1.	How many of the combustion installations have a rated thermal input that exceeds 20 MW but is below 50 MW on 31 December of the reporting year? In total, how many CO2 equivalents were emitted by these installations in the reporting period?

In answering this question, use the table below.

|  | Number | Share in total number of installations or emissions |
| --- | --- | --- |
| Number of installations with a rated thermal input that exceeds 20 MW but is below 50 MW | 29 | 25,66 % |
| CO2 equivalents emitted by those installations | 282 667 tonnes | 1,47 % |

3.2.	What changes occurred during the reporting period in comparison with the national allocation plan table (NAP Table) as entered into the Community Independent Transaction Log on 1 January of the reporting year (new entrants, closures, installations falling below the capacity thresholds)?

In answering this question, use Table 1 of Part 2 of this Annex.

No changes occured during the reporting period in comparison with the NAP table.

3.3.	Did the competent authority receive any application(s) during the reporting period from operators who wish to form a pool pursuant to Article 28 of Directive 2003/87/EC (ET Directive)? If yes, to which activity listed in Annex I to Directive 2003/87/EC (hereinafter — “Annex I activity”) did the application refer to and was the pool formed?

No applications were received.

In answering this question, use the table below.

| Main Annex I activity (a) |  | Number of applications received | Number of pools formed |
| --- | --- | --- | --- |
|  | Energy activities | 0 |  |
| E1 | Combustion installations with a rated thermal input exceeding 20 MW (except hazardous or municipal waste installations) |  |  |
| E2 | Mineral oil refineries |  |  |
| E3 | Coke ovens |  |  |
|  | Production and processing of ferrous metals | 0 |  |
| F1 | Metal ore (including sulphide ore) roasting or sintering installations |  |  |
| F2 | Installations for the production of pig iron or steel (primary or secondary fusion) including continuous casting, with a capacity exceeding 2.5 tonnes per hour |  |  |
|  | Mineral industry | 0 |  |
| M1 | Installations for the production of cement clinker in rotary kilns with a production capacity exceeding 500 tonnes per day or lime in rotary kilns with a production capacity exceeding 50 tonnes per day or in other furnaces with a production capacity exceeding 50 tonnes per day |  |  |
| M2 | Installations for the manufacture of glass including glass fibre with a melting capacity exceeding 20 tonnes per day |  |  |
| M3 | Installations for the manufacture of ceramic products by firing, in particular roofing tiles, bricks, refractory bricks, tiles, stoneware or porcelain, with a production capacity exceeding 75 tonnes per day, and/or with a kiln capacity exceeding 4 m³ and with a setting density per kiln exceeding 300 kg/m³ |  |  |
|  | Other activities | 0 |  |
|  | Industrial plants for the production of |  |  |
| O1 | (a) pulp from timber or other fibrous materials |  |  |
| O2 | (b) paper and board with a production capacity exceeding 20 tonnes per day |  |  |
| (a) If an installation carries out more than one activity, please only count the installation once under its main Annex I activity. |  |  |  |

3.4.	Is there any other relevant information concerning the coverage of installations and activities in your country? If so, please specify.

Norway has included the emissions of nitrous oxide (N2O) associated with the production of nitric acid (HNO3) as an additional gas and activity in the emission trading scheme. Two installations operated by Yara are effected by the mandatory inclusion (see details in the box below). The unilateral opt-in inclusion took effect from 1 July 2008 and will be included at least for the rest of the current five-year period (2008 – 2012).

Rules for monitoring and reporting for the nitric acid producing installations are 	implemented in the Regulations relating to greenhouse gas emission allowance 	trading. These rules are in line with the provisions in Commission Decision of l7 	December 2008 amending Decision 2007/1589/EC as regards the inclusion of 	monitoring and reporting guidelines for emissions of nitrous oxide.

The allocation methodology is laid down in section 4-5 in the Regulations relating 	to greenhouse gas emission allowance trading. The installations are allocated 	allowances each year in the period 2009 – 2012 corresponding to 50 % of 	their 	average emissions of N2O per year in the base period 1998 – 2001. For the year 2008 	the installations are allocated 25 % of the same average emissions.

Box 1: Emissions in 2009 and allocation for opt-in installations

| Installation name | Allocation per year 2009 – 2012 (2008) | Allocation total | Emissions in 2009, CO2 eqv. (CO2 + N2O), tonnes |
| --- | --- | --- | --- |
| Yara Norge AS Glomfjord | 285 355 (142 678) | 1 284 098 | 129 332 |
| Yara Norge AS Porsgrunn | 628 976 (332 229) | 2 848 133 | 359 000 |

4. The issue of permits for installations

4.1.	What measures have been taken to ensure that operators comply with the requirements of their greenhouse gas emissions permits?

Note: Fines or penalties which might be imposed in case of infringements must not be reported here but under section 11.

In answering this question, use the table below. Add further rows if necessary.

The answers given in the table below only reflects if the measure has a legal basis in Norwegian legislation, not necessarily if the measure has been imposed in practise in 2009.

| Which of the following measures are applied in your country (add explanatory text if necessary)? |  |
| --- | --- |
| The account will be blocked in case of irregularities | Yes |
| Selling will be prohibited in case of irregularities | Yes |
| Withdrawal of permit; suspension of the installation | Yes |
| Spot or routine checks or inspections by the administration | Yes |
| Conservative emission estimates in case of missing emission reports | Yes |
| Verification bodies check compliance with the conditions of the permit | Yes 1 |
| Regular meetings with industry & associations to discuss relevant issues | Yes |
| Provision of specific reporting formats and guidance | Yes |
| Naming and shaming of non-compliant operators | Yes |
| Other (please specify): ____________________ |  |

1. On-site inspections performed by one or more inspectors from Department of Control, Water and International Affairs in Klif

4.2.	Where more than one competent authority is involved, how does national legislation ensure that the conditions of and the procedures for the issuance of permits are fully coordinated? How does this coordination work in practice?

In answering this question, use the table below. Add further rows if necessary.

| Which of the following statements applies to your country (add explanatory text if necessary)? |  |
| --- | --- |
| More than one competent authority | No, Klif is the only competent authority involved in issuance of permits |
| If yes, please answer the following questions: |  |
| Cooperation explicitly regulated by a law or a regulation | n.a. |
| Commission or working group or coordination with regular meetings established | n.a. |
| Guidance note for implementation of the national emissions trading law | n.a. |
| Interpretation group to clarify ambiguous issues | n.a. |
| Coordination of administrative acts by one central authority | n.a. |
| Training courses to ensure consistent implementation | n.a. |
| Other (please specify): ____________________ |  |

4.3. 	In cases where installations carry out activities listed in Annex I to Council Directive 96/61/EC (IPPC Directive) what measures have been taken to ensure that conditions and procedure for the issue of a greenhouse gas emissions permit are coordinated with those for the permit provided for in that Directive? Have the requirements laid down in Articles 5, 6 and 7 of Directive 2003/87/EC been integrated into the procedures provided for in Directive 96/61/EC? If so, how was this integration performed?

In answering this question, use the table below. Add further rows if necessary.

| Which of the following statements applies to your country (add explanatory text if necessary)? |  |
| --- | --- |
| Requirements laid down in Articles 5-7 of Directive 2003/87/EC have been transposed by national legislation | Yes |
| Law which transposes the IPPC Directive does not include emission or concentration limits for CO2 | Correct. The Pollution Control Act transposes the IPPC Directive. Section 11 of the Act states that installations covered by the emission trading scheme shall be granted a greenhouse gas permit provided that the installation can document that it is able to monitor and report it’s emissions in a satisfactory way. No limits for emissions of greenhouse gases covered by the Greenhouse Gas Emission Allowance Trading Act shall be set. |
| Integrated permitting procedure under the IPPC Directive and the ET Directive | No |
| Separate permits for IPPC and ET Directive | Yes |
| Granting of an IPPC permit requires a valid emissions trading scheme (ETS) permit | No |
| Granting of an ETS permit requires a valid IPPC permit | No |
| IPPC regulators will check whether ETS permit is necessary and inform ETS regulators | There are no formal procedures for this, but Klif is the competent authority for both ETS permits and IPPC permits, and many regulators are involved in both permitting processes. |
| Other (please specify): ____________________ |  |

4.4.	What are the legislative provisions, procedures and practice concerning updating of permit conditions by the competent authority pursuant to Article 7 of Directive 2003/87/EC?

In answering this question, use the table below. Add further rows if necessary.

| Please refer to the legal provision which transposes Article 7 of Directive 2003/87/EC | Section 2-4 of the Regulations relating to greenhouse gas emission trading |
| --- | --- |
| Which of the following provisions, procedures and practices apply to your country (add explanatory text if necessary)? |  |
| Authorisation for changes in the installation type or operating mode required | Yes |
| Authorisation for changes in the monitoring methodology required | Yes |
| Changes have to be notified in advance | Yes |
| Closures have to be notified immediately | In good time before the actual closure |
| Penalty in case of non-compliance with request to update monitoring methodology | Klif updates the monitoring plans based on information submitted by the installation. Penalty may be issued if the installation does not comply with the updated / changed monitoring methodology. |
| Change of the operator requires an update of permit | Yes |
| Less significant changes are just recorded | Yes |
| Other (please specify): ________________ |  |

4.5. 	How many permits were updated during the reporting period because of a change in the nature or functioning, or extension, of installations made by operators as specified in Article 7 of Directive 2003/87/EC? Please provide for each category (capacity increase, capacity decrease, change in process type, etc.) how many permits were updated.

In answering this question, use the table below. Add further rows if necessary.

| Please state the number of changes in each category: |  |
| --- | --- |
| Total changes | < 82 |
| Revoked | 0 |
| Surrendered | 0 |
| Transferred | 0 |
| Increase of capacity | 0 |
| Decrease of capacity | 0 |
| Changes to monitoring and reporting details | < 20 |
| Change in name of installation or operator | 27 |
| Non-significant amendment | < 20 |
| Notification of changes without update of permit | < 15 |
| Other (please specify): ________________ |  |

4.6.	Is there any other relevant information concerning the issue of permits for installations in your country? If so, please specify.

5.	Application of the monitoring and reporting guidelines

Preliminary remark:

The guidelines for the monitoring and reporting of greenhouse gas emissions from the activities listed in Annex I to Directive 2003/87/EC were set out in the Annexes to Commission Decision 2004/156/EC of 29 January 2004 establishing guidelines for the monitoring and reporting of greenhouse gas emissions pursuant to Directive 2003/87/EC of the European Parliament and of the Council. The review of these guidelines led to the replacement of Decision 2004/156/EC by Commission Decision of 18 July 2007 establishing guidelines for the monitoring and reporting of greenhouse gas emissions pursuant to Directive 2003/87/EC of the European Parliament and of the Council, which repealed Decision 2004/156/EC from 1 January 2008.

The guidelines contained in the Annexes to Decision 2007/589/EC set out the revised detailed criteria for the monitoring and reporting of greenhouse gas emissions resulting from the activities listed in Annex I of Directive 2003/87/EC. These are specified in relation to those activities, based on the principles for monitoring and reporting set out in Annex IV of that Directive that should apply as of 1 January 2008.

Given that installations in the EEA-EFTA States that are covered by the EU greenhouse gas emission trading allowances scheme participate in that scheme only from 1 January 2008, EEA-EFTA ensured that the installations complied with the guidelines for monitoring and reporting of greenhouse gas emissions as set out in the Annexes to Decision 2007/589/EC.

Therefore, in the following questions and corresponding answers, references to the repealed Decision 2004/156/EC have been construed as references to Decision 2007/589/EC and should be read accordingly, notably the provisions on tiers of approaches specified in section 4.2.2.1.4 of Annex I to Decision 2004/156/EC shall be read as the provisions on tiers of approaches specified in section 5.2 of Annex I to Decision 2007/589EC and the provisions on transferred CO2 set out in section 4.2.2.1.2 of Annex I to Decision 2004/156/EC shall be read as the provisions on the provisions on transferred CO2 set out in section 5.7 of Annex I to Decision 2007/589EC.

5.1.	What legal acts have been adopted in your country in order to implement monitoring and reporting guidelines? Are general derogations from the monitoring and reporting guidelines allowed by the legislation of your country, e.g. for specific fuels or activities? If so, please specify.

Regulations relating to greenhouse gas emission allowance trading fully transpose the monitoring and reporting guidelines set out in the Annexes to Decision 2007/589/EC. All installations were issued an individual monitoring plan as an annex to the permit. The monitoring plan specifies the monitoring requirements for the installation. The information and requirements given in the monitoring plan are consistent with the elements listed in section 4.3 (a) to (n) in the Annex I to Decision 2007/589/EC.

There are no specific derogations from the monitoring and reporting guidelines implemented in the Regulations other than the general exemptions which can be given to due to technically not feasible or will lead to unreasonably high costs (paragraph 4 in section 5.2 in Annex I to Decision 2007/589/EC) and installations with average emissions of less than 25 000 tonnes of CO2 per year (section 16 in Annex I). These exemptions are implemented in section 2-3 in the Regulations relating to greenhouse gas emission allowance trading.

5.2.	Which tiers were used in the monitoring methodologies for the major emitting installations (cf. Commission Decision 2004/156/EC)?

In answering this question, use Table 2 of Part 2 of this Annex. The information required in Table 2 need only be given for the largest installations covered by the ET Directive which contribute cumulatively to 50 % of the total emissions included in the trading scheme. No information needs to be reported for sources within these installations with annual emissions below 25 kt CO2 eq.

The required tier level to different source streams and different variables (activity data, emission factor, oxidation factor, conversion factor) for different Annex 1 activities are laid down in Annexes to Regulations relating to greenhouse gas emission allowance trading. Klif issues each installation an individual monitoring plan based on these general requirements.

Category A installations must as a minimum adhere to tier requirements identical to the tiers specified in Table 1 in section 5.2 of Annex I to Decision 2007/589/EC. As a general rule, all category B and C installation are required to use the highest tier approach to determine all variables for all source streams (not including minor source streams and de-minimis source streams). Only if it is shown to the satisfaction of Klif that the highest tier approach is technically not feasible or will lead to unreasonably high costs, may a next lower tier be used for that variable within a monitoring methodology.

Information on level of tiers for the nine biggest emitters in 2009 is given in Table 2. The emissions from these nine installations contributed to 49,8 percent of the total emissions. In relation to the general tier requirements stated in the paragraphs above, the information given in Table 2 only covers source streams which emitted more than 25 000 tonnes of CO2-eqv and where exemptions from the highest tier have been given.

Please be aware that almost all emission factors for fuels are expressed as tonne CO2 / tonne or tonne CO2 / Sm3, rather that tonne CO2 / TJ, in Regulations relating to greenhouse gas emission allowance trading and in the individual monitoring plans. This value has long been used by Norwegian installations and also in the national greenhouse gas inventory report.

5.3.	If tiers below the minimum tiers specified in Table 1 in section 4.2.2.1.4 of Annex I to Decision 2004/156/EC have been accepted in the monitoring methodology, please indicate for each installation for which this situation occurred the coverage of emissions, the activity, the tier category (activity data, net calorific value, emission factor, oxidation factor or conversion factor) and the monitoring approach/tier agreed in the permit.

In answering this question, use Table 3 of Part 2 of this Annex. The information required in Table 3 needs only to be given for installations not reported under question 5.2. General derogations provided for in the national legislation must be reported under question 5.1.

All installations in every category (A, B and C) must as a general rule adhere to the minimum tier requirements specified in Table 1 in section 5.2 of Annex 1 to Decision 2007/589/EC. Category B and C installation are required to use the highest tier approach. See the second paragraph in the answer to question 5.2 for details.

Based on the reasons stated above, there have been given very few exemptions to the minimum tier requirements specified in Table 1 in section 5.2 of Annex 1 to Decision 2007/589/EC. Unfortunately, details of these few exemptions have not been accumulated in a database. For that reason, we are not able to supply the required information in Table 3.

5.4.	Which installations temporarily applied different tier methods than those agreed with the competent authority?

In answering this question, use Table 4 of Part 2 of this Annex.

No installations temporarily applied tier methods which were not agreed with Klif beforehand.

5.5.	In how many installations was continuous emissions measurement applied? Please indicate the number of installations per Annex I activity and within each activity per subcategory based on reported annual emissions (less than 50 kt, 50-500 kt and over 500 kt).

In answering this question, use Table 5 of Part 2 of this Annex.

5.6.	How much CO2 was transferred from installations? Please indicate the number of tonnes of CO2 transferred pursuant to section 4.2.2.1.2 of Annex I to Decision 2004/156/EC and the number of installations that transferred CO2 for each activity listed in Annex I to Directive 2003/87/EC.

In answering this question, use the table below.

| Main Annex I activity | Number of installations | CO2 transferred (kt CO2) | Use of transferred CO2 |
| --- | --- | --- | --- |
| E1 | 0 | - | - |
| E2 | 0 | - | - |
| E3 | 0 | - | - |
| F1 | 0 | - | - |
| F2 | 0 | - | - |
| M1 | 0 | - | - |
| M2 | 0 | - | - |
| M3 | 0 | - | - |
| O1 | 0 | - | - |
| O2 | 0 | - | - |

5.7.	How much biomass was combusted or employed in processes? Please indicate the quantity of biomass as defined in paragraph 2(d) of Annex I to Decision 2004/156/EC combusted (TJ) or employed (t or m3) for each activity listed in Annex I to Directive 2003/87/EC.

We are unfortunately not able to supply this information. The biomass data were stored in a database without the functionalities and variables needed to answer this question.

In answering this question, use the table below.

| Main Annex I activity | Biomass combusted (TJ) | Biomass employed (t) | Biomass employed (m3) |
| --- | --- | --- | --- |
| E1 | not available | not available | not available |
| E2 | not available | not available | not available |
| E3 | not available | not available | not available |
| F1 | not available | not available | not available |
| F2 | not available | not available | not available |
| M1 | not available | not available | not available |
| M2 | not available | not available | not available |
| M3 | not available | not available | not available |
| O1 | not available | not available | not available |
| O2 | not available | not available | not available |

5.8.	What was the total quantity of waste used as fuel or input material per waste type? What was the total quantity of resulting CO2 emissions per waste type?

In answering this question, use the table below. Add further rows if necessary.

When reporting waste used as fuel or input material, the installations are only required to classify the waste type as either hazardous waste or municipality waste from private households (waste subject to the tax on final waste disposal).

| Waste type | Quantity used/deployed (t) | Quantity used/deployed (m3) | CO2 Emissions (t CO2) |
| --- | --- | --- | --- |
| Hazardous waste | 26 850 |  | 47 690 |
| Municipality waste from private households | 827 816 |  | 625 018 |

5.9.	Please submit sample monitoring and reporting documents from some temporarily 	excluded installations, if applicable.

No installations have been temporarily excluded.

5.10.	What measures have been taken to coordinate reporting requirements with any existing reporting requirements in order to minimise the reporting burden on businesses?

In answering this question, use the table below. Add further rows if necessary.

| Which of the following statements applies to your country (add explanatory text if necessary)? |  |
| --- | --- |
| ETS reporting requirements are coordinated with other reporting requirements | Partly. ETS installations which also are comprised by the IPPC Directive use one Internet reporting portal - www.altinn.no – for both reporting requirements. More than 120 public forms from 20 Norwegian government agencies are currently available through this portal. |
| Coordination with greenhouse gas inventory compilation under UNFCCC and Decision 280/2004/EC | Yes |
| Coordination with EPER | No |
| Coordination with IPPC | No |
| Coordination with NEC | No |
| Coordination with LCP | No |
| Coordination with EMEP | No |
| Coordination with voluntary covenants | No |
| Coordination with other trading schemes (please specify) | No |
| ET data can be used by statistical office | Yes |
| Other (please specify): ____________________ |  |

5.11.	What procedures or measures have been implemented to improve monitoring and reporting by operators?

Klif approves and updates the monitoring plan on the basis of information submitted by the installation. The process of verifying the emission reports every March may lead Klif to impose the installation to submit further documentation on how the requirements in the monitoring plan are met, or Klif may change tier methods directly based on the submitted emission report. Findings from on-site inspections may also result in an update of the monitoring plan.

The on-site inspections also serve as practical ways of providing information and guidance to the installations regarding both monitoring and reporting. Every year Klif holds a reporting seminar open to all installations where Klif informs of any new reporting requirements and changes to the reporting template. Klif has produced a guidance document to the Regulations relating to greenhouse gas emission allowance trading which gives detailed information on how all tier methods are to be understood.

5.12.	Is there any other relevant information concerning the application of the monitoring and reporting guidelines in your country? If so, please specify.

6.	Arrangements for verification

6.1.	Please describe the framework for verification of emissions, in particular the role of the competent authorities and other verifiers and any special requirements for verifiers already accredited in another country. Please submit documents setting out the accreditation criteria for verifiers as well as any verification guidance provided for accredited verifiers and documents setting out the mechanisms for supervision and quality assurance for verifiers, if available.

In answering this question, use the table below. Add further rows if necessary.

The Greenhouse Gas Emission Trading Act (GGET) does not require the annual 	emission reports to be verified by an accredited verifier. As stated earlier, the 	emission reports submitted by the installations are verified by Klif. The on-site 	inspections are performed by one or more inspectors from Department of Control, 	Water and International Affairs in Klif, and the results from the inspections feed into 	the verification process performed annually by Klif each March.

Norway is in the process of establishing a system with verification by accredited verifiers. Legislative drafts are now being prepared, and a final draft will be sent on public consultation by summer/early autumn 2010. The system will be implemented at the earliest in time for the verification of the 2011 emission reports.

| Which of the following statements apply to your country (add explanatory text if necessary)? |  |
| --- | --- |
| Independent verifiers can be accredited according to national criteria (if so, please provide relevant documents or Internet link) | No |
| National guidance for verification developed (if so, please provide relevant documents or Internet link) | No |
| Are national rules and procedures for verification based on EN45011 and EA-6/01 | No |
| Verifiers are required to recommend improvements to installation’s monitoring | Klif can recommend AND impose |
| Competent authority or other agency has a right to check verified emission reports | Not applicable |
| Competent authority or other agency has a right to adjust the verified emission report if deemed unsatisfactory | Yes, but only Klif. |
| Competent authority or other agency supervises verifiers (including spot checks, training, quality assurance and quality control procedures) | Not applicable |
| Competent authority has a right to appoint a verifier to an installation | Yes. Klif has the authority to impose third party verification on individual installations if Klif finds it necessary. |
| Verifiers accredited in another Member State are subject to another accreditation process | Not applicable |
| Knowledge of language and/or national laws/regulations required for verifiers accredited in another Member State | Not applicable |
| Special QA/QC procedures in place at CA for verifiers accredited in another Member State | Not applicable |
| Other (please specify): ____________________ |  |

6.2.	Did any operator provide an emission report for the reporting period not considered satisfactory by 31 March? If so, please provide a list of the installations concerned and the reasons why no positive verification statement was given.

In answering this question, use Table 6 of Part 2 of this Annex. Cases where operators did not provide any emission report must be reported under question 6.3.

The deadline for submission of annual emission reports by the installations is 1 March. Klif processes the reports and have decisions on verified emissions for the preceding year ready by 31 March. All emission reports for 2009 were considered satisfactory by 31 March 2009, but many installations were imposed to provide Klif with more documentation and/or more detailed information to show that their tier requirements were met. The deadline for submission of this documentation to Klif differs in each case, and is dependent on the scope of information/documentation which must be provided

6.3.	For how many installations were no emission reports for the reporting period provided by 31 March? Please indicate the number of installations, allocated allowances and allowances blocked in the operators’ holding accounts per Annex I activity and within each activity per subcategory based on reported annual emissions (less than 50 kt, 50-500 kt and over 500 kt).

In answering this question, use Table 7 of Part 2 of this Annex.

All installations provided emission reports on time. See also answer to question 6.2

6.4.	Which measures were undertaken in cases where operators did not provide an emission report by 31 March of the reporting period?

See answer to question 6.3.

6.5.	Did the competent authority carry out any independent checks on verified reports? If yes, please describe how additional checks were undertaken and/or how many reports were checked.

Klif verifies the emission reports. See answer to question 6.1

6.6.	Did the competent authority instruct the registry administrator to correct the annual verified emissions for the previous year for any installation(s) to ensure compliance with the detailed requirements established by the Member State pursuant to Annex V to Directive 2003/87/EC?

Indicate any corrections in Table 6 of Part 2.

No 2009 emissions were corrected.

6.7.	Is there any other relevant information concerning the arrangements for verification in your country? If so, please specify.

7.	Operation of registries

7.1.	Please provide any terms and conditions required to be signed by account holders and provide a description of the identity check of persons undertaken before creating holding accounts (cf. Commission Regulation (EC) No 2216/2004).

In answering this question, use the table below.

| Please provide the link to your registry | www.kvoteregister.no |
| --- | --- |
| Which of the following statements apply to your country (add explanatory text if necessary)? |  |
| Specific terms and conditions elaborated which account holders have to sign (if yes, please provide relevant documents or links) | Yes. The account holders have to agree to the terms and conditions set forth in the User agreement before first log in. The User agreement can be downloaded from the registry’s website: www.kvoteregister.no/docs/Terms%20and%20conditions.pdf |
| Different identity checks applied for operators and individuals | No |
| Personal presence required for ID checks for residents in Member State | Yes |
| ID check through written procedure only for residents | No |
| Personal presence required for ID checks for residents of other countries | Yes |
| ID check through written procedure only for residents in other countries | No |
| Copy of company register or similar documentation required for opening of operator holding account? | Yes |
| Documentation showing right to represent company required for opening of operator holding account? | Yes |
| Other (please specify): ____________________ |  |

7.2	Please provide a summary of all security alerts relevant to the national registry which have occurred during the reporting period, how they were addressed and the time taken for resolution.

In answering this question, use the table below. Add further rows if necessary.

| Which of the following statements apply to your country (add explanatory text if necessary)? |  |  |  |
| --- | --- | --- | --- |
| General procedures in place to prevent occurrence of security alerts |  | Yes |  |
| Security alerts relevant to national registries occurred during the reporting period |  | Yes |  |
| If yes, please fill out the following table |  |  |  |
| Type of security alert | Number of occurrences | Action taken | Time needed for resolution |
| Phising email sent to account representatives – requesting username and password | Once | Disconnected registry from ITL immediately. Suspended all users and informed and warned them of the false email. Checked and verified database that no fraudulent transactions had taken place. | Registry was disconnected for ca 10 hours. |

7.3.	Please state how many minutes for each month of the reporting period the national registry was unavailable to its users (a) due to scheduled downtime, and (b) due to unforeseen problems.

In answering this question, use the table below.

| Month | Scheduled downtime [minutes] | Unscheduled downtime [minutes] |
| --- | --- | --- |
| January | 0 | 0 |
| February | 0 | 0 |
| March | 0 | 0 |
| April | 570 | 0 |
| May | 0 | 0 |
| June | 0 | 0 |
| July | 0 | 0 |
| August | 0 | 480 |
| September | 0 | 0 |
| October | 0 | 0 |
| November | 0 | 0 |
| December | 0 | 0 |

7.4.	Please list and provide details on each upgrade to the national registry scheduled for the next reporting period.

In answering this question, use the table below. Add further rows if necessary.

| Which of the following statements apply to your country (add explanatory text if necessary)? |  |
| --- | --- |
| Regular time slots allocated for maintenance and upgrading of registry (if so, please provide dates) | No. If maintenance is necessary it will be performed preferably in early morning on working days. |
| Registry will be upgraded together with upgrade of software system used | Yes |
| Please provide details for all upgrades scheduled for the next reporting period |  |
| Date | Purpose |
| Greta v5.1 in August 2010 | - Enable operators to manually select Kyoto units when surrendering. - Simplify the compliance process for operators. - Improve the reconciliation process - Maintenance bug fixes. |

7.5.	Is there any other relevant information concerning the operation of registries in your country? If so, please specify.

Since Norway does not use third party verification of emission report, there are no verifiers entered in the registry. The registry team in Klif receives the verified emissions for each installation from the project group verifying the emission reports, and manually enters the verified emissions for the installations in the registry.

When the proposed system with third party verification is implemented, all accredited verifiers who are contracted by the operators to verify their emission reports must be registred in the Norwegian registry.

8.	Arrangements for the allocation of allowances — new entrants — closures

Preliminary remark:

Question 8.1 and 8.2 have been deleted on purpose as these questions are not relevant any more

8.3.	How many allowances were allocated to the new entrants listed in Table 1, if any? Please give the installation identification code for the new entrant and the transaction identification code associated with the allocation of allowances.

In answering this question, use Table 1 of Part 2 of this Annex.

There were two new entrants in 2009. None of them have yet been allocated allowances from the new entrant reserve.

8.4.	How many allowances were left in any new entrants reserve at the end of the reporting period, and what share do they represent of the original reserve?

In answering this question, use the table below.

| Number of allowances left in the new entrants reserve at the end of the reporting period (31 December each year) | 4 200 000 |
| --- | --- |
| Share of allowances remaining in the new entrants reserve, in percent | 100% |

8.5.	If your Member State allocates allowances other than for free, please explain how such allocation is made (e.g. way in which auctioning is undertaken)?

Starting on 18 May and ending on 13 November 2009, Norway sold regular 	volumes of EUAs, on a daily basis, through a combination of spot and December 	2009 deliveries. About 50 per cent were sold on a spot basis; 50 per cent on 	December 2009 delivery. The allocation between spot and futures might be changed 	by the Ministry of Finance. 31.8 million EUAs will be sold in the market. In 2009 	about 12.7 million EUAs were offered in the market by the Ministry of Finance. 	Following a public procurement process Barclays Capital (“Barclays”) was 	awarded the mandate to conduct the sale in 2009. The Ministry of Finance has an 	option to make further sales through Barclays in the 2010 – 2012 period.

8.6.	If auctioning was used as an allocation method, who was allowed to participate in the auction?

In answering this question, use the table below.

| National operators only | No |
| --- | --- |
| National registry account holders only | No |
| All Community operators | No |
| All bidders with an account in a Community registry | Yes |
| Other (please specify): _____________________ |  |

8.7.	If auctioning was used as an allocation method, how many auctions were held during the reporting period, how many allowances were auctioned during each auction, what share do they represent of the total quantity of allowances for the trading period and what was the price per allowance at each auction?

In answering this question, use the table below.

| Was auctioning used as an allocation method? | Yes (direct sales). See comment under table |
| --- | --- |
| If yes, please answer the following questions. |  |
| Number of auctions held during the reporting period (1 January to 31 December) | - |
| Number of allowances auctioned (each auction separately) | - |
| Clearing price of auction (each auction separately) | - |

Comment: Allowances are being sold in daily volumes in the exchange and OTC markets. In the common ETS jargon, this may be characterised as auctioning.

8.8.	If auctioning was used as an allocation method, what use was made of allowances not purchased at the auction(s)?

Not applicable.

8.9.	If auctioning was used as an allocation method, what were the revenues used for?

The revenues are used in the national budget for general purposes such as schools, 	health care, social security and environmental purposes.

8.10.	How were allowances treated that had been allocated but were not issued to installations that closed during the reporting period?

Three installations closed in December 2008 / January 2009. These closures were foreseen when the NAP table was adopted by ESA, and hence they were allocated allowances only for 2008. No allowances for allocated for the years 2009 – 2012.

Question 8.11 is to be answered in the first report following the end of the trading periods set out in Article 11(1) and (2) of Directive 2003/87/EC.

8.11.	Were allowances remaining in the new entrants’ reserve at the end of the trading period cancelled or auctioned?

Not applicable.

8.12.	Is there any other relevant information concerning the arrangements for allocation, new entrants and closures in your country? If so, please specify.

9.	Surrender of allowances by operators

9.1.	In all cases where an account in the registry was closed because there was no reasonable prospect of further allowances being surrendered by the installation’s operator, please describe why there was no reasonable further prospect and state the amount of outstanding allowances.

In answering this question, use the table below. Add further rows if necessary.

No operator holding accounts were closed in 2009.

| Reason for closure of account | Quantity of outstanding allowances (kt CO2 eq) |
| --- | --- |
| - | - |
| - | - |
| - | - |

9.2.	Is there any other relevant information concerning the surrender of allowances by operators in your country? If so, please specify.

10.	Use of emission reduction units (ERUS) and certified emission reductions (CERS) in the community scheme

10.1.	Have ERUs and CERs been issued for which an equal number of allowances had to be cancelled pursuant to Article 11(b)(3) or (4) of Directive 2003/87/EC because the Joint Implementation (JI) or Clean Development Mechanism (CDM) project activities reduce or limit directly or indirectly the emission level of installations falling under the scope of that Directive? If so, please provide the sum of allowances cancelled and the total number of operators concerned separately for cancellation pursuant to Article 11(b)(3) and (4) of that Directive.

In answering this question, use the table below.

|  | Quantity of allowances cancelled | Number of operators affected |
| --- | --- | --- |
| cancellation pursuant to Article 11(b)(3) | None | None |
| cancellation pursuant to Article 11(b)(4) | None | None |

10.2.	Which CERs and ERUs may be used for compliance in your Member State? Please state any project category excluded except those which are already excluded pursuant to Article 11(a)(3) of Directive 2003/87/EC (CERs and ERUs from nuclear or from land use, land use change and forestry project activities).

In answering this question, use the table below.

| CERs and ERUs from all project categories can be used | Yes |
| --- | --- |
| CERs and ERUs from certain project categories are excluded (if yes, please specify) | No |

10.3.	What measures have been taken to ensure that relevant international criteria and guidelines, including those contained in the year 2000 Final Report of the World Commission on Dams (WCD), will be respected during the development of hydroelectric power production projects with a generating capacity exceeding 20MW?

In answering this question, use the table below. Add further rows if necessary.

| Which of the following statements apply to your country (add explanatory text as necessary): |  |
| --- | --- |
| Project participants are legally obliged to adhere to the WCD guidelines | No. Norway has joined the “voluntary harmonisation” developed among European countries. |
| Adherence to WCD guidelines is verified (if so, please provide relevant authority, e.g. competent authority or Designated National Authority) | No. See previous answer. |
| Other international criteria and guidelines have to be respected during the development of large hydroelectric power projects (if so, please provide relevant documents or links) | No |
| Other (please specify): | Norway has not taken steps to host JI projects. Therefore, provisions of the Linking Directive (Directive 2004/101/EC amending Directive 2003/87/EC) do not apply to domestic hydro power projects. |

10.4.	Is there any other relevant information concerning the use of ERUs and CERs in the Community scheme in your country? If so, please specify.

11.	Fees and charges

Preliminary remark:

The fees stated in this chapter are given in Norwegian currency (NOK). The average exchange rate in 2008 was 1 euro = NOK 8.7285

11.1.	Are fees charged to operators for the issuance and update of permits? If so, please provide details on the fees charged, total proceeds and the use of the proceeds.

Fees are charged to operators for both issuance and update of permits.

Fees for the processing of new permits are divided into the following rates:

Category 1:	NOK 108 300 
	Category 2:	NOK 77 500 
	Category 3:	NOK 50 100
	Category 4: 	NOK 19 200

Fees for amendments to permits are divided into the following rates:

Category 1:	NOK 44 400 
	Category 2:	NOK 21 000
	Category 3:	NOK 5 100

The actual resources used in connection with the case processing, and hence the 	actual costs incurred by Klif, determine the rate which is set. The fee is set to apply in 	each case.

Total proceeds from the fees charged for issuance and updates of permits constituted 	NOK 129 500 in 2009.

11.2.	What fees are charged to operators for the issuance of allowances? If so, please provide details on the fees charged, total proceeds and the use of the proceeds.

The fee charged for issuance is set to NOK 0,56 per allowance issued. Given the fact that total allocation to incumbents for the 2008 – 2012 period is 39 397 187 and NE reserve is 4 200 000, the total proceeds for the whole period may reach NOK 24 424 425. The proceeds will be used to cover all costs incurred by Klif in relation to the administration of the emission trading scheme in Norway.

11.3.	What fees are charged for the use of the registry if any? Please give details.

In answering this question, use the table below?

| Which of the following statements apply to your country (add explanatory text as necessary)? |  |
| --- | --- |
| Fees are charged for the use of the registry | No |
| Different fees in place for operators and individuals | No |
| Fee for opening an account | Operators: NOK 400 once Individuals: NOK 400 once |
| Annual fee for maintaining account | Operators: NOK 400 Individuals: NOK 400 |
| Other (please specify): ________________ |  |

11.4.	Is there any other relevant information concerning fees and charges in the Community scheme in your country? If so, please specify.

12.	Issues related to compliance with the ET Directive

12.1.	Please state the relevant national provisions and the penalties for infringements of national provisions pursuant to Article 16(1) of the ET Directive.

In answering this question, use the table below. Add further rows if necessary.

| Kind of infringement | Relevant national provisions | Fines [EUR] |  | Imprisonment (months) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | min | max | min | max |
| Operation without permit | Pollution Control Act § 78 | -* | -* | - | 3 |
| Infringements of monitoring and reporting obligations | Pollution Control Act § 78 Greenhouse Gas Emission Trading Act (GGET) § 20 GGET § 22 | -* -* -* | -* -* -* | - - - | 3 - 3 |
| Omission to notify changes to the installation | GGET § 22 | -* | -* | - | 3 |
| Other (please specify): Surrender allowances | GGET § 21 | - | 100 /allowance not surrendered |  |  |

*No max/min level. The amount is determined on an individual basis.

12.2.	Where penalties were imposed pursuant to Article 16(1) of the ET Directive for infringements of national provisions, please state the relevant national provisions, briefly describe the infringement and give the penalties imposed.

No penalties were imposed.

In answering this question, use the table below. Add further rows if necessary.

|  |  | Penalties imposed |  |
| --- | --- | --- | --- |
| Infringement | National provisions | Fines (EUR) | Imprisonment (months) |

12.3.	Please provide the names of operators for which excess emission penalties were imposed pursuant to Article 16(3) of the ET Directive.

In answering this question, it is sufficient to provide a reference to the publication of the names under Article 16(2) of the ET Directive.

No operators were imposed excess emission penalties pursuant to Greenhouse Gas Emission Trading Act (GGET) § 21.

12.4.	Is there any other relevant information related to compliance with the ET Directive in your country? If so, please specify.

13.	The legal nature of allowances and fiscal treatment

13.1.	What is the legal nature of an allowance (commodity/financial instrument) for the purpose of financial regulation?

Commodity

13.2.	What is the legal status given to allowances and emissions for the purposes of accounting?

Commodity

13.3.	Were any specific accounting rules established or adopted for allowances? If yes, please describe them briefly.

Reverse charge. Changes to the VAT Act were introduced in order to prevent VAT fraud in connection with trading of emissions allowances.

13.4.	Are transactions of allowances subject to VAT?

Yes, allowances are treated as transferable services subject to VAT.

13.5.	Is the issuance of allowances subject to VAT?

Yes

13.6.	If your Member State allocates allowances for payment, is VAT due on the transaction?

Yes

13.7.	Are profits or losses from transactions of allowances subject to a specific income tax (e.g. specific tariffs)?

Profits or losses that descend from transactions of allowances are subject to income tax or tax deductions, respectively, according to general rules of the Act on Income Tax [skatteloven] as of 26 March 1999. The general tax rate of 28 percent on ordinary income applies.

13.8.	Is there any other relevant information concerning the legal nature of allowances and their fiscal treatment in your country? If so, please specify.

14.	Access to information pursuant to article 17 of the ET Directive

14.1.	Where are decisions relating to the allocation of allowances, information on project activities in which a Member State participates or authorises private or public entities to participate, and reports of emissions required under the greenhouse gas emissions permit and held by the competent authority made available to the public?

In answering this question, use the table below:

| Type of information | Information available to public | If information is available, at which location? |  |  |
| --- | --- | --- | --- | --- |
|  |  | Internet | Official Journal | Other (please specify) |
| Allocation rules | Yes | 1) Greenhouse Gas Emission Trading Act § 8 |  |  |
| NAP table | Yes | 2) |  |  |
| Changes to list of installations | Yes | 3) |  |  |
| Verified emission reports | Yes | 3) |  |  |
| Project activities | n.a. | - |  |  |
| Greenhouse gas emissions permit | On request only |  |  |  |
| Information required by Annex XVI to Regulation (EC) No 2216/2004 | Yes | 4) |  |  |
| Other (please specify): |  |  |  |  |

1) http://www.lovdata.no/all/hl-20041217-099.html

2) http://www.Klif.no/kvoteregister/kvoter/nap_tabell08-12.pdf

3) http://www.klif.no/Tema/Klima-og-ozon/CO2-kvoter/Klimakvoter-for-2008/

4) http://www.kvoteregister.no/publicreports and on the web site of CITL: http://ec.europa.eu/environment/ets/

14.2.	Is there any other relevant information concerning the access to information pursuant to Article 17 of the ET Directive in your country? If so, please specify.

15.	Other observations

Preliminary remark:

Question 15.1 has been deleted on purpose as this question is no longer relevant

15.2.	Are there any particular implementation issues that give rise to concerns in your country? If so, please specify.

OJ L 257, 10.10.1996, p. 26. Directive as last amended by Regulation (EC) No 166/2006 of the European Parliament and of the Council (OJ L 33, 4.2.2006, p. 1).

StatoilHydro ASA changed name to Statoil ASA. The company has 25 installations covered by the ETS.

OJ L 59, 26.2.2004, p. 1.

The waste types should be reported using the classification of the “European List of Wastes” (Commission Decision 2000/532/EC of 3 May 2000 replacing Decision 94/3/EC establishing a list of wastes pursuant to Article 1(a) of Council Directive 75/442/EEC on waste and Council Decision 94/904/EC of 22 December 1994 establishing a list of hazardous waste pursuant to Article 1(4) of Council Directive 91/689/EEC on hazardous waste.

United Nations Framework Convention on Climate Change.

European Pollutant Emission Register (Commission Decision 2000/479/EC of 17 July 2000), (OJ L 192, 28.7.2000, p. 36).

National Emissions Ceilings (Directive 2001/81/EC), (OJ L 309, 27.11.2001, p. 22).

Large Combustion Plants (Directive 2001/80/EC), (OJ L 309, 27.11.2001, p. 1).

Co-operative Programme for Monitoring and Evaluation of the Long-range Transmission of Air pollutants in Europe.

European Co-operation for Accreditation’s (EA) Guidance on the application of EN 45011.

OJ L 386, 29.12.2004, p. 1.

This includes ID checks by third parties like post offices or notary where the applicant has to present himself in person.

This includes electronic procedures.

This includes ID checks by third parties like embassies where the applicant has to present himself in person.

This includes electronic procedures.

If the amount of outstanding allowances is not known please provide an estimate of outstanding allowances based on the last verified emission report, remaining allowances in the account and other information available to the Competent Authority.

Indicate the relevant period as well (once/per trading period).

If fees depend on allocation please provide minimum and maximum fees if applicable and the relevant formula.

Please provide web address.

Please provide the title.