Check of implementation of the recommendations from the previous review, sent to Party prior to the review week

- ROMANIA

- Sector Energy

ERT asks Party to give explanation if and how the following paragraphs of the latest ARR were addressed.

Please react to underlined parts of text. Please quote the relevant page in NIR and/or Table in CRF if recommendations were addressed in 2009 submission and explain how single recommendations were implemented and evaluate a progress made from previous submission.

In cases of still pending recommendations please specify intention of Party to implement the recommendation in next submissions.

- Part A. Sector overview

28. In 2006, the energy sector accounted for 67.3 per cent (105,431.5 Gg CO2 eq) of total GHG emissions. Emissions from this sector increased by 3.3 per cent between 2005 and 2006, and decreased by 44.0 per cent between the base year and 2006. Key drivers of the decline between the base year and 2006 are the 53.9 per cent (57,338.2 Gg CO2 eq) decrease in emissions from energy industries, and a 48.4 per cent (18,165.4 Gg CO2 eq) decrease over the same time period in manufacturing industries and construction. Energy industries was the major category in 2006, contributing 46.4 per cent to total sector emissions, while manufacturing industries and construction, other sectors, transport and oil and natural

gas (1.B.2) contributed 18.4, 12.7, 11.7 and 8.3 per cent, respectively. CO2 is the dominant GHG, contributing 88.0 per cent to total sector emissions and 83.6 per cent to total GHG emissions, while CH4 and N2O contributed 11.6 and 0.4 per cent, respectively, to total sector emissions.

29. Emissions from the energy sector have been estimated using a tier 1 method and constant IPCC default EFs, including for key categories. The ERT reiterates the recommendation from the previous review regarding the development of higher tier methods to improve the accuracy of emission estimates of key categories, and requests that Romania resolve this problem and report thereon in its next annual submission.

Answer:

Romania intends to follow in the next years the recommendations on using a higher tier  calculation approach in order to improve the accuracy of the estimates of the emissions for the key categories. The implementation of such methods will take into account the availability of resources needed for the development of necessary data.

30. The inventory is generally complete. However, the ERT identified a number of gaps in the CRF submission. The ERT recommends that Romania explore the possibility of providing emission estimates in its next annual submission for the following categories: stationary combustion – other fuels (all GHGs); mobile (1.A.5.b) (military) – all fuels (all GHGs); road transport – LPG (CH4 and N2O); road transport – biomass (all GHGs); coal mining and handling – CO2 and N2O; solid fuel transformation – all GHGs; and various activities under oil and natural gas (1.B.2).

Answer:

Activity data provided by the National Institute for Statistics (NIS) in the Energy Balance (EB) does not allow to estimate specific emissions from the listed categories. 
Romania intends to provide relevant information in this way in the next submissions, depending on the availability of resources necessary to enable data development.

31. The energy sector is generally transparent considering that the Party has used a tier 1 method. QA/QC procedures are used by Romania in preparing the energy inventory and these procedures are adequately described in the NIR. The NIR also provides information on the uncertainties based on the tier 1 approach. Romania has aggregated emissions (and AD) for all categories within manufacturing industries and construction under other (1.A.2.f), and for petroleum refining and manufacture of solid fuels and other energy industries under public electricity and heat production. The ERT recommends that Romania endeavour to improve transparency in the reporting of the above-mentioned aggregated categories in both the CRF tables and the NIR.

Answer:

Data from the EB does not allow for the estimation of specific emissions of categories listed separately. 
Romania intends to provide relevant information in this way in the next submissions, depending on the availability of resources necessary to enable data development.

32. Romania has performed recalculations in response to recommendations from the previous

review. Recalculations include: correction of the double counting of emissions from manufacturing industries and construction – coke (CO2) for the period 1992–2005; revision of the domestic/international  bunker split of CO2 emissions from navigation and aviation for the period 1989–2005; and a correction of AD for oil and natural gas (1.B.2) – CH4 for the period 1989–2005. Recalculations are time-series consistent. However, the ERT noted that other recalculations have been undertaken by the Party (e.g. other sectors) that are not explained in either the NIR or table 8(b).

33. The ERT noted that Romania is planning to conduct a national study in 2009 to develop countryspecific EFs for fuel combustion in key source categories, and that it also plans to apply the COPERT transport model. The ERT recommends that Romania provide information on the implementation of these improvements in its next annual submission.

Answer:

- Due to a lack of resources, Romania could not develop for using within the 2009 submission national emission factors specific to the fossil fuel combustion activities. However, Romania intends to develop in the future years key-category specific emission factors.

- Romania has examined the possibility of applying the COPERT model but the necessary data are available only for the period 2004 to 2008. Depending on resource availability, Romania intends to develop the necessary data series and apply the COPERT model for the next submissions.

- Part B1.  Comparison RA with SA and international statistics

34. CO2 emissions from fuel combustion were calculated using the reference approach and the sectoral approach, with differences between the approaches varying between 2.9 and 21.8 per cent. For 2006, the difference is 8.8 per cent and explanations are provided in the CRF documentation box. In addition, the NIR provides explanations for the fluctuations in the differences between the two approaches over the inventory time series. The ERT reiterates the recommendation from the previous review regarding the reporting of the notation key not applicable (NA) in CRF table 1.A(c) for apparent energy consumption that excludes non-energy use and feedstocks, and requests Romania to resolve this problem and to report thereon in its next annual submission.

Answer:

Activity data provided by NIS through the EB does not allow to estimate specific emissions of the listed categories.

Romania intends to provide relevant information in this way in the next submissions, depending on the availability of resources necessary to enable data development.

- Part B2.  Bunker fuels

35. Romania has revised its calculation of emissions from bunker fuels in response to a

recommendation of the previous expert review team. The Party has developed a new approach to splitting domestic and international bunkers for aviation and navigation using new data from the Romanian Civil Aeronautical Authority for aviation, and from the statistical yearbook for navigation. The methodology is tier 1 and in line with the IPCC good practice guidance.

- Part B3. Feedstocks

36. Feedstocks and non-energy use of fuels are reported in the reference approach, using data from the national energy balance. However, apparent energy consumption excluding non-energy use and feedstocks is not provided in CRF table 1.A(c) (see para. 34 above).

Answer:

In table 1.A(c) apparent energy consumption excluding non-energy use and feedstocks are provided for fuel categories: liquid, solid and gaseous. For the category “Other” there are no relevant data regarding composition/applicable EF, in order to estimate emissions.

- Part C. Key categories

- 1. Stationary combustion: solid fuels - CO2

37. The ERT noted that the implied emission factor (IEF) for other (1.A.2.f) increased from 111.1 t per TJ in 1992 to 160.0 t per TJ in 2005. Romania did not provide an explanation in the NIR for this trend, nor any data to verify it. The ERT recommends that Romania provide sufficient documentation and explanation in its next annual submission on such trends.

Answer:

For each fuel type the same EF is used, regardless of the subsector in which it is being consumed. Therefore, the IEF depends on the EF specific to each fuel.

38. The ERT could not confirm with Romania whether it had rectified the double counting in the energy and the industrial processes sectors identified by the previous expert review team in relation to coke. The ERT recommends that Romania provide sufficient documentation in its next annual submission to confirm that the double-counting issue has been resolved, with a focus on providing data on coking coal, coke (energy and non-energy), coke gas and furnace gas energy and carbon balance in order to show that there is no double counting or omission of any furnace and coke gas in the energy balance.

Answer:

Romania has confirmed since the 2006 v.2 submission’s revision the correction of the coke double counting issue between the energy and the industrial processes sectors.

- 2. Oil nad natural gas - CH4

39. The ERT noted that Romania did not estimate fugitive emissions from the transit of natural gas through the country. The ERT recommends that Romania explore options for obtaining these data (e.g. National Gas Transmission Company). The ERT also noted that Romania used IPCC default EFs that reflect the technology of the early 1990s and will overestimate fugitive emissions from natural gas distribution for the later years. Considering that this is a key category, the ERT recommends that Romania consider establishing a study to develop country-specific EFs for exploration of oil and gas, distribution of oil products and oil flaring, based on the length, materials and condition of the distribution networks, and encourages Romania to use the tier 2 method.

Answer:

Activity data provided by NIS through the EB does not allow for the estimation of the fugitive emissions from natural gas transit and for the use of calculation methods related to a higher tier  approach for estimating fugitive emissions from natural gas distribution. 
Romania intends to provide relevant information in this way for the next submissions, depending on availability of resources necessary to enable data development.

- Part D. Non-key categories

- 1. Road transportation: liquid fuels - CH4, N2O

40. The ERT noted that Romania has used a default N2O EF (0.6 kg N2O per GJ) for cars with uncontrolled emissions. Recently the Romanian car fleet has more than doubled, with new vehicles having emissions control technologies that increase the N2O EF by one order of magnitude. The ERT recommends that Romania develop capacity to estimate emissions using the COPERT model.

Answer:

Romania has examined the possibility of applying the COPERT model but the necessary data are available only for the period 2004 to 2008. Depending on the resource availability, Romania intends to develop the necessary data series and apply the COPERT model for preparing the next submissions.