7 th Centralized Review of the Romania GHG inventory submission 2009

Questions by the expert review team

- What are the institutional arrangements for GHG inventory preparation for the Agriculture sector in Romania? Is there any agricultural organization involved in the Inventory preparation?

Response:

Based on the Governmental Decision no. 1570 for establishing the National System for the

estimation of anthropogenic greenhouse gas emissions levels from sources and removals

by sinks, adopted in 2007, and the subsequent relevant procedures:

- both the National Institute for Statistics (NIS) and the Ministry of Agriculture (MA) are the potential data providers;

- the direct data provider, on an annual basis, is NIS which works in close relation to the MA;

- the National Environmental Protection Agency (NEPA) is preparing the Agriculture sector of the Romanian GHG Inventory

- Could you provide the ERT explanations on differences in livestock populations (1000 heads) reported by the FAO for Romania and in Tables 4.A, 4.B of the CRFs? For some years the data is not comparable within 2% those reported by the FAO for Romania.

|  | 1989 | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cattle (FAO) | 6416,00 | 6290,70 | 5380,70 | 4355,00 | 3683,10 | 3956,78 | 3480,80 | 3496,30 | 3434,90 | 3235,40 | 3143,00 | 3051,00 | 2870,00 | 2800,00 | 2878,00 | 2897,00 | 2808 | 2862 | 2934 |
| Cattle (ROU) | 6327,46 | 6204,18 | 5306,74 | 4294,90 | 3632,17 | 3547,36 | 3432,96 | 3447,76 | 3387,60 | 3190,36 | 3008,90 | 2830,39 | 2761,36 | 2838,28 | 2857,02 | 2774,27 | 2 816,86 | 2 893,00 | 2 786,83 |
| Difference, % | -1,38 | -1,38 | -1,37 | -1,38 | -1,38 | -10,35 | -1,37 | -1,39 | -1,38 | -1,39 | -4,27 | -7,23 | -3,79 | 1,37 | -0,73 | -4,24 | 0,32 | 1,08 | -5,02 |
| Goats (FAO) | 1078,00 | 1017,20 | 1004,80 | 954,00 | 805,00 | 776,00 | 745,10 | 705,30 | 654,40 | 610,00 | 585,00 | 558,00 | 538,00 | 525,10 | 633,00 | 678,00 | 661 | 687 | 727 |
| Goats (ROU) | 1078,00 | 1017,00 | 1005,00 | 954,00 | 805,00 | 776,00 | 745,00 | 705,00 | 654,00 | 610,00 | 558,00 | 538,00 | 525,00 | 633,00 | 678,00 | 660,72 | 686,77 | 727,41 | 865,07 |
| Difference, % | 0,00 | -0,02 | 0,02 | 0,00 | 0,00 | 0,00 | -0,01 | -0,04 | -0,06 | 0,00 | -4,62 | -3,58 | -2,42 | 20,55 | 7,11 | -2,55 | 3,9 | 5,9 | 19,0 |
| Hourse (FAO) | 702,00 | 663,00 | 670,00 | 749,00 | 720,90 | 751,00 | 784,00 | 806,00 | 816,00 | 822,00 | 839,00 | 858,00 | 865,00 | 860,00 | 879,00 | 897,00 | 840 | 834 | 805 |
| Hourse (ROU) | 702,00 | 663,00 | 670,00 | 749,00 | 721,00 | 751,00 | 784,00 | 806,00 | 816,00 | 822,00 | 858,00 | 865,00 | 860,00 | 879,00 | 897,00 | 839,59 | 833,95 | 804,87 | 862,00 |
| Difference, % | 0,00 | 0,00 | 0,00 | 0,00 | 0,01 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 2,26 | 0,82 | -0,58 | 2,21 | 2,05 | -6,40 | -0,72 | -3,49 | 7,08 |
| Swine (FAO) | 14351,00 | 11671,00 | 12003,30 | 10954,10 | 9852,31 | 9262,00 | 7758,00 | 7758,00 | 8234,50 | 7097,00 | 7194,00 | 5848,00 | 4797,00 | 4447,00 | 5058,00 | 5145,00 | 6495 | 6622 | 6815 |
| Swine (ROU) | 14351,00 | 11671,00 | 12003,00 | 10954,00 | 9852,00 | 9262,00 | 7758,00 | 7960,00 | 8235,00 | 7097,00 | 5848,00 | 4797,00 | 4447,00 | 5058,00 | 5145,00 | 6494,67 | 6 622,30 | 6 814,61 | 6 564,91 |
| Difference, % | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 2,60 | 0,01 | 0,00 | -18,71 | -17,97 | -7,30 | 13,74 | 1,72 | 26,23 | 1,96 | 2,91 | -3,67 |
| Sheep (FAO) | 16210,00 | 15434,80 | 14061,90 | 13879,40 | 12079,40 | 11499,20 | 10896,60 | 10380,90 | 9662,60 | 8937,60 | 8409,00 | 8121,00 | 7657,00 | 7251,20 | 7312,00 | 7447,00 | 7425 | 7611 | 7678 |
| Sheep (ROU) | 16210,00 | 15435,00 | 14062,00 | 13879,00 | 12079,00 | 11499,00 | 10897,00 | 10381,00 | 9663,00 | 8937,00 | 8121,00 | 7657,00 | 7251,00 | 7312,00 | 7447,00 | 7425,33 | 7 610,96 | 7 678,21 | 8 469,20 |
| Difference, % | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | -0,01 | -3,42 | -5,71 | -5,30 | 0,84 | 1,85 | -0,29 | 2,50 | 0,88 | 10,30 |

Response:

According to the relevant methodological note on FAO database, “total live animal numbers have to be considered for the year ending 30 September (e.g. animals enumerated in a given country any time between 1 October and 30 September of the following year should be considered for the later year)”.

Data used within the Romanian GHGI for the year X are livestock data relevant to 1 december of year X.

- Please provide more detailed information on dividing number of animals. It is not clear from the NIR that total number of Buffalos includes males and females Buffalos. How did Romania implement recommendations from previous ARR 2008 in relation to AD for dairy and non-dairy cattle?

Response:

Total number of Buffalos includes males and females Buffalos.

Total bovines data are provided by NIS being released through Statistical Yearbook (SY) 1989-2008 and other relevant correspondence. Beginning with 2004, NIS provides to Eurostat a more complete set of data, comprising also Buffalo data.

By expert judgment, we extended the Buffalo data series to 1989-2003 period,

considering that 1.38% of Total bovines are Buffalo (the percentage was obtained using the arithmetic mean of Buffalo values for 2004 and 2005).

In response to the recommendation from ARR 2008 related to dairy and non-dairy cattle, the emission factors values for Dairy cows have been changed in order to reflect the milk productivity values (recalculations have been performed for 1989-2006).

Considering the provisions in IPCC GPG 2000 corroborated to those in IPCC 1996

Reference Manual, the old default constant emission factor has been replaced with yearly emissions factors calculated through interpolation between default emissions factors values, using the Average milk production per animal (cow and buffalo cow) data series.

- Does Romanian statistic provide the separate data on cow’s milk productivity? Please provide these data if they are.

Response:

The National Institute for Statistics developed for 1989-2003 and released through the Statistical Yearbook data on Cow’s and buffalo cow’s milk (calfs feeding included) production and on the Average production per animal (cow’s and buffalo cow’s milk). Relevant data are attached.

- Does Romanian statistic provide annual data for cultivated organic soils? As reported in EuroStat database total organic crop area is 65111ha for 2006. See as reference:

http://epp.eurostat.ec.europa.eu/portal/page/portal/agriculture/data/database

Response:

Although officially asked, the National Institute for Statistics and the Ministry of Agriculture could not provide data on the area of organic soils cultivated annually.

After analyzing your statement on the availability of this data within the FAO database we concluded that here might be a misunderstanding, as:

- total organic crop area in the FAO context means agricultural area used for obtaining organic crops. Also, “organic farming can be defined as a method of production which places the highest emphasis on environmental protection and, with regard to livestock production, animal welfare considerations. It avoids or largely reduces the use of synthetic chemical units such as fertilisers, pesticides, additives and medicinal products” (http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/en/org_base.htm);

- considering the IPCC GPG 2000, “organic soils are soils described as Histosols which are defined as: ‘Organic soils that have organic soil materials in more

than half of the upper 80 cm, or that are of any thickness of overlying rock or fragmented materials that have interstices filled

with organic soil materials.’ An organic soil material is defined as: ‘soil materials that are saturated with water and have 174

g kg-1 or more organic carbon if the mineral fraction has 500 g kg-1 or more clay, or 116 g kg-1 organic carbon if the mineral

fraction has no clay, or has proportional intermediate contents, or if never saturated with water, have 203 g kg-1 or more organic carbon (SSSA, 1996)”.

- CH4 emissions from enteric fermentations are key category; Tier 1 with IPCC default values was applied. How did Romania address the previous review recommendations on improvements and using higher Tier 2 approach?

Response:

Considering that the CH4 emissions from enteric fermentation is a key category, Romania intends to develop data which allow for the use of the Tier 2 approach within the next submissions; however, due to the lack of resources, Romania could not develop this data until now.

- For estimates of emissions from manure management IPCC default values on percentage of manure N in different AWMS were used. As indicated in previous ARR 2008 that animals spent 50% of time on pasture for grazing and 50% on other AWMS. But as reported in the CRF table 4D the fraction of livestock N excreted on pastures is 0,32. Please provide explanations.

Response:

Due to the lack of national data, within the 2009 submission of the GHGI, Romania considers only default values for:

- the nitrogen excretion per head of animal per region;

- the percentages of manure N produced in different AWMS

Within the 2009 submission of the GHGI, Romania used the expert judgment only to calculate the FracPRP values for every year, using the following equation:

FracPRP = Nex(Pasture Range and Paddock)/Nex

where

Nex(Pasture Range and Paddock) = nitrogen excretion from Pasture Range and Paddock

Nex = nitrogen excretion from all Animal Waste Management Systems

- For N2O emissions from manure management estimates Tier 1 was applied. As reported in CRF 4 B(b) table for other AWMS IEF 0,04 kg N2O-N/kg N was used and in the NIR table 6.16 p. 178 default EF 0,05 kg N2O-N/kg N was applied. Please provide explanations.

Response:

According to the provisions in IPCC GPG 2000, the calculation methodology took into account IPCC default emissions factors (Table 4-12 of IPCC GPG 2000 together with Table 4-22 of Reference Manual). The emission factors for animal waste per AWMS comprise also:

- for Pit (applicable only to Swine) – the 0.001 kg N2O-N/kg N excreted value;

- for Other AWMS – the 0.005 kg N2O-N/kg N excreted value

Due to the fact that the CRF Reporter does not allow for a distinct reporting of the Nitrogen excretion specific to the Pit AWMS, although this value can be calculated distinctly, the Pit and Other AWMS values are reported together under the Other AWMS.

- In CRF table 4 D for fraction manure burned for fuel the notation key NA has been used. If this activity doesn’t occur in Romania it should apply NO notation key.

Response:

The appropriate action on the use of the notation key will be performed within the next year submission.

- In CRF table 4 F for Rye the notation key IE has been used because of unavailable data for rye production for whole time series and wheat and rye data were used. But FOA for Romania provide such separate data for all time series 1990-2007. The ERT note that residue/crop product rates are different for these crops.

Response:

NIS data series on crop productions do not comprise data on Rye production for 1989-1998. According to the provisions of the IPCC methodologies, due to the relevance of NIS as the national entity for statistics we considered the NIS data.