- Agriculture

- Questions to Romania

- 1. Since last submission Romania has made recalculations in estimates of CH4 emissions from rice cultivations because of new activity data was provided by Ministry of agriculture and water regime practice has changed from continuously flooded to intermittently flooded. However, the NIR doesn’t provide explonations on such changes. Could you please provide explanations?

Response:

Comparing to the previous submission, recalculations have been performed only at emission factors level. Relevant information is presented below.

Status of emission factors used

Considering the provisions in IPCC GPG 2000 and the new data provided by the Ministry of Agriculture, the calculation methodology took into account:

- a seasonally integrated emission factor value for continuously flooded fields without organic amendments (EFc) of 20 g CH4/m2;

- a default value of 0.2 for the scaling factor to account for the differences in ecosystem and water management regime (SFW) corresponding to lowland – irrigated – intermittently flooded – multiple aeration water management regime;

- yearly default values for the scaling factor to account for both type and amount of amendment applied (SFO). Default values were selected after the estimation of the rice residues productivity values, considering that all rice residues were incorporated into the soil following the harvesting. Rice residues productivity values and default values for the scaling factor to account for the type and amount of amendment applied are presented in the following Table.

Table Rice residues productivity values and default values for the scaling factor to account for the type and amount of amendment applied (SFO)

| Year | Rice residues productivity [tones d.m./ha] | Scaling factor to account for the type and amount of amendment applied (SFO) |
| --- | --- | --- |
| 1989 | 1.07 | 1.5 |
| 1990 | 1.25 | 1.5 |
| 1991 | 1.09 | 1.5 |
| 1992 | 1.78 | 1.5 |
| 1993 | 2.28 | 1.8 |
| 1994 | 2.48 | 1.8 |
| 1995 | 2.92 | 1.8 |
| 1996 | 2.04 | 1.8 |
| 1997 | 2.01 | 1.8 |
| 1998 | 2.25 | 1.8 |
| 1999 | 1.78 | 1.5 |
| 2000 | 1.93 | 1.5 |
| 2001 | 0.94 | 1 |
| 2002 | 0.90 | 1 |
| 2003 | 2.25 | 1.8 |
| 2004 | 3.13 | 1.8 |
| 2005 | 2.75 | 1.8 |
| 2006 | 2.46 | 1.8 |
| 2007 | 2.46 | 1.8 |

Status of recalculations performed

Recalculations at the emission factors level, performed in order to improve the quality of emissions estimates, comprise:

- in order to account for the lowland – irrigated – intermittently flooded – multiple aeration water management regime currently used, according to the data provided by the Ministry of Agriculture, the default value specific to the scaling factor to account for the differences in ecosystem and water management regime (SFW) has been changed from 1 to 0.2;

- in order to account for the rice residues incorporated into the soil after harvesting, considering the data provided by the Ministry of Agriculture, the value of the scaling factor to account for both type and amount of amendment applied (SFO) has been changed from a constant value of 1 to a distinct yearly value

All the changes made at the emission factors level and their implications on emission estimates are described in Table below.

Table Changes made at emission factors level and their effects on emission estimates

| Year | Changes on the scaling factor to account for the differences in ecosystem and water management regime (SFW) |  | Changes on the scaling factor to account for type and amount of amendment applied (SFO) |  |  | Effects of changes on emission estimates |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | NIR 2008 – CH4 emissions [Gg] | NIR 2009 - CH4 emissions [Gg] | Difference [%] |
|  | NIR 2008 | NIR 2009 | NIR 2008 | NIR 2009 |  |  |  |  |
| 1989 | 1 | 0.2 | 1 | 1.5 |  | 9.86 | 2.96 | -70 |
| 1990 | 1 | 0.2 | 1 | 1.5 |  | 7.98 | 2.39 | -70 |
| 1991 | 1 | 0.2 | 1 | 1.5 |  | 4.32 | 1.30 | -70 |
| 1992 | 1 | 0.2 | 1 | 1.5 |  | 3.28 | 0.98 | -70 |
| 1993 | 1 | 0.2 | 1 | 1.8 |  | 2.4 | 0.86 | -64 |
| 1994 | 1 | 0.2 | 1 | 1.8 |  | 0.92 | 0.33 | -64 |
| 1995 | 1 | 0.2 | 1 | 1.8 |  | 1.24 | 0.45 | -64 |
| 1996 | 1 | 0.2 | 1 | 1.8 |  | 1.7 | 0.61 | -64 |
| 1997 | 1 | 0.2 | 1 | 1.8 |  | 0.8 | 0.29 | -64 |
| 1998 | 1 | 0.2 | 1 | 1.8 |  | 0.34 | 0.12 | -64 |
| 1999 | 1 | 0.2 | 1 | 1.5 |  | 0.32 | 0.10 | -70 |
| 2000 | 1 | 0.2 | 1 | 1.5 |  | 0.28 | 0.08 | -70 |
| 2001 | 1 | 0.2 | 1 | 1 |  | 0.24 | 0.05 | -80 |
| 2002 | 1 | 0.2 | 1 | 1 |  | 0.1 | 0.02 | -80 |
| 2003 | 1 | 0.2 | 1 | 1.8 |  | 0.02 | 0.01 | -64 |
| 2004 | 1 | 0.2 | 1 | 1.8 |  | 0.24 | 0.09 | -64 |
| 2005 | 1 | 0.2 | 1 | 1.8 |  | 0.78 | 0.28 | -64 |
| 2006 | 1 | 0.2 | 1 | 1.8 |  | 1.12 | 0.40 | -64 |
| 2007 |  | 0.2 |  | 1.8 |  |  | 0.60 |  |