2009-09-26

the ERT of 7th centralized review

Completeness and other issues on Romania

The completeness of the annual  submission under the Kyoto Protocol is an important consideration for the forthcoming expert review of the 1st mandatory submission of the 1st commitment period which is due 15 April 2010.  In this context, paragraph 3 of decision 20/CMP.1 states that adjustments shall be applied when submitted GHG inventory data are found to be incomplete and/or prepared in a way that is not consistent with the Revised 1996 IPCC Guidelines for national greenhouse gas inventories as elaborated by the IPCC good practice guidance and any good practice guidance adopted by the CMP (this includes the IPCC good practice guidance for LULUCF).  If completeness issues remain unresolved, the expert review teams can consider applying adjustments.

This refers to categories which are reported as not estimated (“NE”) and for which the Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories, the IPCC Good Practice Guidance and the IPCC Good Practice Guidance for LULUCF provide methodologies for its estimation.

In particular, the ERT brings to the attention of Romania the reporting of notation key “NE” for the following categories:

- 1A3b Fuel Combustion Activities, Road Transport, LPG: CH4

Response:

In the table I-7 of the Reference Manual (Revised 1996 IPCC Guidelines for NGHGI) no EF for CH4 is provided.

We are considering the use of CH4 EF provided in the Table 3.2.2 from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories .

- 1B2a Fugitive Emissions, Oil - Expolration: CH4, CO2

Response:

No activity data are available, covering the entire 1989-2007 period. Another challenge is to determine appropriate EFs.

After the AD will be available, we will consider the use of CH4 and, respectively, CO2 EFs provided in the Table 4.2.5 from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories .

- 1B2b Fugitive Emissions, Gas - Expolration: CH4, CO2

Response:

No activity data are available, covering the entire 1989-2007 period. Another challenge is to determine appropriate EFs.

After the AD will be available, we will consider the use of CH4 and, respectively, CO2 EFs provided in the Table 4.2.5 from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories.

- 1B2c Fugitive Emissions, Oil - Venting: CH4, CO2

Response:

No EFs were provided by the IPCC 1996.

The use of EF provided in the Table 4.2.5 from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories it is being considered.

- 1B2c Fugitive Emissions, Oil - Flaring: CH4, CO2

Response:

No activity data are available, covering the entire 1989-2007 period. Another challenge is to determine appropriate EFs.

The use of EF provided in the Table 4.2.5 from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories it is being considered.

- 1B2c Fugitive Emissions, Gas - Flaring: CH4, CO2

Response:

No activity data are available, covering the entire 1989-2007 period. Another challenge is to determine appropriate EFs.

The use of EF provided in the Table 4.2.5 from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories it is being considered.

- 2.B 5 Other - Dichloroethylene	CH4

- 2.B 5 Other - Styrene		CH4

- 2.B 5 Other - Methanol		CH4

Response on Industrial Processes:

The activity data of the following categories Dichloroethylene (2.B 5 Other) and Styrene (2.B 5 Other) and the CH4 emissions related with these productions have been reported as NE in the CRF because for the last submission (analysing the 1989-2007 period) we did not have any information on Dichloroethylene and Styrene productions. For the next submission we will use the NO in the CRF for these categories because the productions are not occurring in our country (according with the recently information provided by the Ministry of Economy).

The data related with Methanol production (2.B 5 Other) have been provided by the National Institute for Statistic (NIS; for the entire 1989-2007 period), but starting with 2007 the production data on methanol have been provided by NIS as confidential (C).

- 5.A Forst land 2 Land converted to forest land CO2: organic soils, all carbon pools

- 5.B Cropland CO2: organic soils, all carbon pools

- 5.C Grassland CO2: organic soils, all carbon pools

- 5.D Wetlands CO2: organic soils, all carbon pools

- 5.C Settlements CO2: organic soils, all carbon pools

- 5.C Other Land CO2: organic soils, all carbon pools

- 5(III) N2O emissions from disturbance associated with land-use conversion to cropland N2O: area, emissions

- 5(IV) CO2 emissions from agricultural lime application CO2: amount of lime, emissions

Response on LULUCF:

Romania plans to obtain data and estimate emissions/removals levels for LULUCF categories presently assign with NE until the next GHG Inventory submission.

Romania may consider providing revised estimates or explanations as a response to possible underestimates, and provide estimates or explanation as to why the categories reported as “NE” were not estimated, to the ERT by 9 November 2009.