Romanian Informative Inventory Report 2008

Submission  under  UNECE  Convention

on Long-range Transboundary Air Pollution

Prepared by the National Environmental Protection Agency

March 2010

1. INTRODUCTION

This report is Romanian Informative Inventory Report (IIR) regarding the national inventory submitted in February 2010.

Romania signed the Convention on Long-Range Transboundary Air Pollution (CLRTAP) from 1979, transposed in national legislation by Law 271/2003 on CLRTAP and its protocols signed in Geneva on 13 November 1979, in Aarhus on 24 June 1998 and in Gothenburg on 1 December 1999.

In this direction, annual estimated emissions of the atmospheric pollutants at national level are being realized within the framework of reporting obligations assumed by Romania.

1.1. Institutional arrangements:

The inventory system currently used in Romania is presented in figure below. The National Environmental Protection Agency from Romania has the overall responsibility and submits the inventory report to CLRTAP.

UNECE

CLRTAP Secretariat

Local Environmental Protection Agencies (LEPA)

Local data collection

National Institute of Statistics (NIS)

Main statistical data provider

National Environmental Protection Agency (NEPA)

Inventory preparation

Ministry of Agriculture and Rural Development (MAPDR)

Agriculture sector data provider

Romanian Auto Registry (RAR)

Transport sector data provider

“Romanian Waters” National Administration (ANAR)

EEA and CEIP

Sources of the data used for the inventory are listed in the table below:

| Category | Main sources of information - Publications | Institution |
| --- | --- | --- |
| 1. Energy | Energy Balance | NIS |
|  | Statistical Yearbook of Romania | NIS |
| 2. Industrial processes | Statistical Yearbook of Romania | NIS |
|  | Production of industrial goods | NIS |
| 3. Solvent and other product use | Statistical Yearbook of Romania | NIS and LEPA’s |
| 4. Agriculture | Pesticides an fertilizers consumption | MADR (Ministry of Agriculture and Rural Development) |
|  | Statistical Yearbook of Romania | NIS |
| 5. Forestry | Forestry | MADR (Ministry of Agriculture and Rural Development) |
|  | Statistical Yearbook of Romania | NIS |
| 6. Waste | Environmental Protection | NEPA |
|  | Statistical Yearbook of Romania | NIS |
|  | Data regarding hospital wastes | ISPB (Romanian Public Health Institute) |
|  | Data regarding waste water | ANAR |
| 7. Transport | Fuel consumption | NIS |
|  | National fleet | RAR |

1.2. Methodology

The methodology used in estimating and reporting emissions is according to the last version of the guidebook elaborated under the patronage of the Executive Committee of the CLRTAP, “EMEP/EEA air pollutant emission inventory guidebook- 2009”.

The range of pollutants covered by this methodology guide is: SO2, NOx, NH3, NMVOC, CO, CO2, TSP, PM10, PM 2,5, heavy metals, (Cd, Pb, Hg, As, Cr, Cu, Ni, Se, Zn), POP (HCB, HCH, PCB, dioxins/furans), PAH.

The input data was processed using the CollectER III applications. The emissions obtained according to this database are used to complete the last version of the reporting formats requested by the UNECE/CRLTAP Secretariat.

Road transport emissions were estimated using COPERT III and COPERT 4 was used for estimating the NMVOC emissions resulted from gasoline evaporation.

There have been major differences from the last submissions as a result of using the new guidebook. This situation implied the recalculation of the 2007 emissions. For the former years the recalculations are currently in progress.

2. CALCULATING EMISSIONS (NATIONAL LEVEL)

2.1. ENERGY (NFR SECTOR  1)

1.A. Combustion

      - 1.A.1 Combustion in energy industries

      - 1.A.2 Combustion in Manufacturing Industries and construction

      - 1.A.3.b Road transport

      - 1.A.3.b.v Gasoline evaporation

      - 1.A.3.c Railways

      - 1.A.3.d Navigation

      - 1.A.4 Small combustion

1.B. Fugitive Emissions from Fuels

      - 1.B.1.a Coal Mining and Handling

      - 1.B.1.b Solid Fuel Transformation

      - 1.B.2.a.i, 1.B.2.b Exploration Production Transport

      - 1.B.2.a.iv Refining, Storage

      - 1.B.2.a.v Distribution of oil products

The NFR Category - ENERGY represents the main source of emission in Romania. This sector has a share of about 95% of total SO2,  88% of total NOx and 93% of CO. The share of the other pollutants is presented in table 1.

For estimation of emissions from the sources included in this category TIER 1 approaches were used except for some sources included in 1A2b and 1A2fi sectors, where TIER 2 approaches were used. Details about the level of approaches are presented in table 2.

1.A. Combustion

1.A.1 Combustion in energy industries

1.A.1.a — Public electricity and heat production

This activity covers combustion processes from production of electric power and thermal energy in public power and district heating.

The emissions data reported annually under LCP Directive (2001/80/EC) are the basis for the estimating of the emissions for 1A1a. The type of fuel and the percentage of specific fuel consumption used to produce electric power and thermal energy were distributed according to the types and capacities of the boilers.

1.A.1.b – Petroleum refining

To emissions data are taken from the LCP inventory.  This inventory is compiled using data submitted by the refineries operators.

1.A.1.c – Manufacture of solid fuels and other energy industries

In order to estimate emissions resulted from these types of activities, data provided by the NIS are taken into consideration (energy balance and industrial activity data).

There are no national emission factors developed, so emission factors are those from the last version of the guidebook.

1.A.2 Combustion in Manufacturing Industries and construction

This chapter covers combustion activities in industry, for the following source categories: Iron and Steel, Non-ferrous metals, Chemicals, Pulp,Paper and Print, Food processing, Beverages and Tobacco.

1.A.2.a — Iron and steel

The emissions resulted from iron and steel production have been counted starting from the data regarding the industrial activity and from the operators (these date are stored in databases, developed at county level, by the Local Environmental Protections Agencies).

1.A.2.b — Non-ferrous metals

The activity data are provided by NIS and the emission factors are those from the guidebook

1 A 2 f i –Other - Stationary combustion in manufacturing industries and construction

In this sector are included the following activities: production of mineral and glass wool (except binding), asphalt concrete plants, manufacturing bricks and tiles and. cement industry.

Emissions from combustion in the chemicals sector (1.A.2.c – Chemicals), production of pulp and paper (1.A.2.d – Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print) and food processing (1.A.2.e – Food processing, beverages and tobacco) are included (IE) in 1A2fi – TIER 1.

1.A.3.b Road transport

Emissions from fuel combustion in road transportation were estimated using Copert III, taking into account the maximum sulphur content of fuels, based on current legislation. This was done because no studies regarding the characteristics of fuel sold by various distribution companies at national level have been carried out, and usage of year 2005 fuel sulphur content would have resulted in overestimated SOx emissions. Emissions for 2007 have also been recalculated.

The sector includes emissions from passenger cars, light duty vehicles, heavy duty vehicles and  busses, mopeds and motorcycles.

The data on fleet structure, provided by Romanian Auto Registry consist of:

- number of vehicles and technical parameters for each one of the following categories:

- Passenger cars;

- Light-duty vehicles with a capacity under 3,5 t;

- Heavy-duty vehicles with a capacity over 3,5 t;

- Buses and coaches;

- Mopeds and motorcycles with a capacity under 50cm3;

- Motorcycles with a capacity over 50cm3.

- annual number of kilometers per motor vehicle category and driving share: urban, rural and highway;

- average driving speed on motor vehicle and road category;

The number of diesel passenger cars has increased by almost 60% since 2005, while the gasoline fueled ones recorded only a 16% increase.

Annual fuel consumption was provided by the National Institute of Statistics.

Total Diesel fuel sold quantity has increased by 28,5% from 2.121.728 tons in 2005 to 2.967.668 tons in 2008, while gasoline consumption has decreased by almost 6% during the same period. Regarding passenger cars, the number of diesel vehicles has increased by 60%, while the number of gasoline fueled ones has grown only by 16%.

Compared to 2005, there has been a 75% decrease in SOx emissions. Acccording to national legislation, the maximum sulphur content in gasoline was set to 150 mg/kg as of January the 1st,2005 and further decreased to 50 mg/kg starting with the 1st of January 2007. Diesel fuel sulphur content was enforced to decrease from a regulated maximum of 350 mg/kg before January the 1st, 2007 to 50 mg/kg from the 1st of January 2007.

1.A.3.b.v Gasoline evaporation

Emissions from gasoline evaporation were estimated using COPERT IV.

1.A.3.c. - Railways

1.A.3.d. - Navigation

The activities included in these sectors are:

- Motor vehicles and equipments used in the construction activities;

- Motor vehicles and equipments used in other economic branches, including mineral extraction, factory conveyance and hoisting etc;

- Railways;

- National navigation except for fishing, including the naval activities;

The quantity of fuels used in agricultural transports and in national navigation was provided by NIS.

The emission factors are those recommended by the guidebook.

The activities regarding the national fishing were considered as 1.A.4.c.iii and the agriculture/forestry was included in 1.A.4.cii.

1.A.4  Small combustion

Activities from by this chapter cover stationary combustion from the following sectors: 1.A.4.a - Commercial/institutional heating; 1.A.4.b – Residential, cooking;  1.A.4.c -agriculture/forestry. The activities essentially cover combustion in smaller-scale combustion units and installations than those in included 1.A.1.a.

The combustion installations included are considered to have an installed thermal capacity lower than 50 MW.

Data regarding fuel types and consumption were provided by the NIS. Emission factors are those considered in the latest version of the guidebook. District combustion plants and CHP plants were not included in this category.

For agriculture/forestry sector, total fuel sold both for agriculture based activities (plant and animal production and irrigation) and forest activities were submitted by NIS in an aggregated form. In order to avoid double counting, the following assumption was done: diesel and gasoline from energy balance were allocated to this category, while the diesel and heavy oil fuels for naval transport sector at national level, (aggregated by NIS in a specific reporting format, other than energy balance) were allocated to 1.A.3.c. and 1.A.3.d.

1.B. Fugitive Emissions from Fuels

This category comprises fugitive emissions from coal mining and handling, solid fuel transformation and exploration, transport, refining/storage distribution of oil products and gas distribution networks.

The emissions resulted and reported from the activities of this group are mainly methane, non-methane volatile organic compounds.

Activity data are provided by NIS. The emission factors used in the calculation of the emissions from the liquid fuels extraction, treatment and loading are those included in the guidebook.

All resulting emissions from the activities included in 1A, emission factors, activity data and share from national total are presented in table 1, 2, 3 and 4.

For confidential activity data C notation is used .

Table 1 – Share from national total  - ENERGY

| Pollutant | Year | 1A1a | 1A1c | 1A2a | 1A2fi | 1A3bi | 1A3bii | 1A3biii | 1A3c | 1A3dii | 1A4ai | 1A4bi | 1A4ciii | 1B1a | 1B2b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOx | 2007 | 77.10% |  | 15.80% | 3.00% |  |  |  |  |  |  |  |  |  |  |
|  | 2008 | 80.30% |  | 12.00% | 3.80% |  |  |  |  |  |  |  |  |  |  |
| NOx | 2007 | 27.30% |  | 6.80% | 6.80% | 12.30% | 3.80% | 26.60% | 3.10% | 1.90% | 1.60% | 4.90% |  |  |  |
|  | 2008 | 28.30% |  | 5.70% | 6.10% | 12.60% | 4.30% | 20.20% | 3.00% | 2.10% | <1.00% | 5.80% |  |  |  |
| NMVOC | 2007 |  |  |  |  | 12.30% |  |  |  |  |  | 24.20% |  | 6.60% | 3.50% |
|  | 2008 |  |  |  |  | 11.10% |  |  |  |  |  | 30.00% |  | 6.40% | 3.30% |
| CO | 2007 | 2.80% | 4.80% | 6.50% | 2.50% | 29.40% | 4.90% | 1.80% |  |  |  | 41.50% |  |  |  |
|  | 2008 | 2.80% | 3.30% | 4.90% | 3.00% | 22.00% | 3.90% |  |  |  |  | 53.20% |  |  |  |
| TSP | 2007 | 14.50% | 5.00% | 5.90% | 2.50% |  |  |  |  |  |  | 38.40% |  |  |  |
|  | 2008 | 11.50% | 2.80% | 3.60% | 1.70% |  |  |  |  |  |  | 40.20% |  |  |  |
| PM10 | 2007 | 5.10% | 5.00% | 9.10% | 3.60% | 1.70% |  | 3.43% |  |  |  | 60.30% |  |  |  |
|  | 2008 | 4.60% | 3.10% | 6.10% | 2.70% | 1.20% |  | 2.60% |  |  |  | 69.50% |  |  |  |
| PM2.5 | 2007 | 2.90% | 4.60% | 10.10% | 3.90% |  |  |  |  |  |  | 72.60% |  |  |  |
|  | 2008 | 2.50% | 2.80% | 6.50% | 2.80% |  |  |  |  |  |  | 80.60% |  |  |  |
| Pb | 2007 | 5.30% | 3.50% | 21.20% | 2.40% | 5.70% |  |  |  |  |  | 4.70% |  |  |  |
|  | 2008 | 6.40% | 2.90% | 22.30% | 3.70% | 7.40% |  |  |  |  |  | 7.00% |  |  |  |
| Hg | 2007 | 9.90% | 34.70% | 7.20% | <1.00% |  |  |  |  | 37.00% |  |  | 2.60% |  |  |
|  | 2008 | 9.70% | 23.20% | 5.20% | 8.40% |  |  |  |  | 38.00% |  |  | 9.20% |  |  |
| Cd | 2007 | 18.70% | 5.20% | 5.10% | 2.90% |  |  |  |  | 20.70% |  | 6.10% | 10.80% |  |  |
|  | 2008 | 15.10% | 2.90% | 3.30% | 3.60% |  |  |  |  | 17.90% |  | 5.50% | 32.50% |  |  |
| DIOX | 2007 | 2.00% |  | 12.50% | 4.30% |  |  |  |  |  |  | 48.30% |  |  |  |
|  | 2008 | 1.90% |  | 8.80% | 3.50% |  |  |  |  |  |  | 59.10% |  |  |  |
| PAH | 2007 |  |  | 10.60% | 3.30% |  |  |  |  |  |  | 58.10% |  |  |  |
|  | 2008 |  |  | 7.00% | <1.00% |  |  |  |  |  |  | 65.60% |  |  |  |
| HCB | 2007 |  |  | 2.90% | 4.40% |  |  |  |  |  |  | 32.10% |  |  |  |
|  | 2008 |  |  | <1.00% | 7.40% |  |  |  |  |  |  | 44.00% |  |  |  |

Table 2 – EMISSION FACTORS - ENERGY

| Category | Pollutant | Emission Factor | Unit | Fuel Name | Sources (EMEP/EEA Guidebook) |
| --- | --- | --- | --- | --- | --- |
| 1.A.1.a | As | 17 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 4.3 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.09 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 9.5 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 0.09 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.a | Benzo(a) | 0.6 | µg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.6 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 1.6 | µg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 1.12 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | Benzo(b) | 0.8 | µg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.8 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.04 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | Benzo(k) | 0.8 | µg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.8 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.02 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | Cd | 2.1 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 0.5 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.5 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 1.3 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 1.8 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | CO | 39 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 113 | g/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 10 | g/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 258 | g/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 5 | g/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
| 1.A.1.a | Cr | 11 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 2.7 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.7 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 9 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 0.7 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.a | Cu | 5.7 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.4 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 21 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 0.4 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.3 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
| 1.A.1.a | DIOX | 2.5 | ng/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.5 | ng/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 50 | ng/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 0.5 | ng/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 10 | ng/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
| 1.A.1.a | HCB | 6 | µg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | Hg | 3.5 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 0.1 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.1 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.4 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 1.5 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | Indeno | 6.9 | µg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.8 | µg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 2.6 | µg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 0.8 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.37 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | NH3 | 0.9 | g/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.a | Ni | 273 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 1 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 14 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 1 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 12 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
| 1.A.1.a | NMVOC | 1.5 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 2.5 | g/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 1.7 | g/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 7.3 | g/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 0.8 | g/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
| 1.A.1.a | Pb | 18 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 0.2 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.2 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 21 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 4.9 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
| 1.A.1.a | PCBs | 60 | µg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
| 1.A.1.a | PM10 | 20 | g/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 0.9 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 2.5 | g/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 51 | g/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 18 | g/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
| 1.A.1.a | PM25 | 9 | g/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 0.9 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 2.5 | g/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 33 | g/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 13 | g/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
| 1.A.1.a | Se | 2.2 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.01 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 1.2 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 0.01 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 55 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
| 1.A.1.a | TSP | 0.9 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 30 | g/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 5 | g/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 51 | g/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 25 | g/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
| 1.A.1.a | Zn | 94 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 14 | mg/GJ | Gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 4.5 | mg/GJ | Solid fuels | 1.A.1.a - Table 3-4 Tier 1 emission factors for source category 1.A.1.a using brown coal |
|  |  | 181 | mg/GJ | wood and similar wood wastes | 1.A.1.a - Table 3-9 Tier 1 emission factors for source category 1.A.1.a using biomass |
|  |  | 14 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.a - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | As | 4.3 | mg/GJ | Liquid fuels | 1.A.1.a - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.09 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.09 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Benzo(a) | 0.6 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.6 | µg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Benzo(b) | 0.8 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.8 | µg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Benzo(k) | 0.8 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.8 | µg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Cd | 1.3 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.5 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.5 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | CO | 5 | g/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 10 | g/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 39 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | Cr | 2.7 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.7 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.7 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Cu | 5.7 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.4 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.4 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | DIOX | 2.5 | ng/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.5 | ng/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.5 | ng/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Hg | 0.4 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.1 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.1 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Indeno | 6.9 | µg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.8 | µg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 0.8 | µg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Ni | 273 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 1 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 1 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | NMVOC | 0.8 | g/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 1.5 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 2.5 | g/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.b | Pb | 4.9 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.2 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.2 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | PM10 | 18 | g/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 5 | g/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.9 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | PM25 | 13 | g/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 5 | g/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.9 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | Se | 2.2 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 0.01 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.01 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | TSP | 25 | g/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 5 | g/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
|  |  | 0.9 | g/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
| 1.A.1.b | Zn | 94 | mg/GJ | Liquid fuels | 1.A.1.b - Table 3-7 Tier 1 emission factors for source category 1.A.1.a using heavy fuel oil |
|  |  | 14 | mg/GJ | natural gas (except liquefied natural gas) | 1.A.1.b - Table 3-5 Tier 1 emission factors for source category 1.A.1.a using natural gas |
|  |  | 14 | mg/GJ | other gaseous fuels | 1.A.1.a - Table 3-6 Tier 1 emission factors for source category 1.A.1.a using derived gases |
| 1.A.1.c | As | 11.24 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Benzo(a) | 0.64 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Benzo(b) | 0.31 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Benzo(k) | 0.26 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Cd | 1.56 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | CO | 525 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Cr | 5.72 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Cu | 24.4 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | DIOX | 8.9 | ng/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Hg | 29.3 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Indeno | 0.18 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | NH3 | 1.3 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Ni | 5.42 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | NMVOC | 2.4 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | NOX | 22 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Pb | 28.2 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | PM10 | 49 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | PM25 | 38 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Se | 2.87 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | SO2 | 55 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | TSP | 81 | g/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.1.c | Zn | 46.1 | mg/GJ | Solid fuels | 1.A.1.c - Table 5-2 Tier 1 emission factors for source category 1.A.1.c |
| 1.A.2.a | As | 0.09 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 1.4 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 4 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 1 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | Benzo(a) | 0.6 | µg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 45.5 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 44.6 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 5.2 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | Benzo(b) | 0.8 | µg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 58.9 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 64.9 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 6.2 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | Benzo(k) | 23.7 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 23.4 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 0.8 | µg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 4 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | Cd | 1.8 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 0.5 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 1.8 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.3 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | CO | 931 | g/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 40 | g/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 25 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 1596 | g/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | Cr | 0.7 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 13.5 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 12.8 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 6.5 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | Cu | 0.4 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 4.6 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 17.5 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 7.2 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | DIOX | 2 | ng/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 203 | ng/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 326 | ng/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 10 | ng/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | HCB | 0.62 | µg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 6 | µg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | Hg | 0.2 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 0.7 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 7.9 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.1 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | Indeno | 0.8 | µg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 2.2 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 18.5 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 22.3 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | Ni | 1 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 2 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 13 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 260 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | NMVOC | 10 | g/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 88.8 | g/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 2.5 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 146.4 | g/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | NOX | 150 | g/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 173 | g/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 100 | g/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 70 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
| 1.A.2.a | Pb | 0.2 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 24.8 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 134 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 16 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | PCBs | 170 | µg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.06 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | PM10 | 0.5 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 117 | g/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 149.9 | g/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 21.5 | g/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | PM25 | 0.5 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 149.1 | g/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 108 | g/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 16.5 | g/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.a | Se | 0.5 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 0.01 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 1.8 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
| 1.A.2.a | SO2 | 140 | g/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 900 | g/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 38.4 | g/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | TSP | 27.5 | g/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 124 | g/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 156.4 | g/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.a | Zn | 14 | mg/GJ | Gaseous fuels | 1.A.2.a - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 113.6 | mg/GJ | wood and similar wood wastes | 1.A.2.a - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 200 | mg/GJ | Solid fuels | 1.A.2.a - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 8 | mg/GJ | Liquid fuels | 1.A.2.a - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.b | CO | C | g/Mg product | no fuel | 1.A.2.b - Table 3-21 Tier 2 emission factors for source category 1.A.2.b, Alumina production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-13 Tier 2 emission factors for source category 1.A.2.b, Secondary copper production |
| 1.A.2.b | NOX | C | g/Mg product | no fuel | 1.A.2.b - Table 3-15 Tier 2 emission factors for source category 1.A.2.b, Secondary lead production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-13 Tier 2 emission factors for source category 1.A.2.b, Secondary copper production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-18 Tier 2 emission factors for source category 1.A.2.b, Secondary aluminium production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-21 Tier 2 emission factors for source category 1.A.2.b, Alumina production |
| 1.A.2.b | SO2 | C | g/Mg product | no fuel | 1.A.2.b - Table 3-14 Tier 2 emission factors for source category 1.A.2.b, Primary lead production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-15 Tier 2 emission factors for source category 1.A.2.b, Secondary lead production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-16 Tier 2 emission factors for source category 1.A.2.b, Primary zinc production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-13 Tier 2 emission factors for source category 1.A.2.b, Secondary copper production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-18 Tier 2 emission factors for source category 1.A.2.b, Secondary aluminium production |
|  |  | C | g/Mg product | no fuel | 1.A.2.b - Table 3-21 Tier 2 emission factors for source category 1.A.2.b, Alumina production |
| 1.A.2.f.i | As | 0.0265 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 1.4 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 4 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 1 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 0.09 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
| 1.A.2.f.i | Benzo(a) | 0.6 | µg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 0.000065 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 44.6 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 45.5 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 5.2 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.f.i | Benzo(b) | 0.8 | µg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 0.00028 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 64.9 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 58.9 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 6.2 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.f.i | Benzo(k) | 0.000077 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 23.4 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 23.7 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.8 | µg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 4 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.f.i | Cd | 0.008 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 1.8 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 1.8 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.3 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 0.5 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
| 1.A.2.f.i | CO | 1596 | g/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 931 | g/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 40 | g/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 25 | g/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 525 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-27 Tier 2 emission factors for source category 1.A.2.f.i, Mineral wool production |
|  |  | C | g/Mg product | no fuel | 1.A.2.f.i - Table 3-25 Tier 2 emission factors for source category 1.A.2.f.i, Roadstone coating (asphalt) |
|  |  | 415 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-28 Tier 2 emission factors for source category 1.A.2.f.i, Bricks and tiles |
|  |  | 525 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-27 Tier 2 emission factors for source category 1.A.2.f.i, Mineral wool production |
|  |  | 2000 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
| 1.A.2.f.i | Cr | 0.041 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 6.5 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 13.5 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 12.8 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 0.7 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
| 1.A.2.f.i | Cu | 0.0647 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 0.4 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 4.6 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 17.5 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 7.2 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.f.i | DIOX | 0.05 | µg/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 2 | ng/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 326 | ng/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 203 | ng/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 10 | ng/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.f.i | HCB | 6 | µg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 0.62 | µg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 11 | µg/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
| 1.A.2.f.i | Hg | 0.11 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 0.7 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 7.9 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.1 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 0.2 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
| 1.A.2.f.i | Indeno | 0.8 | µg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 0.000043 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 18.5 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 22.3 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 2.2 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.f.i | NH3 | 1400 | g/Mg product | no fuel | 1.A.2.f.i -Table 3.5 Tier 2 emission factors for source category 2.A.7.d Other mineral production, glass production, glass wool |
| 1.A.2.f.i | Ni | 0.049 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 1 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 2 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 13 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 260 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.2.f.i | NMVOC | 10 | g/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 146.4 | g/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 88.8 | g/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 2.5 | g/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 500 | g/Mg product | no fuel | 1.A.2.f.i -Table 3.5 Tier 2 emission factors for source category 2.A.7.d Other mineral production, glass production, glass wool |
|  |  | 100 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
| 1.A.2.f.i | NOX | 100 | g/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 150 | g/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 173 | g/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 70 | g/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 1625 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-27 Tier 2 emission factors for source category 1.A.2.f.i, Mineral wool production |
|  |  | 1625 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-27 Tier 2 emission factors for source category 1.A.2.f.i, Mineral wool production |
|  |  | 142 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-28 Tier 2 emission factors for source category 1.A.2.f.i, Bricks and tiles |
|  |  | C | g/Mg product | no fuel | 1.A.2.f.i - Table 3-25 Tier 2 emission factors for source category 1.A.2.f.i, Roadstone coating (asphalt) |
|  |  | 1550 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
| 1.A.2.f.i | Pb | 0.098 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 24.8 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 134 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 16 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 0.2 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
| 1.A.2.f.i | PCBs | 170 | µg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 1 | µg/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 0.06 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
| 1.A.2.f.i | PM10 | 149.9 | g/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 117 | g/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 21.5 | g/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 590 | g/Mg product | no fuel | 1.A.2.f.i -Table 3.5 Tier 2 emission factors for source category 2.A.7.d Other mineral production, glass production, glass wool |
| 1.A.2.f.i | PM25 | 0.5 | g/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 149.1 | g/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 108 | g/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 16.5 | g/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 520 | g/Mg product | no fuel | 1.A.2.f.i -Table 3.5 Tier 2 emission factors for source category 2.A.7.d Other mineral production, glass production, glass wool |
| 1.A.2.f.i | Se | 0.0253 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 0.01 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 0.5 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 1.8 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
| 1.A.2.f.i | SO2 | 140 | g/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 38.4 | g/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 900 | g/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 223 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-27 Tier 2 emission factors for source category 1.A.2.f.i, Mineral wool production |
|  |  | 223 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-27 Tier 2 emission factors for source category 1.A.2.f.i, Mineral wool production |
|  |  | C | g/Mg product | no fuel | 1.A.2.f.i - Table 3-25 Tier 2 emission factors for source category 1.A.2.f.i, Roadstone coating (asphalt) |
|  |  | 166 | g/Mg product | no fuel | 1.A.2.f.i - Table 3-28 Tier 2 emission factors for source category 1.A.2.f.i, Bricks and tiles |
|  |  | 374 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
| 1.A.2.f.i | TSP | 156.4 | g/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 27.5 | g/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 124 | g/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 670 | g/Mg product | no fuel | 1.A.2.f.i -Table 3.5 Tier 2 emission factors for source category 2.A.7.d Other mineral production, glass production, glass wool |
| 1.A.2.f.i | Zn | 0.424 | g/t | no fuel | 1.A.2.f.i - Table 3-24 Tier 2 emission factors for source category 1.A.2.f.i, Cement production |
|  |  | 14 | mg/GJ | Gaseous fuels | 1.A.2.f.i - Table 3-3 Tier 1 emission factors for 1.A.2 combustion in industry using natural gas or derived gases |
|  |  | 113.6 | mg/GJ | wood and similar wood wastes | 1.A.2.f.i - Table 3-5 Tier 1 emission factors for 1.A.2 combustion in industry using biomass |
|  |  | 200 | mg/GJ | Solid fuels | 1.A.2.f.i - Table 3-2 Tier 1 emission factors for 1.A.2 combustion in industry using hard or brown coal |
|  |  | 8 | mg/GJ | Liquid fuels | 1.A.2.f.i - Table 3-4 Tier 1 emission factors for 1.A.2 combustion in industry using liquid fuels |
| 1.A.3.c | Benzo(a) | 0.03 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | Benzo(b) | 0.05 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | Cd | 0.01 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | CO | 10.7 | kg/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | Cr | 0.05 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | Cu | 1.7 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | NH3 | 0.007 | kg/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | Ni | 0.07 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | NMVOC | 4.65 | kg/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | NOX | 52.4 | kg/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | PM10 | 1.44 | kg/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | PM25 | 1.37 | kg/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | Se | 0.01 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | TSP | 1.52 | kg/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.c | Zn | 1 | g/t | diesel oil for road transport | 1.A.3.c -Table 3-1 Tier 1 emission factors for railways |
| 1.A.3.d.ii | As | 0.05 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Benzo(a) | 0 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Benzo(b) | 0.02 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Benzo(k) | 0 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Cd | 0.01 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | CO | 7.4 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Cr | 0.04 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Cu | 0.05 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Hg | 0.05 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Indeno | 0.01 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Ni | 0.07 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | NMVOC | 2.4 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | NOX | 72 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Pb | 0.1 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | PM10 | 1.1 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | PM25 | 1.1 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Se | 0.2 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | SO2 | 2 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | TSP | 1.1 | kg/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.3.d.ii | Zn | 0.5 | g/t | diesel oil for road transport | 1.A.3.d.ii - Table 3-2 Tier 1 emission factors for medium to large ships using marine diesel oil |
| 1.A.4.a.i | As | 1.4 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 1 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.0937 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 4 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.a.i | Benzo(a) | 0.562 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 44.6 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 5.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 45.5 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.a.i | Benzo(b) | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 64.9 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 58.9 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 6.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Benzo(k) | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 23.4 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 23.7 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 4 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Cd | 1.8 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 1.8 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.515 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 0.3 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | CO | 1600 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 931 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 25 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 40 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Cr | 6.5 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 0.656 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 13.5 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 12.8 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Cu | 4.6 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 7.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.398 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 17.5 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.a.i | DIOX | 326 | ng/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 2 | ng/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 10 | ng/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 203 | ng/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.a.i | HCB | 0.62 | µg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 6 | µg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.a.i | Hg | 0.7 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 0.234 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 7.9 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.1 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Indeno | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 22.3 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 18.5 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 2.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Ni | 2 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 0.984 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 260 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 13 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.a.i | NMVOC | 146 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 88.8 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 2.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 10 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | NOX | 150 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 173 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 70 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 100 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Pb | 24.8 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 134 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.984 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 16 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | PCBs | 170 | µg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.06 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.a.i | PM10 | 150 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 117 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 21.5 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | PM25 | 149 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 108 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 16.5 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Se | 0.5 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 0.0112 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 1.8 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.a.i | SO2 | 38.4 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 900 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 140 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | TSP | 156 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 124 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 27.5 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.a.i | Zn | 114 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 13.6 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 200 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 8 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.b.i | As | 2.5 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 0.9 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.0937 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 1 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Benzo(a) | 0.562 | µg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 230 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 22 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 210 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Benzo(b) | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 330 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 25.7 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 220 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Benzo(k) | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 130 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 12.5 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 130 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Cd | 1.5 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 1.5 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.515 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 1.4 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | CO | 4600 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 31 | g/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 46 | g/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 5300 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Cr | 11.2 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 15.5 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.656 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 2.9 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Cu | 22.3 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 7.9 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.398 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 8.6 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | DIOX | 800 | ng/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 10 | ng/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.5 | ng/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 700 | ng/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | HCB | 0.62 | µg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 6 | µg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Hg | 5.1 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 0.03 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.234 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 0.5 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Indeno | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 110 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 140 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
|  |  | 14.8 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
| 1.A.4.b.i | NH3 | 0.3 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 3.8 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Ni | 12.7 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 240 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.984 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 4.4 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | NMVOC | 484 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 15.5 | g/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 10.5 | g/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 925 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | NOX | 110 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 68 | g/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 57 | g/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 74.5 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Pb | 130 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 15.5 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.984 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 40 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | PCBs | 170 | µg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 0.06 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | PM10 | 404 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 3.7 | g/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 695 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | PM25 | 398 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 3.7 | g/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 695 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Se | 1 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 0.0112 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 0.5 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | SO2 | 20 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
|  |  | 900 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 140 | g/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
| 1.A.4.b.i | TSP | 444 | g/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 6 | g/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 730 | g/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.b.i | Zn | 220 | mg/GJ | Solid fuels | 1.A.4.b.i - Table 3-3 Tier 1 emission factors for NFR source category 1.A.4.b, using hard coal and brown coal |
|  |  | 8.5 | mg/GJ | Liquid fuels | 1.A.4.b - Table 3-5 Tier 1 emission factors for NFR source category 1.A.4.b, using other liquid fuels |
|  |  | 13.6 | mg/GJ | Gaseous fuels | 1.A.4.b.i - Table 3-4 Tier 1 emission factors for NFR source category 1.A.4.b, using natural gas |
|  |  | 130 | mg/GJ | wood and similar wood wastes | 1.A.4.b - Table 3-6 Tier 1 emission factors for NFR source category 1.A.4.b, using biomass |
| 1.A.4.c.i | As | 4 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 1 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.0937 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 1.4 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | Benzo(a) | 0.562 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 5.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 44.6 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 45.5 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | Benzo(b) | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 6.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 58.9 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 64.9 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | Benzo(k) | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 4 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 23.4 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 23.7 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | Cd | 0.3 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.515 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 1.8 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 1.8 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | CO | 40 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 25 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 1600 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 931 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | Cr | 13.5 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.656 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 6.5 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 12.8 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
| 1.A.4.c.i | Cu | 17.5 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 7.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.398 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 4.6 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | DIOX | 2 | ng/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 10 | ng/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 326 | ng/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 203 | ng/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | HCB | 6 | µg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 0.62 | µg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | Hg | 0.1 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 7.9 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.234 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 0.7 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | Indeno | 0.843 | µg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 2.2 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 18.2 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 22.3 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | Ni | 13 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 260 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.984 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 2 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | NMVOC | 2.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 88.8 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 10 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 146 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | NOX | 70 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 173 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 100 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 150 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | Pb | 16 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.984 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 24.8 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 134 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | PCBs | 170 | µg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.06 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | PM10 | 21.5 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 150 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 117 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | PM25 | 16.5 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 149 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 108 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | Se | 1.8 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.0112 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 0.5 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | SO2 | 900 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 140 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 38.4 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.i | TSP | 27.5 | g/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 0.5 | g/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 156 | g/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
|  |  | 124 | g/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
| 1.A.4.c.i | Zn | 200 | mg/GJ | Solid fuels | 1.A.4.a/c - Table 3-7 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using hard and brown coal |
|  |  | 8 | mg/GJ | Liquid fuels | 1.A.4.a/c - Table 3-9 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using liquid fuels |
|  |  | 13.6 | mg/GJ | Gaseous fuels | 1.A.4.a/c - Table 3-8 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using gaseous fuels |
|  |  | 114 | mg/GJ | wood and similar wood wastes | 1.A.4.a.i - Table 3-10 Tier 1 emission factors for NFR source category 1.A.4.a/c, 1.A.5.a, using biomass |
| 1.A.4.c.ii | CO | 620793 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery - 4 stroke |
| 1.A.4.c.ii | NH3 | 8 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery |
|  |  | 3 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery - 4 stroke |
| 1.A.4.c.ii | NMVOC | 242197 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery - 4 stroke |
|  |  | 3366 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery |
| 1.A.4.c.ii | NOX | 35043 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery |
|  |  | 2765 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery - 4 stroke |
| 1.A.4.c.ii | PM10 | 3762 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery - 4 stroke |
| 1.A.4.c.ii | PM25 | 3762 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery - 4 stroke |
| 1.A.4.c.ii | SO2 | 0 | kg/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery |
| 1.A.4.c.ii | TSP | 3762 | g/t | Liquid fuels | 1.A.4.c.ii - Table 3-1 Tier 1 emission factors for off-road machinery - 4 stroke |
| 1.A.4.c.iii | As | 0.5 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Benzo(a) | 0 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Benzo(b) | 0.02 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Benzo(k) | 0 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Cd | 0.03 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | CO | 7.4 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Cr | 0.2 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Cu | 0.5 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | DIOX | 0 | mg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | HCB | 0 | mg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Hg | 0.02 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Indeno | 0.01 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Ni | 30 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | NMVOC | 2.4 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | NOX | 72 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Pb | 0.2 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | PM10 | 6.7 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | PM25 | 6.7 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Se | 0.4 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | SO2 | 30 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | TSP | 6.7 | kg/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.A.4.c.iii | Zn | 0.9 | g/t | Liquid fuels | 1.A.4.c.iii - Table 3-1 Tier 1 emission factors for medium to large ships using bunker fuel oil |
| 1.B.1.a. | PM10 | 3 | g/Mg coal | no fuel | 1.B.1.a. - Table 3-1 Tier 1 emission factors for source category 1.B.1.a Coal mining and handling |
| 1.B.1.a. | NMVOC | 0.8 | kg/Mg coal | no fuel | 1.B.1.a. - Table 3-1 Tier 1 emission factors for source category 1.B.1.a Coal mining and handling |
| 1.B.1.b | As | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Benzo(a) | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Benzo(b) | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Benzo(k) | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Cd | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Hg | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Indeno | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | NH3 | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Ni | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | NMVOC | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | PAH | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | Pb | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | PM10 | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | PM25 | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.1.b | TSP | C | g/Mg product | no fuel | 1.B.1.b. - Table 3-1 Tier 1 emission factors for source category 1.B.1.b Solid fuel transformation |
| 1.B.2.a.i | NMVOC | 0.2 | kg/t | no fuel | 1.B.1.a.i. - Table 3-1 Tier 1 emission factors for source category 1.B.2.a.i Exploration, production, transport |
| 1.B.2.a.iv | NMVOC | 0.2 | kg/Mg product | crude oil | 1.B.1.a.iv. - Table 3-1 Tier 1 emission factors for source category 1.B.2.a.iv Refining, storage |
| 1.B.2.a.iv | PAH | 9.6 | mg/Mg product | crude oil | 1.B.1.a.iv. - Table 3-1 Tier 1 emission factors for source category 1.B.2.a.iv Refining, storage |
| 1.B.2.a.v | NMVOC | 2 | kg/Mg product | no fuel | 1.B.2.a.v -Table 3-1 Tier 1 emission factor for source category 1.B.2.a.v Distribution of oil products |
| 1.B.2.b | NMVOC | 100 | g/1000 m3 | no fuel | 1.B.2.b - Table 3-5 Tier 2 emission factors for source category 1.B.2.b Natural gas, Onshore facilities |
|  |  | 880 | g/1000 m3 | no fuel | 1.B.2.b - Table 3-16 Emission factors for gas distribution (Corinair 1990) |

Table 3 – ACTIVITY DATA - ENERGY

| Category | FUEL NAME | 2007 | 2008 | Unit |
| --- | --- | --- | --- | --- |
| 1.A.1.a | Liquid fuels | 12947063.64 | 9805610.97 | GJ |
| 1.A.1.a | natural gas (except liquefied natural gas) | 150825104 | 132071422.9 | GJ |
| 1.A.1.a | Solid fuels | 309834775.4 | 313911371 | GJ |
| 1.A.1.a | wood and similar wood wastes | 60315 | 0 | GJ |
| 1.A.1.a | Gaseous fuels | 2303208 | 1168619 | GJ |
| 1.A.1.b | other gaseous fuels | 4306745 | 3297935 | GJ |
| 1.A.1.b | Liquid fuels | 2078205 | 2455270 | GJ |
| 1.A.1.b | natural gas (except liquefied natural gas) | 2770767 | 3505094.232 | GJ |
| 1.A.1.c | Solid fuels | 131569364 | 91088484 | GJ |
| 1.A.2.a | wood and similar wood wastes | 5801 | 88 | GJ |
| 1.A.2.a | Gaseous fuels | 45417213 | 50175153 | GJ |
| 1.A.2.a | Solid fuels | 100106470 | 74422881 | GJ |
| 1.A.2.a | Liquid fuels | 4410970 | 4082720 | GJ |
| 1.A.2.b | Activity data from production | C | C | Mg product |
| 1.A.2.f.1 | Activity data from production | Not available | 7780028 | t |
| 1.A.2.f.1 | Activity data from production | C | C | Mg product |
| 1.A.2.f.1 | Solid fuels | 6489274 | 9336295 | GJ |
| 1.A.2.f.1 | wood and similar wood wastes | 14762894 | 8830875 | GJ |
| 1.A.2.f.1 | Liquid fuels | 77570259 | 68419438 | GJ |
| 1.A.2.f.1 | Gaseous fuels | 111316326 | 122525304 | GJ |
| 1.A.3.b. | Road transport | 289059743 | 266686503 | GJ |
| 1.A.3.c | diesel oil for road transport | 180192 | 167552 | t |
| 1.A.3.d.2 | diesel oil for road transport | 82261 | 87582 | t |
| 1.A.4.a.1 | wood and similar wood wastes | 6032196 | 5599670 | GJ |
| 1.A.4.a.1 | Solid fuels | 66453 | 15881 | GJ |
| 1.A.4.a.1 | Gaseous fuels | 45696573 | 34170012 | GJ |
| 1.A.4.a.1 | Liquid fuels | 9236211 | 3668682 | GJ |
| 1.A.4.b.1 | Gaseous fuels | 87432226 | 92556796 | GJ |
| 1.A.4.b.1 | Solid fuels | 435837 | 1973091 | GJ |
| 1.A.4.b.1 | wood and similar wood wastes | 112254072 | 143311076 | GJ |
| 1.A.4.b.1 | Liquid fuels | 25742482 | 12721356 | GJ |
| 1.A.4.c.1 | Solid fuels | 3194 | 1085 | GJ |
| 1.A.4.c.1 | wood and similar wood wastes | 1563566 | 561037 | GJ |
| 1.A.4.c.1 | Gaseous fuels | 1063121 | 2013379 | GJ |
| 1.A.4.c.1 | Liquid fuels | 182978 | 1752668 | GJ |
| 1.A.4.c.2 | Liquid fuels | 120920 | 111192 | t |
| 1.A.4.c.3 | Liquid fuels | 14342 | 53051 | t |
| 1.B.1.a | no fuel | 35779285 | 35860994 | Mg coal |
| 1.B.1.b | no fuel | C | C | Mg product |
| 1.B.2.a.1 | no fuel | 13278924 | 13138445 | t |
| 1.B.2.a.4 | crude oil | 13171653 | 13100469 | Mg product |
| 1.B.2.a.5 | no fuel | 4488723 | 4308874 | Mg product |
| 1.B.2.b | no fuel | 27606766 | 26916411 | 1000 m3 |

Table 4 – EMISSION DATA FROM ENERGY SECTOR

| NFR | Main Pollutants |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nox (as NO2) |  | NMVOC |  | SOx (as SO2) |  | NH3 |  |
|  | Gg NO2 |  | Gg |  | Gg SO2 |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 1 A 1 a Public electricity and heat production | 83.445 | 84.357 | 0.74252 | 0.769 | 451.098 | 443.389 | 0.00105 | 0.00207 |
| 1 A 1 b Petroleum refining | 1.293 | 1.427 | 0.015 | 0.016 | 2.033 | 2.396 | NA | NA |
| 1 A 1 c Manufacture of solid fuels and other energy industries | 2.004 | 2.894 | 0.219 | 0.316 | 5.00987 | 7.236 | 0.118 | 0.171 |
| 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 16.796 | 20.939 | 6.775 | 9.047 | 67.577 | 90.736 | NA | NA |
| 1 A 2 b Stationary Combustion in manufacturing industries and construction: Non-ferrous metals | 0.146 | 0.166 | NA | NA | 0.711 | 0.711 | NA | NA |
| 1 A 2 c Stationary combustion in manufacturing industries and construction: Chemicals | IE | IE | IE | IE | IE | IE | NA | IE |
| 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | IE | IE | IE | IE | IE | IE | NA | IE |
| 1 A 2 e Stationary combustion in manufacturing industries and construction: Food processing, beverages and tobacco | IE | IE | IE | IE | IE | IE | NA | IE |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other | 30.4921 | 18.949 | 3.8904 | 3.791 | 21.3204 | 17.35 | 0 | 0.0001 |
| 1 A 2 f ii Mobile Combustion in manufacturing industries and construction | IE | IE | IE | IE | IE | IE | IE | IE |
| 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | NE | NE | NE | NE | NE | NE | NE | NE |
| 1 A 3 a i (i) International aviation (LTO) | NE | NE | NE | NE | NE | NE | NE | NE |
| 1 A 3 b i Road transport: Passenger cars | 37.275 | 38.002 | 49.872 | 53.612 | 0.206 | 0.166 | 1.487 | 1.187 |
| 1 A 3 b ii Road transport:Light duty vehicles | 12.686 | 11.783 | 6.975 | 7.734 | 0.061 | 0.044 | 0.098 | 0.091 |
| 1 A 3 b iii Road transport:, Heavy duty vehicles | 59.718 | 82.175 | 10.54 | 10.017 | 0.174 | 0.173 | 0.031 | 0.027 |
| 1 A 3 b iv Road transport: Mopeds & motorcycles | 0.082 | 0.057 | 1.187 | 0.906 | 0.001 | 0.001 | 0.001 | 0.001 |
| 1 A 3 b v Road transport: Gasoline evaporation | NA | NA | 1.05 | 0.9 | NA | NA | NA | NA |
| 1 A 3 b vi Road transport: Automobile tyre and brake wear | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 b vii Road transport: Automobile road abrasion | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 c Railways | 8.78 | 9.442 | 0.78 | 0.838 | NE | NE | 0.00117 | 0.0013 |
| 1 A 3 d i (ii) International inland waterways | NE | NE | NE | NE | NE | NE | NE | NE |
| 1 A 3 d ii National navigation (Shipping) | 6.306 | 5.923 | 0.21 | 0.197 | 0.175 | 0.164 | NA | 0 |
| 1 A 3 e Pipeline compressors | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 a i Commercial / institutional: Stationary | 3.601 | 5.039 | 0.941 | 1.093 | 0.76 | 1.607 | NA | NA |
| 1 A 4 a ii Commercial / institutional: Mobile | IE | IE | IE | IE | IE | IE | IE | IE |
| 1 A 4 b i Residential: Stationary plants | 17.035 | 15.145 | 134.687 | 105.363 | 6.469 | 6.284 | 0.545 | 0.427 |
| 1 A 4 b ii Residential: Household and gardening (mobile) | IE | IE | IE | IE | IE | IE | IE | IE |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 0.401 | 0.328 | 0.105 | 0.233 | 0.269 | 0.089 | NA | NA |
| 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 3.729 | 4.057 | 1.614 | 1.741 | 0 | 0 | 0.0008 | 0.0009 |
| 1A 4 c iii Agriculture/Forestry/Fishing: National fishing | 3.82 | 1.032 | 0.127 | 0.034 | 1.592 | 0.43 | NA | NA |
| 1 B 1 a Fugitive emission from solid fuels: Coal mining and handling | NA | NA | 28.689 | 28.6234 | NE | NE | NA | NA |
| 1 B 1 b Fugitive emission from solid fuels: Solid fuel transformation | NA | NA | 0.018 | 0.026 | NA | NA | 0.005 | 0.008 |
| 1 B 1 c Other fugitive emissions from solid fuels | NE | NE | NE | NE | NE | NE | NE | NE |
| 1 B 2 a i Exploration, production, transport | NA | NA | 2.628 | 2.656 | NA | NA | NA | NA |
| 1 B 2 a iv Refining / storage | NA | NA | 2.62 | 2.634 | NA | NA | NA | NA |
| 1 B 2 a v Distribution of oil products | NA | NA | 8.618 | 8.977 | NA | NA | NA | NA |
| 1 B 2 b Natural gas | NA | NA | 14.917 | 15.419 | NA | NA | NA | NA |
| 1 B 2 c Venting and flaring | NE | NE | NE | NE | NE | NE | NE | NE |

| NFR | PM2.5 |  | PM10 |  | TSP |  | CO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gg |  | Gg |  | Gg |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 1 A 1 a Public electricity and heat production | 3.074461 | 3.1 | 6.576514 | 6.5743 | 30.21747 | 31.0321 | 40.68348 | 40.997 |
| 1 A 1 b Petroleum refining | 0.051563 | 0.051 | 0.063839 | 0.0614 | 0.202819 | 0.209 | 0.181954 | 0.161 |
| 1 A 1 c Manufacture of solid fuels and other energy industries | 3.461362 | 5 | 4.463336 | 6.4468 | 7.378167 | 10.657 | 47.82145 | 69.074 |
| 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 8.130137 | 10.908 | 8.820356 | 11.8308 | 9.365813 | 12.558 | 70.70553 | 94.52 |
| 1 A 2 b Stationary Combustion in manufacturing industries and construction: Non-ferrous metals | NA | NA | NA | NA | NA | NA | 0.062485 | 0.089 |
| 1 A 2 c Stationary combustion in manufacturing industries and construction: Chemicals | NA | NA | NA | NA | NA | NA | IE | NA |
| 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | NA | NA | NA | NA | NA | NA | IE | NA |
| 1 A 2 e Stationary combustion in manufacturing industries and construction: Food processing, beverages and tobacco | NA | NA | NA | NA | NA | NA | IE | NA |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other | 3.515211 | 4.238 | 3.948402 | 4.696 | 4.481677 | 5.302 | 44.25474 | 35.58 |
| 1 A 2 f ii Mobile Combustion in manufacturing industries and construction | IE | NA | IE | NA | IE | NA | IE | NA |
| 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 a i (i) International aviation (LTO) | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 b i Road transport: Passenger cars | NE | NE | 1.687 | 2.235 | NE | NE | 319.085 | 425.134 |
| 1 A 3 b ii Road transport: Light duty vehicles | NE | NE | 0.904 | 1.214 | NE | NE | 56.314 | 70.781 |
| 1 A 3 b iii Road transport:, Heavy duty vehicles | NE | NE | 3.787 | 4.351 | NE | NE | 23.285 | 25.555 |
| 1 A 3 b iv Road transport: Mopeds & motorcycles | NE | NE | 0 | 0 | NE | NE | 7.557 | 5.525 |
| 1 A 3 b v Road transport: Gasoline evaporation | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 b vi Road transport: Automobile tyre and brake wear | NE | NE | NE | NE | NE | NE | NE | NE |
| 1 A 3 b vii Road transport: Automobile road abrasion | NE | NE | NE | NE | NE | NE | NA | NA |
| 1 A 3 c Railways | 0.229546 | 0.247 | 0.241275 | 0.2594 | 0.254679 | 0.274 | 1.792806 | 1.928 |
| 1 A 3 d i (ii) International inland waterways | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 d ii National navigation (Shipping) | 0.09634 | 0.09 | 0.09634 | 0.0904 | 0.963402 | 0.09 | 0.648107 | 0.609 |
| 1 A 3 e Pipeline compressors | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 a i Commercial / institutional: Stationary | 0.913684 | 1.081 | 0.93777 | 1.134 | 0.993492 | 1.226 | 9.975255 | 11.225 |
| 1 A 4 a ii Commercial / institutional: Mobile | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 b i Residential: Stationary plants | 100.4798 | 78.33 | 100.4917 | 78.3316 | 105.6157 | 82.337 | 772.0794 | 600.846 |
| 1 A 4 b ii Residential: Household and gardening (mobile) | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 0.113637 | 0.237 | 0.122972 | 0.2393 | 0.136861 | 0.25 | 1.019111 | 2.538 |
| 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | 0.019535 | 0.021 | 0.019535 | 0.021 | 0.019535 | 0.021 | 3.223571 | 3.468 |
| 1A 4 c iii Agriculture/Forestry/Fishing: National fishing | 0.355442 | 0.096 | 0.355442 | 0.096 | 0.355442 | 0.096 | 0.392577 | 0.106 |
| 1 B 1 a Fugitive emission from solid fuels: Coal mining and handling | NA | NA | 0.108 | 0.107 | NA | NA | NA | NA |
| 1 B 1 b Fugitive emission from solid fuels: Solid fuel transformation | 0.09729 | 0.14013 | 0.1081 | 0.1557 | 0.11891 | 0.17127 | NA | NA |
| 1 B 1 c Other fugitive emissions from solid fuels | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a i Exploration, production, transport | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a iv Refining / storage | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a v Distribution of oil products | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 b Natural gas | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 c Venting and flaring | NA | NA | NA | NA | NA | NA | NE | NA |

| NFR | Pb |  | Cd |  | Hg |  | As |  | Cr |  | Cu |  | Ni |  | Se |  | Zn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 1 A 1 a Public electricity and heat production | 5.725 | 5.672 | 0.739 | 0.744 | 1.116 | 1.105 | 5.391 | 5.337 | 3.573 | 3.551 | 0.203 | 0.230 | 6.577 | 7.407 | 17.288 | 17.071 | 4.200 | 4.766 |
| 1 A 1 b Petroleum refining | 0.013 | 0.012 | 0.007 | 0.006 | 0.002 | 0.002 | 0.011 | 0.010 | 0.011 | 0.011 | 0.017 | 0.015 | 0.677 | 0.574 | 0.005 | 0.005 | 0.326 | 0.294 |
| 1 A 1 c Manufacture of solid fuels and other energy industries | 2.569 | 3.710 | 0.142 | 0.205 | 2.669 | 3.855 | 1.024 | 1.479 | 0.521 | 0.752 | 2.223 | 3.210 | 0.494 | 0.713 | 0.261 | 0.378 | 4.199 | NA |
| 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 20.073 | 22.568 | 0.160 | 0.204 | 0.598 | 0.800 | 0.306 | 0.409 | 1.092 | 1.440 | 1.352 | 1.802 | 2.079 | 2.494 | 0.134 | 0.181 | 15.620 | 6.065 |
| 1 A 2 b Stationary Combustion in manufacturing industries and construction: Non-ferrous metals | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | 20.693 |
| 1 A 2 c Stationary combustion in manufacturing industries and construction: Chemicals | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 2 e Stationary combustion in manufacturing industries and construction: Food processing, beverages and tobacco | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 3.352 | 2.500 | 0.177 | 0.117 | 0.967 | 0.092 | 0.335 | 0.134 | 1.464 | 1.254 | 1.249 | 0.785 | 18.432 | 20.393 | 0.219 | 0.020 | 8.432 | 5.154 |
| 1 A 2 f ii Mobile Combustion in manufacturing industries and construction: (Please specify in your IIR) | IE | NA | IE | NA | IE | NA | IE | NA | IE | NA | IE | NA | IE | NA | IE | NA | IE | NA |
| 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 a i (i) International aviation (LTO) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 b i Road transport: Passenger cars | 6.680 | 6.050 | 0.021 | 0.017 | NE | NE | NE | NE | 0.103 | 0.083 | 3.494 | 2.826 | 0.144 | 0.116 | 0.021 | 0.017 | 2.056 | 1.662 |
| 1 A 3 b ii Road transport:Light duty vehicles | 0.922 | 0.934 | 0.006 | 0.004 | NE | NE | NE | NE | 0.031 | 0.022 | 1.040 | 0.752 | 0.043 | 0.031 | 0.006 | 0.004 | 0.612 | 0.442 |
| 1 A 3 b iii Road transport:, Heavy duty vehicles | 0.020 | 0.020 | 0.018 | 0.018 | NE | NE | NE | NE | 0.087 | 0.087 | 2.952 | 2.685 | 0.122 | 0.122 | 0.018 | 0.018 | 1.737 | 1.738 |
| 1 A 3 b iv Road transport: Mopeds & motorcycles | 0.057 | 0.040 | 0.000 | 0.000 | NE | NE | NE | Ne | 0.000 | 0.000 | 0.020 | 0.013 | 0.001 | 0.000 | 0.000 | 0.000 | 0.011 | 0.008 |
| 1 A 3 b v Road transport: Gasoline evaporation | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 b vi Road transport: Automobile tyre and brake wear | NE | NE | NE | NE | NA | NA | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| 1 A 3 b vii Road transport: Automobile road abrasion | NA | NA | NA | NA | NA | NA | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE | NE |
| 1 A 3 c Railways | NA | NA | 0.002 | 0.002 | NA | NA | NA | NA | 0.008 | 0.010 | 0.285 | 0.306 | 0.012 | 0.013 | 0.002 | 0.002 | 0.168 | 0.180 |
| 1 A 3 d i (ii) International inland waterways | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 d ii National navigation (Shipping) | 0.009 | 0.008 | 0.876 | 0.822 | 4.379 | 4.113 | 4.379 | 4.113 | 0.004 | 0.003 | 4.379 | 4.113 | 0.006 | 0.006 | 0.018 | 0.016 | 0.044 | 0.041 |
| 1 A 3 e Pipeline compressors | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 a i Commercial / institutional: Stationary | 0.233 | 0.351 | 0.029 | 0.037 | 0.012 | 0.016 | 0.015 | 0.022 | 0.106 | 0.188 | 0.066 | 0.114 | 0.999 | 2.460 | 0.003 | 0.004 | 1.136 | 1.396 |
| 1 A 4 a ii Commercial / institutional: Mobile | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 b i Residential: Stationary plants | 6.277 | 5.032 | 0.270 | 0.241 | 0.104 | 0.080 | 0.168 | 0.145 | 0.696 | 0.786 | 1.414 | 1.213 | 3.800 | 6.764 | 0.075 | 0.058 | 20.431 | 16.097 |
| 1 A 4 b ii Residential: Household and gardening (mobile) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 0.044 | 0.043 | 0.003 | 0.003 | 0.001 | 0.001 | 0.003 | 0.002 | 0.027 | 0.013 | 0.016 | 0.009 | 0.459 | 0.052 | 0.000 | 0.001 | 0.106 | 0.195 |
| 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1A 4 c iii Agriculture/Forestry/Fishing: National fishing | 0.011 | 0.003 | 1.592 | 0.430 | 1.061 | 0.287 | 26.526 | 7.171 | 0.011 | 0.003 | 26.526 | 7.171 | 1.592 | 0.430 | 0.021 | 0.006 | 0.048 | 0.013 |
| 1 B 1 a Fugitive emission from solid fuels: Coal mining and handling | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 1 b Fugitive emission from solid fuels: Solid fuel transformation | 1.300 | 1.868 | 0.032 | 0.047 | 0.022 | 0.031 | 0.108 | 0.156 | NA | NA | NA | NA | 0.324 | 0.467 | NA | NA | NA | NA |
| 1 B 1 c Other fugitive emissions from solid fuels | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a i Exploration, production, transport | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a iv Refining / storage | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a v Distribution of oil products | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 b Natural gas | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

| NFR | PCDD/ PCDF |  | benzo(a) pyrene |  | benzo(b) fluoranthene |  | benzo(k) fluoranthene |  | Indeno (1,2,3-cd) pyrene |  | HCB |  | PCBs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | g I-Teq |  | Mg |  | Mg |  | Mg |  | Mg |  | kg |  | kg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 1 A 1 a Public electricity and heat production | 3.230 | 3.210 | 0.001 | 0.001 | 0.000 | 0.000 | 0.000 | 0.000 | 0.001 | 0.001 | 0.000 | 0.000 | 0.000 | 0.004 |
| 1 A 1 b Petroleum refining | 0.010 | 0.009 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | NA | NA | NA | NA |
| 1 A 1 c Manufacture of solid fuels and other energy industries | 0.811 | 1.171 | 0.058 | 0.084 | 0.028 | 0.041 | 0.024 | 0.034 | 0.016 | 0.024 | NA | NA | NA | NA |
| 1 A 2 a Stationary combustion in manufacturing industries and construction: Iron and steel | 15.249 | 20.458 | 3.408 | 4.578 | 4.409 | 5.924 | 1.780 | 2.400 | 1.386 | 1.861 | 0.046 | 0.062 | 12.652 | 17.018 |
| 1 A 2 b Stationary Combustion in manufacturing industries and construction: Non-ferrous metals | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 2 c Stationary combustion in manufacturing industries and construction: Chemicals | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 2 d Stationary combustion in manufacturing industries and construction: Pulp, Paper and Print | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 2 e Stationary combustion in manufacturing industries and construction: Food processing, beverages and tobacco | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 2 f i Stationary combustion in manufacturing industries and construction: Other (Please specify in your IIR) | 6.092 | 7.128 | 1.175 | 1.357 | 1.550 | 1.821 | 0.702 | 0.809 | 0.521 | 0.620 | 0.144 | 0.093 | 2.125 | 1.989 |
| 1 A 2 f ii Mobile Combustion in manufacturing industries and construction: (Please specify in your IIR) | IE | IE | IE | IE | IE | IE | IE | IE | IE | NA | IE | NA | IE | NA |
| 1 A 3 a ii (i) Civil aviation (Domestic, LTO) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 a i (i) International aviation (LTO) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 b i Road transport: Passenger cars | 0.600 | 0.600 | 0.070 | 0.056 | 0.129 | 0.108 | 0.113 | 0.094 | 0.081 | 0.068 | NA | NA | NA | NA |
| 1 A 3 b ii Road transport:Light duty vehicles | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | NA | NA | NA | NA |
| 1 A 3 b iii Road transport:, Heavy duty vehicles | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | NA | NA | NA | NA |
| 1 A 3 b iv Road transport: Mopeds & motorcycles | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | NA | NA | NA | NA |
| 1 A 3 b v Road transport: Gasoline evaporation | NA | NA | NE | NE | NE | NE | NE | NE | NE | NE | NA | NA | NA | NA |
| 1 A 3 b vi Road transport: Automobile tyre and brake wear | NA | NA | NE | NE | NE | NE | NE | NE | NE | NE | NA | NA | NA | NA |
| 1 A 3 b vii Road transport: Automobile road abrasion | NA | NA | NE | NE | NE | NE | NE | NE | NE | NE | NA | NA | NA | NA |
| 1 A 3 c Railways | NA | NA | 0.005 | 0.005 | 0.008 | 0.010 | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 d i (ii) International inland waterways | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 3 d ii National navigation (Shipping) | NA | NA | 0.000 | 0.000 | 0.002 | 0.002 | 0.000 | 0.000 | 0.001 | 0.001 | NA | NA | NA | NA |
| 1 A 3 e Pipeline compressors | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 a i Commercial / institutional: Stationary | 1.934 | 2.164 | 0.270 | 0.320 | 0.387 | 0.453 | 0.146 | 0.180 | 0.133 | 0.156 | 0.034 | 0.036 | 0.339 | 0.373 |
| 1 A 4 a ii Commercial / institutional: Mobile | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 b i Residential: Stationary plants | 102.070 | 79.228 | 30.829 | 24.240 | 32.507 | 25.501 | 19.046 | 14.971 | 20.469 | 16.144 | 0.861 | 0.674 | 8.934 | 6.809 |
| 1 A 4 b ii Residential: Household and gardening (mobile) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 A 4 c i Agriculture/Forestry/Fishing: Stationary | 0.205 | 0.513 | 0.034 | 0.071 | 0.047 | 0.103 | 0.020 | 0.037 | 0.016 | 0.035 | 0.003 | 0.009 | 0.034 | 0.094 |
| 1 A 4 c ii Agriculture/Forestry/Fishing: Off-road vehicles and other machinery | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1A 4 c iii Agriculture/Forestry/Fishing: National fishing | 0.000 | 0.000 | 0.000 | 0.000 | 0.001 | 0.000 | 0.000 | 0.000 | 0.001 | 0.000 | 0.000 | 0.000 | NA | NA |
| 1 B 1 a Fugitive emission from solid fuels: Coal mining and handling | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 1 b Fugitive emission from solid fuels: Solid fuel transformation | NA | NA | 0.811 | 1.167 | 0.270 | 0.389 | 0.270 | 0.389 | 0.032 | 0.467 | NA | NA | NA | NA |
| 1 B 1 c Other fugitive emissions from solid fuels | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a i Exploration, production, transport | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a iv Refining / storage | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 a v Distribution of oil products | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 B 2 b Natural gas | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

2.2. INDUSTRIAL PROCESSES (NFR SECTOR 2)

2.A. Mineral industry

      - 2.A.1 Cement production

      - 2.A.2 Lime production

      - 2.A.4 Soda Ash production and use

      - 2.A.5 Asphalt roofing

2.B. Chemical industry

2.C. Metal industry

      - 2.C.1 Iron and Steel Production

      - 2.C.2 Ferroalloys production

      - 2.C.3 Aluminum production

      - 2.C.5.a Copper Production

      - 2.C.5.b Lead Production

      - 2.C.5.d Zinc Production

2.D. Other production industry

      - 2.D.1 Pulp and paper

      - 2.D.2 Food and drink

      - 2.D.3 Wood processing

2.G Other industrial processes

In this category are considered only process related emissions, emissions due to fuel combustion in manufacturing industries being allocated in NFR Category 1 A 2 Fuel Combustion – Manufacturing Industries and Construction.

The methodology for estimating emissions for the industrial processes sector implies multiplying production data for each process by an emission factor. The activity data are provided by NIS and emission factors are established using the guidebook recommendation. There are no plant specific data, except of aluminum production and some subsector from chemical industry (data are mainly related to the processes technology).

The emission factors and the activity data are presented in tables 6 and 7. The table 8 contains the emissions calculated for industrial processes.

Table 5 - Key Source Analysis – INDUSTRIAL PROCESSES

| Pollutant | Year | 2A1 | 2B2 | 2B5a | 2C1 | 2C3 | 2C5a | 2C5b | 2C5d | 2D3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx | 2007 |  | <1.00% |  |  |  |  |  |  |  |
|  | 2008 |  | 1.50% |  |  |  |  |  |  |  |
| NMVOC | 2007 |  |  | 2.60% |  |  |  |  |  |  |
|  | 2008 |  |  | 3.60% |  |  |  |  |  |  |
| CO | 2007 |  |  |  |  | 2.50% |  |  |  |  |
|  | 2008 |  |  |  |  | 2.60% |  |  |  |  |
| TSP | 2007 | 1.00% |  | 27.30% |  |  |  |  |  | 1.40% |
|  | 2008 | <1.00% |  | 34.20% |  |  |  |  |  | 1.20% |
| PM10 | 2007 | 1.50% |  |  |  |  |  |  |  |  |
|  | 2008 | 1.50% |  |  |  |  |  |  |  |  |
| PM2.5 | 2007 | 1.00% |  |  |  |  |  |  |  |  |
|  | 2008 | <1.00% |  |  |  |  |  |  |  |  |
| Pb | 2007 |  |  |  | 41.70% |  | 2.80% | 8.60% |  |  |
|  | 2008 |  |  |  | 34.00% |  | 2.40% | 10.00% |  |  |
| Hg | 2007 |  |  |  | 3.70% |  |  |  |  |  |
|  | 2008 |  |  |  | 2.80% |  |  |  |  |  |
| Cd | 2007 |  |  |  | 17.60% |  | 5.10% |  | 3.70% |  |
|  | 2008 |  |  |  | 9.90% |  | 3.00% |  | 3.20% |  |
| DIOX | 2007 |  |  |  | 7.80% |  |  |  |  |  |
|  | 2008 |  |  |  | 6.00% |  |  |  |  |  |
| PAH | 2007 |  |  |  |  | 24.90% |  |  |  |  |
|  | 2008 |  |  |  |  | 22.90% |  |  |  |  |
| HCB | 2007 |  |  |  | 8.20% |  |  |  |  |  |
|  | 2008 |  |  |  | 9.50% |  |  |  |  |  |

Table 6 – EMISSION FACTORS – INDUSTRIAL PROCESSES

| Category | Pollutant | Emission Factor | Unit | Sources (EMEP/EEA Guidebook) |
| --- | --- | --- | --- | --- |
| 2.A.1 | PM10 | 200 | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.A.1 Cement production |
|  | PM25 | 110 | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.A.1 Cement production |
|  | TSP | 220 | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.A.1 Cement production |
| 2.A.2 | PM10 | 0.24 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 2.A.2 Lime production |
|  | PM25 | 0.05 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 2.A.2 Lime production |
|  | TSP | 0.59 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 2.A.2 Lime production |
| 2.A.4 | CO | 9 | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.A.4 Soda ash production and use |
|  | NH3 | 0.9 | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.A.4 Soda ash production and use |
|  | TSP | 0.1 | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.A.4 Soda ash production and use |
| 2.A.5 | CO | 10 | g/t | Table 3.1 Tier 1 emission factors for source category 2.A.4 Asphalt roofing |
|  | NMVOC | 5 | g/t | Table 3.1 Tier 1 emission factors for source category 2.A.4 Asphalt roofing |
|  | TSP | 600 | g/t | Table 3.1 Tier 1 emission factors for source category 2.A.4 Asphalt roofing |
| 2.B.1 | CO | 0.1 | kg/t | Table 3.1 Tier 1 emission factors for source category 2.B.1 Ammonia production |
|  | NH3 | 0.01 | kg/t | Table 3.1 Tier 1 emission factors for source category 2.B.1 Ammonia production |
|  | NOX | 1 | kg/t | Table 3.1 Tier 1 emission factors for source category 2.B.1 Ammonia production |
| 2.B.2 | NOX | C | g/Mg product | Table 3.2 Tier 1 emission factor for source category 2.B.2 Nitric acid production |
| 2.B.5.a | NH3 | 2.5 | kg/Mg product | Table 3.26 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, urea |
| 2.B.5.a | NMVOC | 120 | g/t | Table 3.44 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polystyrene, general purpose polystyrene (GPPS) (040511) |
|  |  | 96 | g/t | Table 3.40 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyvinylchloride, suspension PVC (S-PVC) |
|  |  | 5 | kg/Mg product | Table 3.49 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, styrene-butadiene rubber (SBR) (040514) |
|  |  | 4 | kg/t | Table 3.42 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polypropylene (040509) |
|  |  | 2.3 | kg/t | Table 3.39 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyethylene high density (040507) |
|  |  | 2.4 | kg/t | Table 3.38 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyethylene low density (040506) |
|  |  | 0.6 | kg/t | Table 3.35 Tier 2 emission factors for source category 2.B.5.a Other chemical industry |
|  |  | 6 | kg/t | Table 3.35 Tier 2 emission factors for source category 2.B.5.a Other chemical industry |
|  |  | 2 | kg/t | Table 3.51 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, ethylene oxide production |
|  |  | 8 | kg/t | Table 3.5 Tier 1 emission factors for source category 2.B.5.a Other chemical industry |
| 2.B.5.a | PM10 | 100 | g/t | Table 3.40 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyvinylchloride, suspension PVC (S-PVC) |
|  |  | 1.2 | kg/Mg product | Table 3.26 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, urea |
| 2.B.5.a | PM25 | 5 | g/t | Table 3.40 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyvinylchloride, suspension PVC (S-PVC) |
|  |  | 0.9 | kg/Mg product | Table 3.26 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, urea |
| 2.B.5.a | TSP | 97 | g/t | Table 3.39 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyethylene high density (040507) |
|  |  | 4 | g/t | Table 3.44 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polystyrene, general purpose polystyrene (GPPS) (040511) |
|  |  | 31 | g/t | Table 3.38 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyethylene low density (040506) |
|  |  | 263 | g/t | Table 3.40 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polyvinylchloride, suspension PVC (S-PVC) |
|  |  | 1.5 | kg/Mg product | Table 3.26 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, urea |
|  |  | 1.5 | kg/t | Table 3.42 Tier 2 emission factors for source category 2.B.5.a Other chemical industry, polypropylene (040509) |
|  |  | 50 | kg/t | Table 3.5 Tier 1 emission factors for source category 2.B.5.a Other chemical industry |
| 2.C.1 | As | 0.4 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.015 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 0.018 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | Cd | 0.067 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.004 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
|  |  | 0.2 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
| 2.C.1 | CO | 3.5 | kg/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 1.7 | kg/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
| 2.C.1 | Cr | 2.3 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 2.3 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.016 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
|  |  | 0.1 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
| 2.C.1 | Cu | 0.02 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.02 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 0.015 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 0.033 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | DIOX | 0.8 | µg/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 0.00775 | µg/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 1.8 | µg/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
|  |  | 0.002 | µg/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
| 2.C.1 | HCB | 32 | µg/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | Hg | 0.0014 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.0001 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 0.049 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
|  |  | 0.05 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
| 2.C.1 | Ni | 0.7 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 0.13 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.09 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | NMVOC | 138 | g/Mg product | Tb 3.2-Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
|  |  | 7 | g/Mg product | Table 3.25 Tier 2 emission factors for source category 2.C.1 Iron and steel production, rolling mills, hot rolling mills |
|  |  | 46 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
| 2.C.1 | NOX | 10 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 130 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
| 2.C.1 | PAH | 16 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 0.32 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
|  |  | 2.5 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 0.1 | mg/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
| 2.C.1 | Pb | 4 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.0006 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 2.6 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 3.5 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | PCBs | 8 | mg/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 3.6 | mg/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 3.6 | mg/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
|  |  | 2 | mg/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
| 2.C.1 | PM10 | 32 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 40 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 24 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 100 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | PM25 | 28 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 25 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 21 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 80 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | Se | 0.003 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.02 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | SO2 | 60 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
| 2.C.1 | TSP | 35 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 50 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 9 | g/Mg product | Table 3.25 Tier 2 emission factors for source category 2.C.1 Iron and steel production, rolling mills, hot rolling mills |
|  |  | 30 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 96 | g/Mg product | Table 3.24 Tier 2 emission factors for source category 2.C.1 Iron and steel production, rolling mills, cold rolling mills |
|  |  | 200 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.1 | Zn | 3.6 | g/Mg product | Table 3.17 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, electric arc furnace steel plant |
|  |  | 4 | g/Mg product | Table 3.15 Tier 2 emission factors for source category 2.C.1 Iron and steel production, steel making, basic oxygen furnace |
|  |  | 0.073 | g/Mg product | Table 3.8 Tier 2 emission factors for source category 2.C.1 Iron and steel production, pig iron |
|  |  | 0.06 | g/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.1 Iron and steel production, sinter |
| 2.C.2 | TSP | 1 | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.C.2 Ferroalloys production |
| 2.C.3 | Benzo(a) | C | g/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | Benzo(b) | C | g/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | Benzo(k) | C | g/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | CO | C | kg/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | Indeno | C5 | g/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | NOX | C | kg/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | PM10 | C | kg/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | PM25 | C | kg/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | SO2 | C | kg/Mg product | Table 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.3 | TSP | C | kg/Mg product | able 3.2 Tier 2 emission factors for source category 2.C.3 Aluminium production, primary aluminium production, pre-bake cell |
| 2.C.5.a | As | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | Cd | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | Cr | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | Cu | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | DIOX | C | µg/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | Hg | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | Ni | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | Pb | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | PCBs | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | PM10 | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | PM25 | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.a | TSP | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.a Copper production |
| 2.C.5.b | As | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | Cd | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | DIOX | C | µg/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | Hg | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | Pb | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | PCBs | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | PM10 | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | PM25 | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | TSP | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.b | Zn | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.b Lead production |
| 2.C.5.d | As | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | Cd | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | DIOX | C | µg/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | Hg | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | Pb | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | PCBs | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | PM10 | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | PM25 | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | TSP | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.C.5.d | Zn | C | g/Mg product | Table 3-1 Tier 1 emission factors for source category 2.C.5.d Zinc production |
| 2.D.1 | CO | C | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.1 Pulp and paper |
| 2.D.1 | NMVOC | C | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.1 Pulp and paper |
| 2.D.1 | NOX | C | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.1 Pulp and paper |
| 2.D.1 | PM10 | C | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.1 Pulp and paper |
| 2.D.1 | PM25 | C | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.1 Pulp and paper |
| 2.D.1 | SO2 | C | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.1 Pulp and paper |
| 2.D.1 | TSP | C | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.1 Pulp and paper |
| 2.D.2 | NMVOC | 0.08 | kg/hl product | Table 3-25 Tier 2 emission factors for source category 2.D.2 Food and drink, Red Wine |
|  |  | 15 | kg/hl product | Table 3-28 Tier 2 emission factors for source category 2.D.2 Food and drink, Spirits |
|  |  | 0.035 | kg/hl product | 2D2 - Table 3-26 Tier 2 emission factors for source category 2.D.2 Food and drink, White Wine |
|  |  | 0.4 | kg/hl product | Table 3-32 Tier 2 emission factors for source category 2.D.2 Food and drink, Other spirits |
|  |  | 3.5 | kg/hl product | Table 3-31 Tier 2 emission factors for source category 2.D.2 Food and drink, Brandy |
|  |  | 0.035 | kg/hl product | Table 3-27 Tier 2 emission factors for source category 2.D.2 Food and drink, Beer |
|  |  | 4.5 | kg/Mg product | 2D2 Table 3-11 Tier 2 emission factors for source category 2.D.2 Food and drink, Bread (typical) |
| 2.D.3 | TSP | 1 | kg/Mg product | Table 3.1 Tier 1 emission factors for source category 2.D.3 Wood processing |
| 2.G | NMVOC | 0.55 | kg/Mg product | Table 3-23 Tier 2 emission factors for source category 2.D.2 Food and drink, Coffee roasting |
|  |  | 10 | kg/Mg product | 2D1 - Table 3-21 Tier 2 emission factors for source category 2.D.2 Food and drink, Margarine |

Table 7 – ACTIVITY DATA – INDUSTRIAL PROCESSES

| Category | NAME | 2007 | 2008 | Unit |
| --- | --- | --- | --- | --- |
| 2.A.1 | Cement production | 10060000 | 10660000 | Mg product |
| 2.A.2 | Lime production | 2044000 | 2174137 | Mg product |
| 2.A.4 | Soda Ash production and use | C | C | Mg product |
| 2.A.5 | Asphalt roofing | 2269 | 1434 | t |
| 2.B.1 | Ammonia production | 1371000 | 1275000 | t |
| 2.B.2 | Nitric acid production | C | C | Mg product |
| 2.B.5.a | Other chemical industry | C | C | Mg product |
| 2.B.5.a | Other chemical industry | C | C | t |
| 2.C.1 | Iron and Steel Production | 22814244 | 17189249 | Mg product |
| 2.C.2 | Ferroalloys production | 26868 | 14162 | Mg product |
| 2.C.3 | Aluminum production | C | C | Mg product |
| 2.C.5.a | Copper production | C | C | Mg product |
| 2.C.5.b | Lead production | C | C | Mg product |
| 2.C.5.d | Zinc production | C | C | Mg product |
| 2.D.1 | Pulp and paper | C | C | Mg product |
| 2.D.2 | Food and drink | 197508554 | 25747864 | hl product |
| 2.D.2 | Food and drink | 750370 | 702715 | Mg product |
| 2.D.3 | Wood processing | 2974827 | 3098447 | Mg product |
| 2.G | Other | 120940.535 | 89927 | Mg product |

Table 8 – EMISSION DATA FROM INDUSTRIAL PROCESSES SECTOR

| NFR | Main Pollutants |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nox (as NO2) |  | NMVOC |  | SOx (as SO2) |  | NH3 |  |
|  | Gg NO2 |  | Gg |  | Gg SO2 |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 2 A 1 Cement production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 2 Lime production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 3 Limestone and dolomite use | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 4 Soda ash production and use | NA | NA | NA | NA | NA | NA | 0.445 | 0.407 |
| 2 A 5 Asphalt roofing | NA | NA | 0.000 | 0.000 | NA | NA | NA | 0.000 |
| 2 B 1 Ammonia production | 1.275 | 1.371 | NA | NA | NA | NA | 0.013 | 0.014 |
| 2 B 2 Nitric acid production | 4.471 | 4.362 | NA | NA | NA | NA | NA | NA |
| 2 B 3 Adipic acid production | NE | NE | NA | NA | NA | NA | NA | NA |
| 2 B 4 Carbide production | NE | NE | NE | NE | NE | NE | NA | NA |
| 2 B 5 a Other chemical industry | NA | NA | 15.980 | 11.195 | NA | NA | 0.556 | 0.509 |
| 2 B 5 b Storage, handling and transport of chemical | NE | NE | IE | IE | NA | NA | IE | IE |
| 2 C 1 Iron and steel production | 0.225 | 0.292 | 0.800 | 0.992 | 0.094 | 0.115 | NA | NA |
| 2 C 2 Ferroalloys production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 3 Aluminum production | 0.625 | 0.602 | NA | NA | 4.374 | 4.214 | NA | NA |
| 2 C 5 a Copper production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 5 b Lead production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 5 d Zinc production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 D 1 Pulp and paper | 0.022 | 0.086 | 0.044 | 0.172 | 0.044 | 0.172 | NA | NA |
| 2 D 2 Food and drink | NA | NA | 4.442 | 10.626 | NA | NA | NA | NA |
| 2 D 3 Wood processing | NE | NE | NE | NE | NE | NE | NE | NE |
| 2 G Other production, consumption, storage, transportation or handling of bulk products | NE | NE | 0.620 | 0.643 | NE | NE | NE | NE |

| NFR | PM2.5 |  | PM10 |  | TSP |  | CO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gg |  | Gg |  | Gg |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 2 A 1 Cement production | 1.1726 | 1.107 | 2.132 | 2.012 | 2.3452 | 2.2132 | NA | NA |
| 2 A 2 Lime production | 0.108707 | 0.1022 | 0.521793 | 0.4905 | 1.282741 | 1.20596 | NA | NA |
| 2 A 3 Limestone and dolomite use | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 4 Soda ash production and use | NA | NA | NA | NA | 0.0494 | 0.0452 | 4.446 | 4.068 |
| 2 A 5 Asphalt roofing | NA | NA | NA | NA | 0.0008 | 0.001 | 0 | 0.00002 |
| 2 B 1 Ammonia production | NA | NA | NA | NA | NA | NA | 0.1275 | 0.137 |
| 2 B 2 Nitric acid production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 B 4 Carbide production | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 B 5 a Other chemical industry | 0.201069 | 0.185 | 0.287992 | 0.2709 | 89.88627 | 58.669 | NA | NA |
| 2 C 1 Iron and steel production | 0.572541 | 0.762 | 0.731326 | 0.9692 | 1.384746 | 1.784 | 10.49315 | 18.505 |
| 2 C 2 Ferroalloys production | NA | NA | NA | NA | 0.014162 | 0.027 | NA | NA |
| 2 C 3 Aluminum production | 0.437427 | 0.4214 | 0.999834 | 0.963 | 1.249792 | 1.204 | 37.49376 | 36.12 |
| 2 C 5 a Copper production | 0.003 | 0.004 | 0.004 | 0.006 | 0.005 | 0.007 | NA | NA |
| 2 C 5 b Lead production | 0.007 | 0.007 | 0.014 | 0.014 | 0.017 | 0.018 | NA | NA |
| 2 C 5 d Zinc production | 0.019 | 0.018 | 0.025 | 0.023 | 0.031 | 0.029 | NA | NA |
| 2 D 1 Pulp and paper | 0.0132 | 0.0516 | 0.0176 | 0.068 | 0.022 | 0.086 | NA | 0.473 |
| 2 D 2 Food and drink | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 D 3 Wood processing | NA | NA | NA | NA | 3.098447 | 2.975 | NA | NA |
| 2 G Other production, consumption, storage, transportation or handling of bulk products | NA | NA | NA | NA | NA | NA | NA | NA |

| NFR | Pb |  | Cd |  | Hg |  | As |  | Cr |  | Cu |  | Ni |  | Se |  | Zn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 2 A 1 Cement production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 2 Lime production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 3 Limestone and dolomite use | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 4 Soda ash production and use | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 5 Asphalt roofing | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 B 1 Ammonia production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 B 2 Nitric acid production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 B 3 Adipic acid production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 1 Iron and steel production | 30.655 | 44.352 | 0.482 | 0.700 | 0.328 | 0.409 | 1.011 | 1.885 | 12.194 | 19.340 | 0.287 | 0.400 | 1.836 | 2.469 | 0.108 | 0.138 | 15.093 | 24.978 |
| 2 C 2 Ferroalloys production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 3 Aluminum production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 5 a Copper production | 2.132 | 2.926 | 0.147 | 0.201 | 0.000 | 0.000 | 0.520 | 0.713 | 0.213 | 0.293 | 0.933 | 1.280 | 0.187 | 0.256 | NA | NA | NA | NA |
| 2 C 5 b Lead production | 8.995 | 9.189 | 0.024 | 0.024 | 0.013 | 0.013 | 0.073 | 0.074 | NA | NA | NA | NA | NA | NA | NA | NA | 2.422 | 2.474 |
| 2 C 5 d Zinc production | 0.868 | 0.818 | 0.155 | 0.146 | 0.236 | 0.222 | 0.007 | 0.007 | NA | NA | NA | NA | NA | NA | NA | NA | 2.479 | 2.337 |
| 2 D 1 Pulp and paper | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 D 2 Food and drink | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 D 3 Wood processing | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

| NFR | PCDD/ PCDF |  | benzo(a) pyrene |  | benzo(b) fluoranthene |  | benzo(k) fluoranthene |  | Indeno (1,2,3-cd) pyrene |  | HCB |  | PCBs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | g I-Teq |  | Mg |  | Mg |  | Mg |  | Mg |  | kg |  | kg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 2 A 1 Cement production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 2 Lime production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 3 Limestone and dolomite use | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 4 Soda ash production and use | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 A 5 Asphalt roofing | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 B 1 Ammonia production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 B 2 Nitric acid production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 1 Iron and steel production | 10.340 | 12.854 | NA | NA | NA | NA | NA | NA | NA | NA | 0.161 | 0.201 | 44.591 | 61.400 |
| 2 C 2 Ferroalloys production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 3 Aluminum production | NA | NA | 9.373 | 9.030 | 12.498 | 12.041 | 12.498 | 12.040 | 1.562 | 1.505 | NA | NA | NA | NA |
| 2 C 5 a Copper production | 0.067 | 0.091 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | 11.991 | 16.460 |
| 2 C 5 b Lead production | 0.173 | 0.177 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | 65.732 | 67.148 |
| 2 C 5 c Nickel production | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 C 5 d Zinc production | 0.310 | 0.292 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | 55.780 | 52.589 |
| 2 D 1 Pulp and paper | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 D 2 Food and drink | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 D 3 Wood processing | NA | NA | NA | NA | NA | 0.000 | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 G Other production, consumption, storage, transportation or handling of bulk products | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

2.3. PRODUCT USE (NFR SECTOR 3)

3.A. Paint Application

3.B. Degreasing and Dry Cleaning

      - 3.B.1 Degreasing

      - 3.B.2 Dry Cleaning

3.C Chemical products

3.D. Other Product Use

      - 3.D.1 Printing

      - 3.D.2 Domestic solvent use

      - 3.D.3 Other product use

This group of activities includes:

- The application of organic solvents based dyes/paints and water based (automobiles, ships, engines, heavy machinery)

- Using of solvents for metallic surfaces degreasing and for dry cleaning

- The production and processing of chemical products: tire, medicines, paints, polystyrene.

- Using of solvents in some other activities, including domestic ones

The specific pollutants of this group are the non-methane VOC’s (NMVOC’s). The main part of the activity data were provided by NIS and another part were risen  from the Inventory of Installations/Activities VOC under 1999/13/EC Directive.

The uncertainty for NMVOC emissions is supposed to be high due to the lack of information concerning the ink, glue, foam and adhesives usage, activities with high emission values factors.  Emission estimations will be improved after the establishing of a link between the guidebook categories and all the activities described in Solvents Emission Directive.

All resulting emissions from the activities included in product use sector, emission factors, activity data and share from national total are presented in table 9, 10, 11 and 12.

Table 9 - Key Source Analysis – PRODUCT USE

| Pollutant | Year | 3C | 3D2 |
| --- | --- | --- | --- |
| NMVOC | 2007 | 6.60% | 5.00% |
|  | 2008 | 5.80% | 4.80% |

Table 10 – EMISSION FACTORS – PRODUCT USE

| Category | Pollutant | Emission Factor | Unit | Sources (EMEP/EEA Guidebook |
| --- | --- | --- | --- | --- |
| 3.A.1 | NMVOC | 230 | g/kg product | Table 3-4 Tier 2 emission factors for source category 3.A.1 Decorative coating application |
| 3.A.2 | NMVOC | 720 | g/kg product | Table 3-7 Tier 2 emission factors for source category 3.A.2 Industrial coating application, Vehicles refinishing |
| 3.A.2 | NMVOC | 150 | kg/caput | Table 3-12 Tier 2 emission factors for source category 3.A.2 Industrial coating application, Bus coating |
| 3.A.2 | NMVOC | 8 | kg/caput | able 3-11 Tier 2 emission factors for source category 3.A.2 Industrial coating application, Truck Cabin coating |
| 3.A.2 | NMVOC | 8 | kg/caput | Table 3-6 Tier 2 emission factors for source category 3.A.2 Industrial coating application, Car coating |
| 3.A.2 | NMVOC | 28 | kg/caput | Table 3-10 Tier 2 emission factors for source category 3.A.2 Industrial coating application, Truck/Van coating |
| 3.B.1 | NMVOC | 460 | g/kg solvent | Table 3-1 Tier 1 emission factors for source category 3.B.1 Degreasing |
| 3.B.2 | NMVOC | 0.3 | kg/caput | The factor is 0.3 kg/inhabitant/year and has a quality rating of E.(pag 10) |
| 3.C | As | 0.0005 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.C | Cd | 0.0001 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.C | Cr | 0.006 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.C | Ni | 0.05 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.C | NMVOC | 11 | g/kg product | Table 3-11 Tier 2 emission factors for source category 3.C Chemical Products, Manufacture of paints, inks and glues |
| 3.C | NMVOC | 60 | g/kg product | Table 3-4 Tier 2 emission factors for source category 3.C Chemical Products, Polystyrene Foam Processing |
| 3.C | NMVOC | 40 | g/kg product | emission factors from the old version of CORINAIR guidebook |
| 3.C | NMVOC | 11 | g/kg product | Table 3-11 Tier 2 emission factors for source category 3.C Chemical Products, Manufacture of paints, inks and glues |
| 3.C | NMVOC | 8 | g/kg product | Table 3-5 Tier 2 emission factors for source category 3.C Chemical Products, Rubber processing except tyre production |
| 3.C | NMVOC | 11 | g/kg product | Table 3-11 Tier 2 emission factors for source category 3.C Chemical Products, Manufacture of paints, inks and glues |
| 3.C | NMVOC | 120 | g/kg product | Table 3-3 Tier 2 emission factors for source category 3.C Chemical Products, Polyurethane Foam Processing |
| 3.C | NMVOC | 1710 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.C | PAH | 4000 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.C | Se | 0.0005 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.C | TSP | 12000 | g/Mg product | Table 3-10 Tier 2 emission factors for source category 3.C Chemical Products, Asphalt blowing |
| 3.D.1 | NMVOC | 500 | g/kg product | Table 3-1 Tier 1 emission factors for source category 3.D.1 Printing |
| 3.D.2 | NMVOC | 1 | kg/caput | Table 3-1 Tier 1 emission factors for source category 3.D.2 Domestic solvent use including fungicides |
| 3.D.3 | Benzo(a) | C | mg/kg solvent | Table 3-3 Tier 2 emission factors for source category 3.D.3 Other product use, Wood preservation, Creosote preservative type |
| 3.D.3 | Benzo(b) | C | mg/kg solvent | Table 3-3 Tier 2 emission factors for source category 3.D.3 Other product use, Wood preservation, Creosote preservative type |
| 3.D.3 | Indeno | C | mg/kg solvent | Table 3-3 Tier 2 emission factors for source category 3.D.3 Other product use, Wood preservation, Creosote preservative type |
| 3.D.3 | NMVOC | C | g/kg solvent | Table 3-4 Tier 2 emission factors for source category 3.D.3 Other product use, Wood |
| 3.D.3 | NMVOC | C | g/kg solvent | Table 3-3 Tier 2 emission factors for source category 3.D.3 Other product use, Wood preservation, Creosote preservative type |
| 3.D.3 | NMVOC | 0.003 | g/t | Table 3-2 Tier 2 emission factors for source category 3.D.3 Other product use, Fat, edible |
| 3.D.3 | NMVOC | 0.2 | kg/caput | Table 3-7 Tier 2 emission factors for source category 3.D.3 Other product use, Treatment of vehicles |
| 3.D.3 | PM10 | 0.0009 | g/t | Table 3-2 Tier 2 emission factors for source category 3.D.3 Other product use, Fat, edible |
| 3.D.3 | PM25 | 0.0006 | g/t | Table 3-2 Tier 2 emission factors for source category 3.D.3 Other product use, Fat, edible |
| 3.D.3 | TSP | 0.0011 | g/t | Table 3-2 Tier 2 emission factors for source category 3.D.3 Other product use, Fat, edible |

Table 11 – ACTIVITY DATA – PRODUCT USE

| Category | NAME | 2007 | 2008 | Unit |
| --- | --- | --- | --- | --- |
| 3.A.1 | Paint Application | 14432854 | 20603520 | kg product |
| 3.A.2 | Paint Application | 8249722 | 1493412 | kg product |
| 3.A.2 | Paint Application | 241767 | 241023 | caput |
| 3.A.3 | Paint Application | 0 | 0 | caput |
| 3.B.1 | Degreasing |  | 5640329 | kg solvent |
| 3.B.2 | Dry Cleaning | 21584365 | 21504442 | caput |
| 3.C | Chemical Products | 447703000 | 505407009 | kg product |
| 3.C | Chemical Products | 160000 | 240000 | Mg product |
| 3.D.1 | Printing | 501000 | 601950 | kg product |
| 3.D.2 | Domestic solvent use | 21584365 | 21504442 | caput |
| 3.D.3 | Other product use | 21584365 | 21584365 | caput |
| 3.D.3 | Other product use | C | C | kg solvent |
| 3.D.3 | Other product use | 32218 | 30767 | t |

Table 12 – EMISSION DATA FROM PRODUCT USE SECTOR

| NFR | Main Pollutants |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nox (as NO2) |  | NMVOC |  | SOx (as SO2) |  | NH3 |  |
|  | Gg NO2 |  | Gg |  | Gg SO2 |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 3 A 1 Decorative coating application | NA | NA | 4.739 | 3.32 | NA | NA | NA | NA |
| 3 A 2 Industrial coating application | NA | NA | 3.205 | 8.03 | NA | NA | NA | NA |
| 3 A 3 Other coating application | NA | NA | NE | NE | NA | NA | NA | NA |
| 3 B 1 Degreasing | NA | NA | 2.595 | NE | NA | NA | NA | NA |
| 3 B 2 Dry cleaning | NA | NA | 6.451 | 6.475 | NA | NA | NA | NA |
| 3 C Chemical products | NA | NA | 26 | 28.87 | NA | NA | NA | NA |
| 3 D 1 Printing | NA | NA | 0.301 | 0.25 | NA | NA | NA | NA |
| 3 D 2 Domestic solvent use including fungicides | NA | NA | 21.504 | 21.584 | NA | NA | NA | NA |
| 3 D 3 Other product use | NA | NA | 4.438 | 4.56 | NA | NA | NA | NA |

| NFR | PM2.5 |  | PM10 |  | TSP |  | CO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gg |  | Gg |  | Gg |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 3 A 1 Decorative coating application | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 A 2 Industrial coating application | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 A 3 Other coating application | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 B 1 Degreasing | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 B 2 Dry cleaning | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 C Chemical products | NA | NA | NA | NA | 2.88 | 1.92 | NA | NA |
| 3 D 1 Printing | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 D 2 Domestic solvent use including fungicides | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 D 3 Other product use | 0 | 0 | 0 | 0 | 0 | 0 | NA | NA |

| NFR | Pb |  | Cd |  | Hg |  | As |  | Cr |  | Cu |  | Ni |  | Se |  | Zn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 3 A 1 Decorative coating application | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 A 2 Industrial coating application | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 A 3 Other coating application | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 B 1 Degreasing | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 B 2 Dry cleaning | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 C Chemical products | NA | NA | 0 | 0.00002 | NA | NA | 0.00012 | 0.00008 | 0.00144 | 0.001 | NA | NA | 0.012 | 0.008 | 0.00012 | 0.00008 | NA | NA |
| 3 D 1 Printing | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 D 2 Domestic solvent use including fungicides | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 D 3 Other product use | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

| NFR | PCDD/ PCDF |  | benzo(a)pyrene |  | benzo(b)fluoranthene |  | benzo(k)fluoranthene |  | Indeno (1,2,3-cd) pyrene |  | HCB |  | PCBs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | g I-Teq |  | Mg |  | Mg |  | Mg |  | Mg |  | kg |  | kg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 3 A 1 Decorative coating application | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 A 2 Industrial coating application | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 A 3 Other coating application | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 B 1 Degreasing | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 B 2 Dry cleaning | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 C Chemical products | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 D 1 Printing | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 D 2 Domestic solvent use including fungicides | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3 D 3 Other product use | NA | NA | 0.00006 | 0.000093 | 0.00003 | NA | NA | NA | 0.00003 | 0.00005 | NA | NA | NA | NA |

2.4. AGRICULTURE (NFR SECTOR 4)

4.B. Manure Management

      - 4.B Animal Husbandry and Manure Management.

4.D. Agricultural Soils

      - 4.D Crop production and agricultural soils

In this group of activities are included the emissions from:

      - livestock feeding and housing;

      - manure storage;

      - field-applied manure;

      - fertiliser application (NH3);

      - soil cultivation and crop harvesting.

The activity data was provided by the NIS and Ministry of Agriculture and Rural Development.

Emission factors for total NH3 emissions from soils due to N fertiliser was calculated according to the guidebook assuming a 10 grd mean spring temperature.

All resulting emissions from the activities included in product use sector, emission factors, activity data and share from national total are presented in table 13, 14, 15 and 16.

Table 13 - Key Source Analysis – AGRICULTURE

| Pollutant | Year | 4B1a | 4B1b | 4B3 | 4B4 | 4B6 | 4B8 | 4B9a | 4B9b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NH3 | 2007 | 31.70% | 5.90% | 5.90% |  | 6.30% | 23.70% | 10.70% | 4.00% |
|  | 2008 | 27.20% | 6.10% | 6.70% |  | 6.50% | 24.00% | 11.70% | 4.60% |
| NMVOC | 2007 | 5.40% |  |  |  |  | 6.80% | 3.10% |  |
|  | 2008 | 5.00% |  |  |  |  | 6.10% | 3.00% |  |
| PM10 | 2007 |  |  |  |  |  | 2.60% |  | 1.50% |
|  | 2008 |  |  |  |  |  | 2.20% |  | 1.40% |

Table 14 – EMISSION FACTORS – AGRICULTURE

| Category | Pollutant | Emission Factor | Unit | Source (EMEP/EEA Guidebook) |
| --- | --- | --- | --- | --- |
| 4.B.01.a | NH3 | 39.3 | kg/caput | Table A5-1 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on slurry |
| 4.B.01.a | NH3 | 28.7 | kg/caput | Table B–2 Tier 1 emission factors for source category 4.B.1.a – Dairy Cattle on solid |
| 4.B.01.a | NMVOC | 13.6 | kg/caput | Table A5-1 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on slurry |
| 4.B.01.a | NMVOC | 13.6 | kg/caput | Table B–2 Tier 1 emission factors for source category 4.B.1.a – Dairy Cattle on solid |
| 4.B.01.a | NOX | 0.154 | kg/caput | Table B-2 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on solid |
| 4.B.01.a | NOX | 0.007 | kg/caput | Table B-1 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on slurry |
| 4.B.01.a | PM10 | 0.36 | kg/caput | Table B-1 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on slurry |
| 4.B.01.a | PM10 | 0.36 | kg/caput | Table B-2 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on solid |
| 4.B.01.a | PM25 | 0.23 | kg/caput | Table B-2 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on solid |
| 4.B.01.a | PM25 | 0.23 | kg/caput | Table B-1 Tier 1 emission factors for source category 4.B.1.a Dairy cattle on slurry |
| 4.B.01.b | NH3 | 9.2 | kg/caput | Table B–4 Tier 1 emission factors for source category 4.B.1.b Other cattle on solid |
| 4.B.01.b | NH3 | 13.4 | kg/caput | Table B–3 Tier 1 emission factors for source category 4.B.1.b Other cattle on solid |
| 4.B.01.b | NMVOC | 7.4 | kg/caput | Table B–3Tier 1 emission factors for source category 4.B.1.b Other cattle on solid |
| 4.B.01.b | NMVOC | 7.4 | kg/caput | Table B–4 Tier 1 emission factors for source category 4.B.1.b Other cattle on solid |
| 4.B.01.b | NOX | 0.094 | kg/caput | Table B-4 Tier 1 emission factors for source category 4.B.1.a other cattle on solid |
| 4.B.01.b | NOX | 0.002 | kg/caput | Table B-3 Tier 1 emission factors for source category 4.B.1.a other cattle on slurry |
| 4.B.01.b | PM10 | 0.24 | kg/caput | Table B-3 Tier 1 emission factors for source category 4.B.1.a other cattle on slurry |
| 4.B.01.b | PM10 | 0.24 | kg/caput | Table B-4 Tier 1 emission factors for source category 4.B.1.a other cattle on solid |
| 4.B.01.b | PM25 | 0.16 | kg/caput | Table B-3 Tier 1 emission factors for source category 4.B.1.a other cattle on slurry |
| 4.B.01.b | PM25 | 0.16 | kg/caput | Table B-4 Tier 1 emission factors for source category 4.B.1.a other cattle on solid |
| 4.B.02 | NH3 | 9 | kg/caput | Table 3-1 Default Tier 1 EF (EFNH3) for calculation of NH3 emissions from manure |
| 4.B.03 | NH3 | 1.4 | kg/caput | Table B–6 Tier 1 emission factors for source category 4.B.3 and 4.B.4 Sheep and goats |
| 4.B.03 | NMVOC | 0.2 | kg/caput | Table B–6 Tier 1 emission factors for source category 4.B.3 and 4.B.4 Sheep and goats |
| 4.B.03 | NOX | 0.005 | kg/caput | Table B–6 Tier 1 emission factors for source category 4.B.3 and 4.B.4 Sheep and goats on solid |
| 4.B.04 | NH3 | 1.4 | kg/caput | Table B–6 Tier 1 emission factors for source category 4.B.3 and 4.B.4 Sheep and goats |
| 4.B.04 | NMVOC | 0.2 | kg/caput | Table B–6 Tier 1 emission factors for source category 4.B.3 and 4.B.4 Sheep and goats |
| 4.B.04 | NOX | 0.005 | kg/caput | Table B–6 Tier 1 emission factors for source category 4.B.3 and 4.B.4 Sheep and goats on solid |
| 4.B.06 | NH3 | 14.8 | kg/caput | Table 3-1 Default Tier 1 EF (EFNH3) for calculation of NH3 emissions from manure |
| 4.B.06 | NOX | 0.131 | kg/caput | Table B–7 Tier 1 emission factors for source category 4.B.6 and 4.B.7 Horses and mules on solid |
| 4.B.06 | PM10 | 0.18 | kg/caput | Table B–7 Tier 1 emission factors for source category 4.B.6 and 4.B.7 Horses and mules on solid |
| 4.B.06 | PM25 | 0.12 | kg/caput | Table B–7 Tier 1 emission factors for source category 4.B.6 and 4.B.7 Horses and mules on solid |
| 4.B.08 | NH3 | 15.8 | kg/caput | Table B–8 Tier 1 emission factors for source category 4.B.8 Fattening pigs on slurry |
| 4.B.08 | NH3 | 6.7 | kg/caput | Table B–8 Tier 1 emission factors for source category 4.B.8 Fattening pigs on slurry |
| 4.B.08 | NMVOC | 13.3 | kg/caput | Table B–8 Tier 1 emission factors for source category 4.B.8 Fattening pigs on slurry |
| 4.B.08 | NMVOC | 3.9 | kg/caput | Table B–8 Tier 1 emission factors for source category 4.B.8 Fattening pigs on slurry |
| 4.B.08 | NOX | 0.004 | kg/caput | Table B–10 Tier 1 emission factors for source category 4.B.8 Sows on slurry |
| 4.B.08 | NOX | 0.045 | kg/caput | Table B–9 Tier 1 emission factors for source category 4.B.8 Fattening pigs on solid |
| 4.B.08 | PM10 | 0.5 | kg/caput | Table B–9 Tier 1 emission factors for source category 4.B.8 Fattening pigs on solid |
| 4.B.08 | PM10 | 0.58 | kg/caput | Table B–10 Tier 1 emission factors for source category 4.B.8 Sows on slurry |
| 4.B.08 | PM25 | 0.08 | kg/caput | Table B–9 Tier 1 emission factors for source category 4.B.8 Fattening pigs on solid |
| 4.B.08 | PM25 | 0.09 | kg/caput | Table B–10 Tier 1 emission factors for source category 4.B.8 Sows on slurry |
| 4.B.09.a | NH3 | 0.48 | kg/caput | Table B–13 Tier 1 emission factors for source category 4.B.9.a Laying hens |
| 4.B.09.a | NMVOC | 0.3 | kg/caput | Table B–13 Tier 1 emission factors for source category 4.B.9.a Laying hens |
| 4.B.09.a | NOX | 0.0001 | kg/caput | Table B–12 Tier 1 emission factors for source category 4.B.9.a Laying hens on slurry |
| 4.B.09.a | PM10 | 0.017 | kg/caput | Table B–12 Tier 1 emission factors for source category 4.B.9.a Laying hens on slurry |
| 4.B.09.a | PM25 | 0.002 | kg/caput | Table B–12 Tier 1 emission factors for source category 4.B.9.a Laying hens on slurry |
| 4.B.09.b | NH3 | 0.22 | kg/caput | Table B–14 Tier 1 emission factors for source category 4.B.9.b Broilers |
| 4.B.09.b | NMVOC | 0.1 | kg/caput | Table B–14 Tier 1 emission factors for source category 4.B.9.b Broilers |
| 4.B.09.b | NOX | 0.001 | kg/caput | Table B–14 Tier 1 emission factors for source category 4.B.9.b Broilers |
| 4.B.09.b | PM10 | 0.052 | kg/caput | Table B–14 Tier 1 emission factors for source category 4.B.9.b Broilers |
| 4.B.09.b | PM25 | 0.007 | kg/caput | Table B–14 Tier 1 emission factors for source category 4.B.9.b Broilers |
| 4.D.1.a | NH3 | 9 | kg/t | Table 3-2 Emission factors for total NH3 emissions from soils due to N fertiliser - assuming a temp = 10 grd |
| 4.D.1.a | NH3 | 17 | kg/t | Table 3-2 Emission factors for total NH3 emissions from soils due to N fertiliser - assuming a temp = 10 grd |
| 4.D.1.a | NH3 | 9 | kg/t | Table 3-2 Emission factors for total NH3 emissions from soils due to N fertiliser - assuming a temp = 10 grd |
| 4.D.1.c | NMVOC | 6 | mg/t | Table 3-1 Tier 1 emission factors for source category 4.D crop production and agricultural soils |

Table 15 – ACTIVITY DATA – AGRICULTURE

| Category | NAME | 2007 | 2008 | Unit |
| --- | --- | --- | --- | --- |
| 4.B.01.a | Dairy cattle | 1732000 | 1638534 | caput |
| 4.B.01.b | Non-dairy cattle | 1087000 | 1045077 | caput |
| 4.B.02 | Buffalo | 0 | 0 | caput |
| 4.B.03 | Sheep | 8469195 | 8881582 | caput |
| 4.B.04 | Goats | 865070 | 898307 | caput |
| 4.B.06 | Horses | 862000 | 819513 | caput |
| 4.B.08 | Swine | 6564907 | 6173682 | caput |
| 4.B.09.a | Laying hens | 45208000 | 45529000 | caput |
| 4.B.09.b | Broilers | 36828000 | 38844000 | caput |
| 4.D.1 | Direct soil emissions | 387000 | 398000 | t |
| 4.D.1.a | Synthetic fertilizers | 387000 | 398000 | t |

Table 16 – EMISSION DATA FROM AGRICULTURE SECTOR

| NFR | Main Pollutants |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nox (as NO2) |  | NMVOC |  | SOx (as SO2) |  | NH3 |  |
|  | Gg NO2 |  | Gg |  | Gg SO2 |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 4 B 1 a Cattle dairy | 0.202 | 0.065 | 22.284 | 23.555 | NA | NA | 50.673 | 64.212 |
| 4 B 1 b Cattle non-dairy | 0.058 | 0.06 | 7.734 | 8.044 | NA | NA | 11.458 | 11.918 |
| 4 B 2 Buffalo | NA | NA | NA | NA | NA | NA | IE | IE |
| 4 B 3 Sheep | 0.044 | 0.042 | 1.776 | 1.694 | NA | NA | 12.434 | 11.857 |
| 4 B 4 Goats | 0.004 | 0.004 | 0.18 | 0.173 | NA | NA | 1.258 | 1.211 |
| 4 B 6 Horses | 0.107 | 0.113 | NA | NA | NA | NA | 12.129 | 12.758 |
| 4 B 7 Mules and asses | NA | NA | NE | NE | NA | NA | NE | NE |
| 4 B 8 Swine | 0.262 | 0.277 | 27.616 | 29.762 | NA | NA | 44.789 | 48.011 |
| 4 B 9 a Laying hens | 0.005 | 0.004 | 13.659 | 13.562 | NA | NA | 21.854 | 21.7 |
| 4 B 9 b Broilers | 0.039 | 0.037 | 3.884 | 3.683 | NA | NA | 8.546 | 8.102 |
| 4 D 1 a Synthetic N-fertilizers | NA | NA | NA | NA | NA | NA | 4.398 | 4.307 |

| NFR | PM2.5 |  | PM10 |  | TSP |  | CO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gg |  | Gg |  | Gg |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 4 B 1 a Cattle dairy | 0.376863 | 0.398 | 0.589872 | 0.623 | NA | NA | NA | NA |
| 4 B 1 b Cattle non-dairy | 0.167212 | 0.174 | 0.250818 | 0.261 | NA | NA | NA | NA |
| 4 B 2 Buffalo | NA | NA | NA | NA | NA | NA | NA | NA |
| 4 B 3 Sheep | NA | NA | NA | NA | NA | NA | NA | NA |
| 4 B 4 Goats | NA | NA | NA | NA | NA | NA | NA | NA |
| 4 B 6 Horses | 0.098342 | 0.10344 | 0.147512 | 0.155 | NA | NA | NA | NA |
| 4 B 7 Mules and asses | NA | NA | NA | NA | NA | NA | NA | NA |
| 4 B 8 Swine | 0.497659 | 0.53 | 3.116956 | 3.318 | NA | NA | NA | NA |
| 4 B 9 a Laying hens | 0.091058 | 0.09 | 0.773993 | 0.769 | NA | NA | NA | NA |
| 4 B 9 b Broilers | 0.271908 | 0.258 | 2.019888 | 1.915 | NA | NA | NA | NA |

2.5. WASTE (NFR Sector 6)

      - 6.A. Solid Waste Disposal on Land

      - 6.A Solid Waste Disposal on Land

      - 6.B. Waste water handling

      - 6.B Waste water handling

      - 6.C. Waste Incineration

      - 6.C.a Clinical Waste Incineration

      - 6.C.b Industrial Waste Incineration

      - 6.C.e Small scale waste burning

      - 6.D. Other Waste

      - 6.D Other waste

The activity data for estimating of the emissions that came from solid waste disposal, industrial waste incineration and compost production was provided by national database from NEPA.

The emissions from the incineration of agricultural waste in open air were calculated considering a 25 kilos/hectare of agricultural residual waste (the cultivated areas with cereals, vinery and orchards are provided by NIS). Part of the data used for this sector is preliminary because the statistical survey on waste sector for 2008 will be finished by the end of march, 2010.

The incineration of sludge which came from waste water treatment is not currently practiced in Romania.

Data for estimating of emissions from waste-water handling are provided by Romanian Waters National Administration.

All resulting emissions from the activities included in product use sector, emission factors, activity data and share from national total are presented in table 17, 18, 19 and 20.

Table 17 - Key Source Analysis – WASTE

| Pollutant | Year | 6B | 6Ca | 6Cb |
| --- | --- | --- | --- | --- |
| NH3 | 2007 | 7.60% |  |  |
|  | 2008 | 8.30% |  |  |
| DIOX | 2007 |  | 18.30% | 3.00% |
|  | 2008 |  | 11.20% | 6.40% |
| HCB | 2007 |  | 47.50% | <1.00% |
|  | 2008 |  | 32.90% | 3.20% |

Table 18 – EMISSION FACTORS – WASTE

| Category | Pollutant | Emission Factor | Unit | Source (EMEP/EEA guidebook) |
| --- | --- | --- | --- | --- |
| 6.B | NH3 | 1.6 | kg/caput | Table 3-2 Tier 2 emission factors for source category 6.B Waste water handling, latrines |
| 6.B | NMVOC | 15 | g/1000 m3 | Table 3-1 Tier 1 emission factors for source category 6.B Waste water handling |
| 6.C.a | As | 1.3 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | Cd | 1 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | CO | 2.8 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | Cr | 4.7 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | Cu | 2.6 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | DIOX | 3000 | µg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | HCB | 0.1 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | Hg | 8 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | Ni | 0.4 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | NMVOC | 0.7 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | NOX | 1.4 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | PAH | 0.04 | mg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | Pb | 13 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | PCBs | 0.02 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | SO2 | 1.4 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.a | TSP | 0.5 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.a Clinical waste incineration |
| 6.C.b | As | 0.016 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | Cd | 0.1 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | CO | 0.07 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | Cr | 0.3 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | Cu | 3 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | DIOX | 350 | µg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | HCB | 0.002 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | Hg | 0.056 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | Ni | 0.14 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | NMVOC | 7.4 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | NOX | 0.87 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | PAH | 0.02 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | Pb | 1.3 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | PM10 | 0.007 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | PM25 | 0.004 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | SO2 | 0.047 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | TSP | 0.01 | kg/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.b | Zn | 21 | g/Mg waste | Table 3-1 Tier 1 emission factors for source category 6.C.b Industrial waste incineration |
| 6.C.e | DIOX | 10 | µg/Mg product | Table 3-1 Tier 1 emission factors for source category 6.C.e Small-scale waste burning |
| 6.C.e | NH3 | 1.9 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 6.C.e Small-scale waste burning |
| 6.C.e | NMVOC | 2 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 6.C.e Small-scale waste burning |
| 6.C.e | PAH | 100 | g/Mg product | Table 3-1 Tier 1 emission factors for source category 6.C.e Small-scale waste burning |
| 6.C.e | PM10 | 0.979 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 6.C.e Small-scale waste burning |
| 6.C.e | PM25 | 0.839 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 6.C.e Small-scale waste burning |
| 6.C.e | TSP | 1.4 | kg/Mg product | Table 3-1 Tier 1 emission factors for source category 6.C.e Small-scale waste burning |
| 6.D | NH3 | 0.24 | kg/Mg product | Table 3-1 Tier 2 emission factors for source category 6.D Other waste, compost production |

Table 19 – ACTIVITY DATA – WASTE

| Category | NAME | 2007 | 2008 | Unit |
| --- | --- | --- | --- | --- |
| 6.A.1 | Managed waste disposal on land | 254.37768 | 256.41265 | Gg product |
| 6.B | Waste-water handling | 2210285 | 1868832 | 1000 m3 |
| 6.B | Waste-water handling | 9656357 | 9669114 | caput |
| 6.C.a | Clinical Waste Incineration | 9997 | 6442 | Mg waste |
| 6.C.b | Industrial Waste Incineration | 14118 | 31633 | Mg waste |
| 6.C.e | Small Scale Waste Burning | 128230 | 130276 | Mg product |
| 6.D | Compost production | 2344 | 2360 | Mg product |

Table 20 – EMISSION DATA FROM WASTE SECTOR

| NFR | Main Pollutants |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nox (as NO2) |  | NMVOC |  | SOx (as SO2) |  | NH3 |  |
|  | Gg NO2 |  | Gg |  | Gg SO2 |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 6 A Solid waste disposal on land | NA | NA | 3.377 | 3.35 | NA | NA | NA | NA |
| 6 B Waste-water handling | NA | NA | 0.028 | 0.033 | NA | NA | 15.471 | 15.45 |
| 6 C a Clinical waste incineration | 0.009 | 0.014 | 0.005 | 0.007 | 0.009 | 0.014 | NA | NA |
| 6 C b Industrial waste incineration | 0.028 | 0.012 | 0.234 | 0.104 | 0.001 | 0.0007 | NA | NA |
| 6 C c Municipal waste incineration | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 C d Cremation | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 C e Small scale waste burning | NA | NA | 0.261 | 0.256 | NA | NA | 0.248 | 0.244 |
| 6 D Other waste | NA | NA | NA | NA | NA | NA | 0.001 | 0.00056 |

| NFR | PM2.5 |  | PM10 |  | TSP |  | CO |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Gg |  | Gg |  | Gg |  | Gg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 6 A Solid waste disposal on land | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 B Waste-water handling | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 C a Clinical waste incineration | NA | NA | NA | NA | 0.003221 | 0.005 | 0.018038 | 0.028 |
| 6 C b Industrial waste incineration | 0.000127 | 0.00006 | 0.000221 | 0.00009 | 0.000316 | 0.0001 | 0.002214 | 0.00098 |
| 6 C c Municipal waste incineration | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 C d Cremation | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 C e Small scale waste burning | 0.109 | 0.108 | 0.12754 | 0.126 | 0.182386 | 0.18 | NA | NA |
| 6 D Other waste | NA | NA | NA | NA | NA | NA | NA | NA |

| NFR | Pb |  | Cd |  | Hg |  | As |  | Cr |  | Cu |  | Ni |  | Zn |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  | Mg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 6 A Solid waste disposal on land | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 B Waste-water handling | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 C a Clinical waste incineration | 0.084 | 0.13 | 0.006 | 0.01 | 0.0052 | 0.08 | 0.008 | 0.013 | 0.03 | 0.047 | 0.017 | 0.026 | 0.0025 | 0.004 | NA | NA |
| 6 C b Industrial waste incineration | 0.041 | 0.018 | 0.003 | 0.0014 | 0.002 | 0.0008 | 0.0005 | 0.0002 | 0.01 | 0.004 | 0.095 | 0.042 | 0.0044 | 0.002 | 0.664 | 0.296 |
| 6 C c Municipal waste incineration | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 C e Small scale waste burning | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 6 D Other waste | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

| NFR | PCDD/ PCDF |  | HCB |  | PCBs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | g I-Teq |  | kg |  | kg |  |
|  | 2008 | 2007 | 2008 | 2007 | 2008 | 2007 |
| 6 A Solid waste disposal on land | NA | NA | NA | NA | NA | NA |
| 6 B Waste-water handling | NA | NA | NA | NA | NA | NA |
| 6 C a Clinical waste incineration | 19.326 | 29.991 | 0.6442 | 0.999 | 0.129 | 0.199 |
| 6 C b Industrial waste incineration | 11.072 | 4.941 | 0.063 | 0.028 | NA | NA |
| 6 C c Municipal waste incineration | NA | NA | NA | NA | NA | NA |
| 6 C e Small scale waste burning | 1.303 | 1.2823 | NA | NA | NA | NA |
| 6 D Other waste | NA | NA | NA | NA | NA | NA |

3. THE RESULTS ANALYSYS AND THE QA/QC ACTIVITIES

Romania has not yet implemented a formal QA/QC procedure, including verification plan, for the national emissions inventory. However, several checks are routinely carried out to eliminate possible errors.

The first draft of the inventory is usually produced 1 months after the end of the given year depending primarily on the availability of required activity data. During the following several weeks, extensive checks and other activities for completing the database are done followed by consultations with data providers.

The calculated emissions figures for a given year, are compared to the respective figures from previous years (time series), and outliers are scrutinized in more detail. After the checking stage is completed, the final inventory is prepared together with the draft version of the accompanying reports.

After this step, the inventory is submitted to the CLRTAP Secretariat and E.E.A., in the requested NFR format.