2008 QUESTIONNAIRE

VII.	THE 1999 GOTHENBURG PROTOCOL

- The questions in this section are based on the reporting obligation of Parties in accordance with article 7, paragraph 1 (a), and enable Parties to provide information on the implementation of the obligations under articles 3.2, 3.3, 3.5, 3.8 and 6.1(a) of the Protocol. Any Party that applies different emission reduction strategies that achieve equivalent overall emission levels for all source categories together, in accordance with article 3.2 and 3.3 and article 7(a)(i), may go directly to question 47. By virtue of article 3.10(b), questions 51–58 do not apply to the United States.

- They refer to the following Parties to the Protocol: Belgium, Bulgaria, Cyprus, the Czech Republic, Denmark, Finland, France, Germany, Hungary, Latvia, Lithuania, Luxembourg, the Netherlands, Norway, Portugal, Romania, Slovakia, Slovenia, Spain, Sweden, Switzerland, the United Kingdom, the United States and the European Community.

- Question 39: With reference to article 6, paragraph 1(a), please provide details of the supporting strategies, policies and programmes your country has adopted to facilitate the implementation of its obligations under article 3 of the Protocol. Where pollutant specific policies, strategies or programmes are used, please make a clear distinction between (a) sulphur; (b) NOx; (c) VOCs and (d) ammonia.

Answer 39:

National Environmental Action Plan (NEAP) 2005-2012 (OJ RS, No. 2/2006) was adopted by the National Assembly at the end of 2005. It is a continuation of the environmental policy focused on the implementation of legislation. Policy general principles and strategic orientation comprise: support of effective implementation of the adopted legislation, integration of environmental issues in spatial planning and in sectoral policies, promotion of environmental technologies and sustainable production, consumption and use of natural resources, promotion of environmental economic policy (including green public finances), rehabilitation of degraded areas and others.

Air quality issues are addressed under NEAP priority quality of life. The programme’s first priority is to reduce greenhouse gas emissions, which will, by reducing use of fossil fuels, indirectly contribute to significant emission reductions of air pollutants. NEAP envisages finances from the national budget for modernisation of ambient air quality monitoring and for action programmes for improving air quality, while emission reductions from stationary sources are carried out by plants owners and operators (most of large combustion plants are operated by state owned companies).

NEAP also envisages specific action plans and measures. The following are relevant for air quality management:

(i) 	Action programmes for improving air quality to achieve ambient air quality limit values; address SO2 and PM10/2.5 in polluted areas (in preparation);

(ii)	Operational programme for complying with national emission ceilings for atmospheric pollutants - Revision on Operational programme for complying with national emission ceilings for atmospheric pollutants from 2005 (adopted by the Government in January 2007); includes policy and measures for reducing emissions of SO2, NOx, VOCs, NH3, and PM10/2.5 – related to Directive 2001/81/EC on national emission ceilings for certain atmospheric pollutants and the 1999 Gothenburg protocol;

    - Operational programme for reducing atmospheric emissions from large combustion plants (adopted by the Government in February 2006); includes measures for the Slovenian large combustion plants (over 50 MW) to achieve emission limitations of  SO2, NOx, CO and PM for the period 2004-2020.

- Question 40: With reference to article 3, paragraph 2, and annex IV, paragraph 9, specify the limit values for sulphur emissions applied to each new stationary source (construction or substantial modification commenced after 17 May 2006) in your country within stationary source categories identified in that annex. If you have applied alternative emission reduction strategies, please go to question 49. Please complete the table below.

Answer 40:

SO2 emission limit values for large combustion plants built after 27 November 2002 are laid down in the Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007). They are given in Table 19 ( Legend: SF-solid fuel, LF-liquid fuel).

Table 19: Question 40

| Stationary source category1/ | O2 in flue gas (%) | Limit value2/ | # Alternative: Desulphurization rate for domestic solid fuel | # National legislation |
| --- | --- | --- | --- | --- |
| 1. Solid and liquid fuels 50-100 MWth | 6% (SF), 3% (LF) | 850 mg SO2/Nm3(SF, LF); 200 mgSO2/Nm3 (biomass) | 92% for 50-300 MWth (SF) | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 2. Solid and liquid fuels 100-300 MWth | 6% (SF), 3% (LF) | 200 mg SO2/Nm3 (SF), 400-200 mg SO2/Nm3 (LF) | 92% for 50-300 MWth (SF) | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 3. Solid and liquid fuels >300 MWth | 6% (SF), 3% (LF) | 200 mg SO2/Nm3 (SF, LF) | 95 % (SF); emissions of SO2 shall not exceed 400 mg/m3 | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 4. Gaseous fuels | 3% | 35 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 5. Liquified gas | 3% | 5 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 6. Low-calorific-value gases (e.g. gasification of refinery residues or combustion of coke oven gas) | 3% | 400 mg SO2/Nm3 |  | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 7. Blast furnace gas | 3% | 200 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 8. Combustion plant in refineries >50 MWth total refinery capacity (average of all new installations) | 3% | 600 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |

1/ For new stationary source, see article 1 (Definitions); for further information on stationary source categories see annex IV (paras. 9–12).

2/ Different limit values for different types of fuels may be provided, e.g. biomass, peat, etc.

- Question 41: With reference to article 3, paragraph 3 and annex IV, paragraph 9, please provide details of the limit values for sulphur emissions applied in your country to each existing stationary source (construction commenced on or before 17 May 2006) within a stationary source category identified in that annex, in so far as it is technically and economically feasible and taking into consideration the costs and advantages. If you have applied alternative emission reduction strategies, please go to question 49. Please complete the table below.

Answer 41:

SO2 emission limit values for large combustion plants built before 27 November 2002 are laid down in the Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007). They are given in Table 20. For large combustion plants built after 27 November 2002 SO2 emission limit values from Table 19 apply.

Table 20: Question 41

| Stationary source category1/ | O2 in flue gas (%) | Limit value2/ | # Alternative: Desulphurization rate for domestic solid fuel (%) | # National legislation |
| --- | --- | --- | --- | --- |
| 1. Solid fuels 50-100 MWth 1/ | 6% | 2000 mg SO2/Nm3 | 60% | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 2. Solid fuels 100-500 MWth 2/ | 6% | 2000-400 mg SO2/Nm3 (linear decrease) | 75% for 100-300 MWth, 90% for 300-500 MWth | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 3. Solid fuels >500 MWth | 6% | 400 mg SO2/Nm3 | 92-94%; emissions of SO2 shall not exceed 400 mg/m3 | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 4. Liquid fuels 50-300 MWth | 3% | 1700 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 5. Liquid fuels 300-500 MWth | 3% | 1700-400 mg SO2/Nm3 (linear decrease) | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 6. Liquid fuels >500 MWth | 3% | 400 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 7. Gaseous fuels | 3% | 35 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 8. Liquified gas | 3% | 5 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 9. Low-calorific-value gases (e.g. gasification of refinery residues or combustion of coke oven gas) | 3% | 800 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 10. Blast furnace gas | 3% | 800 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 11. Combustion plant in refineries (average of all existing installations) | 3% | 1000 mg SO2/Nm3 | n.a. | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |

1/ If you apply, as an alternative, a desulphurization rate, the category should be 50-150 MWth.

2/ If you apply, as an alternative, a desulphurization rate, the category should be 150-500 MWth.

- Question 42: With reference to article 3, paragraph 2, and annex IV, paragraphs 11 and 12, please provide details of the limit values currently applied in your country for sulphur recovery for new and existing Claus plants and sulphur dioxide emissions from new and existing installations for TiO2 production.

Answer 42:

Limit values applied for sulphur recovery for new Claus plants are regulated in Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 31/2007) as follows:

(i) production capacity up to 20 t of sulphur per day: sulphur recovery >97 %,

(ii) production capacity 20-50 t of sulphur per day: sulphur recovery >98 %;

(iii) production capacity >50 t of sulphur per day: sulphur recovery > 99.8 %.

For existing Claus plants emission limit value of 500 mg SO2/m3 applies until 31 Dec. 2011, according to the Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 73/94, 68/96, 109/01). However, there are presently no Claus plants in operation in Slovenia.

For new and existing installations on TiO2 production, limit value for discharges arising from digestion and calcinations steps is 10 kg of SO2 in flue gas per 1t of TiO2 produced. Slovenian legislation: Decree on the quantity of waste from titanium dioxide industry discharged into water and on the emission of substances into the atmosphere from the titanium dioxide industry (OJ RS, No. 64/2000).

- Question 43: With reference to article 3, paragraph 2, and annex IV, paragraph 10, please provide details of the limit value for sulphur content of gas oil that is currently applied in your country.

Answer 43:

National standards for the sulphur content of gas oil are laid down in the Decree on the physical and chemical properties of liquid fuels (OJ RS, No. 63/2006, 92/2006). Limit value (LV) for sulphur content of gas oil and extra light gas oil is 0.10% by weight (until 31 December 2007: 0.20% by weight), LV for sulphur content of diesel fuel is 50 mg/kg (10 mg/kg after 1 January 2009), LV for sulphur content of heating oil (i.e. heavy fuel oil) is 1% by weight, and LV for gas oil for vessels is 0.10% by weight (until 31 December 2007: 0.20% by weight).

- Question 44: With reference to article 3, paragraph 2 and annex V, please provide details of the limit values for NOx emissions applied to each new stationary source (construction or substantial modification commenced after 17 May 2006) within stationary source categories identified in that annex. If you have applied alternative emission reduction strategies, please go to question 49. Please complete the table below.

Answer 44:

According to the Environmental Protection Act (OJ RS, No. 2/2006) and Decree on activities and installations causing large-scale environmental pollution (OJ RS, No. 97/2004, 71/2007, 122/2007) all stationary sources (i.e. industrial installations) listed in Annex V of the Protocol have to obtain for their operation an environmental permit. Slovenian competent authority for issuing environmental permits is the Environmental Agency (EARS). Basic requirements for prevention and control of emissions in permits are: required application of best available technique (as defined in the BREF documents of the European IPPC Bureau in Sevilla), regulated emission limit values (from the national legislation) and regulated control measures (from the national legislation).  This applies to both existing and new installations.

National legislation on NOx emission limit values for stationary sources listed in Annex V of the Protocol:

- Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 31/2007),

- Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007),

- Decree on the emission of substances into the atmosphere from stationary gas turbines with thermal input power under 50 MW and from stationary internal combustion engines (OJ RS, No. 34/2007, 81/2007),

- Decree on the emission of substances into the atmosphere from installations for the production of cement (OJ RS, No. 34/2007).

NOx emission limit values for new stationary sources are given in Table 21.

Table 21: Question 44

| Stationary source category 1/ | Limit value (mg/Nm3) | National legislation |
| --- | --- | --- |
| A. Boilers |  | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 1. Solid fuels 50-100 MWth | 400 mg NOx/Nm3 |  |
| 2. Solid fuels 100-300 MWth | 200 mg NOx/Nm3 (400 biomass) |  |
| 3. Solid fuels >300 MWth | 200 mg NOx/Nm3 |  |
| 4. Liquid fuels 50-100 MWth | 400 mg NOx/Nm3 |  |
| 5. Liquid fuels 100-300 MWth | 200 mg NOx/Nm3 |  |
| 6. Liquid fuels>300 MWth | 200 mg NOx/Nm3 |  |
| 7. Natural gas 50-300 MWth | 150 mg NOx/Nm3 |  |
| 8. Natural gas >300 MWth | 100 mg NOx/Nm3 |  |
| 9. Other gases | 200 mg NOx/Nm3 |  |
| B. Onshore combustion turbines >50 MWth |  | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 1. Natural gas | 50 mg NOx/Nm3 |  |
| 2. Liquid fuels | 120 mg NOx/Nm3 |  |
| C. Cement production | (10% O2) | Decree on the emission of substances into the atmosphere from installations for the production of cement (OJ RS, No. 34/2007) |
| 1. Dry kilns | 500 mg NOx/Nm3 |  |
| 2. Other kilns | 500 mg NOx/Nm3 |  |
| D. Stationary engines |  | Decree on the emission of substances into the atmosphere from stationary gas turbines with thermal input power under 50 MW and from stationary internal combustion engines (OJ RS, No. 34/2007, 81/2007) |
| 1. Spark ignition engines, 4-stroke, >1 MWth: Lean-burn engines | 250 mg NOx/Nm3 |  |
| 2. All other spark-ignition engines | 500 mg NOx/Nm3 |  |
| 3. Compression ignition (=Diesel) engines, >5 MWth: natural gas (jet ignition engines) | 500 mg NOx/Nm3 |  |
| 4. Compression ignition (=Diesel) engines, >5 MWth: heavy fuel oil | 500 mg NOx/Nm3 |  |
| 5. Compression ignition (=Diesel) engines, >5 MWth: diesel oil or gas oil | 500 mg NOx/Nm3 |  |
| E. Sinter plants | 400 mg NOx/Nm3 | Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 31/2007) |
| F. Nitric acid production, excl. acid concentration units | 200 mg NOx/Nm3 | Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 31/2007) |

- Question 45: With reference to article 3, paragraph 3, and annex V, please provide details of the limit values for NOx emissions applied in your country to each existing stationary source (construction commenced on or before 17 May 2006) within a stationary source category identified in that annex, in so far as it is technically and economically feasible and taking into consideration the costs and advantages. If you have applied alternative emission reduction strategies, please go to question 49. Please complete the table below.

Answer 45:

For the national legislation on NOx emission limit values and their application in environmental permits please see text in A. 44. In addition to the (new) Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 31/2007), the (old) Decree (OJ RS, No. 73/94, 68/96, 109/01) applies for emission limit values of some existing stationary sources,

NOx emission limit values for existing stationary sources are given in Table 22.

Table 22: Question 45

| Stationary source category 1/ | Limit value (mg /Nm3) | National legislation |
| --- | --- | --- |
| A. Boilers |  | Decree on emission limit values discharged into the atmosphere from large combustion plants (OJ RS, No. 73/2005, 92/2007) |
| 1. Solid fuels 50-100 MWth | 600 mg NOx/Nm3 |  |
| 2. Solid fuels 100-300 MWth | 600 mg NOx/Nm3 |  |
| 3. Solid fuels >300 MWth | 600 mg NOx/Nm3 for 300-500 MWth. 200 mg NOx/Nm3 for P>500 MWth |  |
| 4. Liquid fuels 50-100 MWth | 450 mg NOx/Nm3 |  |
| 5. Liquid fuels 100-300 MWth | 450 mg NOx/Nm3 |  |
| 6. Liquid fuels>300 MWth | 450 mg NOx/Nm3 for 300-500 MWth, 400 mg NOx/Nm3 for P>500 MWth |  |
| 7. Natural gas 50-300 MWth | 300 mg NOx/Nm3 |  |
| 8. Natural gas >300 MWth | 300 mg NOx/Nm3 for 300-500 MWth, 200 mg NOx/Nm3 for P>500 MWth |  |
| 9. Other gases | Limit value same as for natural gas |  |
| B. Onshore combustion turbines >50Mwth |  | Decree on the emission of substances into the atmosphere from stationary gas turbines with thermal input power under 50 MW and from stationary internal combustion engines (OJ RS, No. 34/2007, 81/2007) |
| 1. Natural gas | 300 mg NOx/Nm3 |  |
| 2. Liquid fuels | 400 mg NOx/Nm3 |  |
| C. Cement production | (10% O2) | Decree on the emission of substances into the atmosphere from installations for the production of cement (OJ RS, No. 34/2007) |
| 1. Dry kilns | 1300 mg NOx/Nm3 |  |
| 2. Other kilns | 1300 mg NOx/Nm3 |  |
| D. Sinter plants | 400 mg NOx/Nm3 | Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 31/2007) and (old) Decree (OJ RS, No. 73/94, 68/96, 109/01) |
| E. Nitric acid production, excl. acid concentration units | 450 mg NOx/Nm3 | Decree on the emission of substances into the atmosphere from stationary sources of pollution (OJ RS, No. 31/2007) and (old) Decree (OJ RS, No. 73/94, 68/96, 109/01) |

- Question 46: With reference to article 3, paragraphs 2 and 3, and annex VI, please provide details of the limit values for VOCs emissions applied in your country to new stationary sources (construction or substantial modification commenced after 17 May 2006) for the stationary source category defined in Table I of that annex and to existing stationary sources (construction commenced on or before 17 May 2006), in so far as it is technically and economically feasible and taking into consideration the costs and advantages. If you have applied alternative emission reduction strategies, please go to question 49. Please complete the table below.

Answer 46:

Emission limit value of 35 g VOC/Nm3 for release from storage and distribution of petrol is laid down in the Decree on emissions of volatile organic compounds into the atmosphere from petrol terminals and service stations (OJ RS, No. 11/1999).

Table 23: Question 46

| Source category | Limit value (g VOC/Nm3) | National legislation |
| --- | --- | --- |
| Storage and distribution of petrol, excluding loading of seagoing ships: Vapour recovery unit serving storage and distribution facilities at refinery tank farms or terminals with petrol throughput of 5000 m3 annually | 35 g VOC/Nm3 (all facilities; i.e. no annual threshold for petrol throughput is set) | Decree on emissions of volatile organic compounds into the atmosphere from petrol terminals and service stations (OJ RS, No. 11/1999) |

- Question 47: With reference to article 3, paragraphs 2 and 3, and annex VI, please provide details of the limit values for VOCs emissions applied in your country to new stationary sources (construction or substantial modification commenced after 17 May 2006) for the stationary source categories defined in Tables II, V, VI,VIII, IX, X, XI, XII, XIV and XV of that annex and to existing stationary sources (construction commenced on or before 17 May 2006), in so far as it is technically and economically feasible and taking into consideration the costs and advantages. If you have applied alternative emission reduction strategies, please go to question  49. Please complete the table below.

Answer 47:

VOCs emission limit values for new and existing stationary sources are laid down in the Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007), which is in accordance with Directive 1999/13/EC.

Existing installation is an installation in operation since 24 May 2002 or an installation which is authorised or registered before 24 May 2002 provided that the installation is put into operation no later than 30 April 2003. Existing installations have to comply with the specified limit values from 31 October 2007 onwards.

Table 24: Question 47

| Source category and solvent consumption (Mg/year) | Limit value (mg C/Nm3) and relevant conditions / alternatives e.g. solvent reuse, process sub-part, % of solvent input | Limit value for fugitive emissions of non-methane VOCs (% of solvent input) and relevant conditions / alternatives e.g. process sub-part, kg solvent per unit of product | National legislation |
| --- | --- | --- | --- |
| 1. Adhesive coating (Annex VI, Table II) |  |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (a) Footwear manufacture >5 Mg/year 1/ | 25 g solvent per pair of shoes |  |  |
| (b) Other adhesive coating, excl. (a) |  |  |  |
| (i) 5-15 Mg/year (ii) >15 Mg/year | 5-15 Mg/year: 50 mg C/Nm3; 150 mg C/Nm3 (if techniques are used which allow reuse of recovered solvent); >15 Mg/year: 50 mg C/Nm3; 150 mg C/Nm3 (if techniques are used which allow reuse of recovered solvent); 20 mg C/Nm3 (with subsequent incineration) | 5-15 Mg/year: 25% >15 Mg/year: 20% |  |
| 2. Coating processes in various industrial sectors (Annex VI, Table V) |  |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (a) Other coating, incl metal, plastics, textile, fabric, foil and paper (excl. web screen printing for textiles) (i) 5-15 Mg/year | 100 mg C/Nm3 (emission limit value applies to coating application and drying processes) | 25%(metal and plastics) 20% (textile, fabric, foil and paper) |  |
| (ii) >15 Mg/year | 50 mg C/Nm3 (for drying processes); 75 mg C/Nm3 (for coating processes); 20 mg C/Nm3 (with subsequent incineration); 150 mg C/Nm3 (for installations which use techniques which allow reuse of recovered solvents) | 20% |  |
| (b) Wood coating (i) 5-15 Mg/year | 15-25 Mg/year: 100 mg C/Nm3 (emission limit applies to coating application and drying processes operated under contained conditions) | 25% |  |
| (ii) >15 Mg/year | >25 Mg/year: 50 mg C/Nm3 (for drying processes); 75 mg C/Nm3 (for coating processes); 20 mg C/Nm3 (with subsequent incineration) | 20% |  |
| 3. Coil coating > 25 Mg/year (Annex VI, Table VI) |  |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (i) New installations | 50 mg C/Nm3, 20 mg C/Nm3 (with subsequent incineration); 150 mg C/Nm3 (for installations which use techniques which allow reuse of recovered solvents) | 5% |  |
| (ii) Existing installations | 50 mg C/Nm3; 20 mg C/Nm3 (with subsequent incineration); 150 mg C/Nm3 (for installations which use techniques which allow reuse of recovered solvents) | 10% |  |
| 4. Manufacturing of coatings, varnishes, inks and adhesives (Annex VI, Table VIII) 100-1,000 Mg/year >1,000 Mg/year | 150 mg C/Nm3 | 100-1,000 Mg/year: 5% ; >1,000 Mg/year 3% | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| 5. Printing processes (Annex VI, Table IX) |  |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (a) Heat set web offset (i) 15-25 Mg/year | 100 mg C/Nm3 20 mg C/Nm3 (with subsequent incineration) | 30% |  |
| (ii) > 25 Mg/year | 20 mg C/Nm3 | 30% |  |
| (b) Publication rotogravure > 25 Mg/year |  |  |  |
| (i) New installations | 75 mg C/Nm3 | 10% |  |
| (ii) Existing installations | 75 mg C/Nm3 | 15% |  |
| (c) Other rotogravure, flexography, rotary screen printing, lamination and varnishing units (i) 15-25 Mg/year | 100 mg C/Nm3 20 mg C/Nm3 (with subsequent incineration) | 25% |  |
| (ii) >25 Mg/year | 100 mg C/Nm3 20 mg C/Nm3 (with subsequent incineration) | 20% |  |
| (e) Rotary screen printing on textiles, paperboard > 30 Mg/year | 100 mg C/Nm3 | 20% |  |
| 6. Manufacturing of pharmaceutical products >50 Mg/year (Annex VI, Table X) |  |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (i) New installations | 20 mg C/Nm3; 150 mg C/Nm3 (if techniques are used which allow reuse of recovered solvent) | 5% |  |
| (ii) Existing installations | 20 mg C/Nm3; 150 mg C/Nm3 (if techniques are used which allow reuse of recovered solvent) | 15% |  |
| 7. Conversion of natural or synthetic rubber > 15 Mg/year (Annex VI, Table XI) | 20 mg C/Nm3; 150 mg C/Nm3 (if techniques are used which allow reuse of recovered solvent) | 25% | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| 8. Surface cleaning (Annex VI, Table XII) |  |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (a) Using substances mentioned in para. 3(w) 1-5 Mg/year | 20 mg C/Nm3 | 15% |  |
| >5 Mg/year | 20 mg C/Nm3 | 10% |  |
| (b) Other surface cleaning 2-10 Mg/year | 75 mg C/Nm3 (limit values do not apply if content of all cleaning material used does not exceed 30% w/w) | 20% |  |
| >10 Mg/year | 75 mg C/Nm3 | 15% |  |
| 9. Vehicle refinishing >0.5 Mg/year (Annex VI, Table XIV) | 50 mg C/Nm3 (applies to 15 minute averages) | 25% | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| 10. Impregnation of wooden surfaces >25 Mg/year (Annex VI, Table XV) | 100 mg C/Nm3 | 45% | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |

1/ Specify limit value in g solvent per pair

- Question 48: With reference to article 3, paragraphs 2 and 3, and annex VI, please provide details of the limit values for VOCs emissions applied in your country to new stationary sources (construction or substantial modification commenced after 17 May 2006) for the stationary source categories defined in Tables III, IV, VII and XIII of that annex and to existing stationary sources (construction commenced on or before 17 May 2006), in so far as it is technically and economically feasible and taking into consideration the costs and advantages. If you have applied alternative emission reduction strategies, please go to question 49. Please complete the table below.

Answer 48:

VOCs emission limit values for new and existing stationary sources are laid down in the Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007), which is in accordance with Directive 1999/13/EC.

Existing installation is an installation in operation since 24 May 2002 or an installation which is authorised or registered before 24 May 2002 provided that the installation is put into operation no later than 30 April 2003. Existing installations have to comply with the specified limit values from 31 October 2007 onwards.

Table 25: Question 48

| Capacity, technique, further specification and solvent consumption | Limit value for total emissions of NMVOCs (specify unit) | National legislation |
| --- | --- | --- |
| 1. Wood and plastic lamination >5 Mg/year (Annex VI, Table III) | 30 g/m2 | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| 2. Coating processes in the car industry (Annex VI, Table IV) |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (a) Car coating (M1, M2) > 15 Mg/year of solvent consumption (i) >5,000 coated items a year |  |  |
| - New installations | 45 g/m2 or 1.3 kg/body + 33 g/m2 |  |
| - Existing installations | 60 g/m2 or 1.9 kg/body + 41 g/m2 |  |
| (ii) ≤5,000 coated monocoques or >3,500 coated chassis a year | 90 g/m2 or 1.5 kg/body + 70 g/m2 |  |
| (b) Coating of new truck cabins (N1, N2, N3) >15 Mg/year of solvent consumption (i) ≤5,000 coated items a year | ... |  |
| -New installations | 65 g/m2 |  |
| - Existing installations | 85 g/m2 |  |
| (ii) >5,000 coated items a year |  |  |
| - New installations | 55 g/m2 |  |
| - Existing installations | 75 g/m2 |  |
| (c) Coating of new trucks and vans (without cabin) (N1, N2,N3) >15 Mg/year of solvent consumption (i) ≤2,500 coated items a year |  |  |
| - New installations | 90/m2 |  |
| - Existing installations | 120 g/m2 |  |
| (ii) >2,500 coated items a year |  |  |
| - New installations | 70 g/m2 |  |
| - Existing installations | 90 g/m2 |  |
| (d) Coating of new buses (M3) >15 Mg/year of solvent consumption (i) ≤2,000 coated items a year |  |  |
| - New installations | 210 g/m2 |  |
| - Existing installations | 290 g/m2 |  |
| (ii) >2,000 coated items a year |  |  |
| - New installations | 150 g/m2 |  |
| - Existing installations | 225 g/m2 |  |
| 3. Dry cleaning (Annex VI, Table VII) | 20 g/kg | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| 4. Extraction of vegetable and animal fat and refining of vegetable oil >10 Mg of solvent consumption a year (Annex VI, Table XIII) |  | Decree on the emission limit values of volatile organic compounds into the atmosphere from installations using organic solvents (OJ RS, No. 112/2005, 37/2007) |
| (a) Animal fat | 1.5 kg/Mg |  |
| (b) Castor | 3.0 kg/Mg |  |
| (c) Rape seed | 1.0 kg/Mg |  |
| (d) Sunflower seed | 1.0 kg/Mg |  |
| (e) Soya beans (normal crush) | 0.8 kg/Mg |  |
| (f) Soya beans (white flakes) | 1.2 kg/Mg |  |
| (g) Other seeds and vegetable material | 3.0 kg/Mg |  |
| (h) All fractionation processes, excl. degumming | 1.5 kg/Mg |  |
| (i) Degumming | 4.0 kg/Mg |  |

- Question 49: With reference to article 7, paragraph 1(a)(i), please specify whether your country, instead of applying the measures referred to in articles 3.2 and 3.3, has applied any alternative emission reduction strategies to achieve overall emission levels for all source categories together, equivalent to those resulting from the measures. Please provide details of any such strategies and the way in which overall emission levels are achieved.

Answer 49:

Alternative emission reduction strategy is applied for VOCs emissions from stationary sources (i.e. industrial installations) as follows:

(a) 	An operator may apply to the competent authority (Environmental Agency - EARS) for VOCs emission reduction scheme by which at least equal reduction of emissions as with complying with the limit values would be achieved (as outlined in Appendix II of Annex VI of the Protocol). If approved, installation is not obliged to comply with emission limit values for VOCs. 53 out of total 154 VOC installations in Slovenia utilize emission reduction scheme. In cases where substitutes containing little or no solvent are still under development, a time extension of was given to the operators to implement emission reduction plans.

(b) 	Existing VOCs emitting installations which operate existing abatement equipment and comply with the following emission limit values: (i) 50 mg VOCs/Nm3 in the case of incineration and (ii) 150 mg VOCs/Nm3 in the case of any other abatement equipment, are exempt from the waste gaseous emissions limit values given in Tables 24 and 25 until 21 Dec. 2013.

- Question 50: With reference to article 7, paragraph 1(a)(ii), where your country, taking into consideration the costs and advantages, considers certain limit values, as specified in accordance with article 3.3, not to be technically and economically feasible for specific existing stationary sources, please provide a justification for this.

Answer 50:

Slovenia applies the limit values specified in Anexes IV, V and VI of the Protocol.

- Question 51: With reference to article 3, paragraph 5 and annex VIII, please provide details of the most recent limit values applied in your country to new passenger cars and light-duty vehicles. Please complete the table below.

Answer 51:

The EC Directives on limit values for new passenger cars and light-duty vehicles are transposed into national legislation with Technical specifications for motor vehicles and trailer vehicles (OJ  RS, No. 33/2003, 61/2004, 142/2004, 75/2005). Emission limit values for EURO IV from the national legislation are given in Table 26.

Table 26: Question 51

| Category, class | Reference mass (RW) (kg) | National legislation and date of application | Limit values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CO |  | HC | NOx |  | HC+NOx |  | ## Particulates |
|  |  |  | L1(g/km) |  | L2 (g/km) | L3(g/km) |  | L2+L3 (g/km) |  | L4 (g/km) |
|  |  |  | Petrol | Diesel | Petrol | ### Petrol | Diesel | Petrol | Diesel | Diesel |
| M | All | Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005) Date of application: 2006.01 | 1.0 | 0.50 | 0.10 | 0.08 | 0.25 | - | 0.30 | 0.025 |
| N1 (I) | RW≤1305 | Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005) Date of application: 2006.01 | 1.0 | 0.50 | 0.1 | 0.08 | 0.25 | - | 0.30 | 0.025 |
| N1 (II) | 1305< RW≤1760 | Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005) Date of application: 2007.01 | 1.81 | 0.63 | 0.13 | 0.10 | 0.33 | - | 0.39 | 0.04 |
| N1 (III) | 1760<RW | Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005) Date of application: 2007.01 | 2.27 | 0.74 | 0.16 | 0.11 | 0.39 | - | 0.46 | 0.06 |

1/ For further information, see annex VIII, Table I.

- Question 52: With reference to article 3, paragraph 5 and annex VIII, please provide details of the most recent limit values applied in your country to new heavy-duty vehicles if the ESC/ELR test1/ is used. Please complete the table below.

Answer 52:

The EC Directives on limit values for new heavy-duty vehicles are transposed into national legislation with Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005). LVs (ESC/ELR test) from the national legislation are given in Table 27.

Table 27: Question 52

| National legislation and date of application | CO (g/kWh) | HC (g/kWh) | NOx (g/kWh) | # Particulates (g/kWh) | Smoke (m-1) |
| --- | --- | --- | --- | --- | --- |
| Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005) Date of application: 2006.10 | 1.5 | 0.46 | 3.5 | 0.02 | 0.5 |

1/ For further information, see annex VIII, Table II.

- Question 53: With reference to article 3, paragraph 5, and annex VIII, please provide details of the most recent limit values applied in your country to new heavy-duty vehicles if the ETC test1 is used. Please complete the table below.

Answer 53:

The EC Directives on limit values for new heavy-duty vehicles are transposed into national legislation with Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005). LVs (ETC test) from the national legislation are given in Table 28.

Table 28: Question 53

| National legislation and date of application | CO (g/kWh) | Non-methane HC (g/kWh) | Methane (g/kWh) | NOx (g/kWh) | # Particulates (g/kWh) |
| --- | --- | --- | --- | --- | --- |
| Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005) Date of application: 2006.10 | 4.0 | 0.55 | 1.1 | 3.5 | 0.03 |

1/ For further information, see annex VIII, Table III.

Question 54: With reference to article 3, paragraph 5, and annex VIII, please provide details of the most recent limit values applied in your country to new diesel engines for non-road mobile machines (ISO 8178). Please complete the table below.

Answer 54:

Limit values for new diesel engines for non-road mobile machines are laid down in Rules on the emission of gaseous and particulate pollutants from internal combustion engines installed in non-road mobile machinery (OJ RS, No. 85/2003, 22/2005, 92/2005, 95/2007).

Table 29: Question 54

| Net power1/ (p) (kW) | National legislation and date of application | CO (g/kWh) | HC (g/kWh) | NOx (g/kWh) | # PM (g/kWh) |
| --- | --- | --- | --- | --- | --- |
| 130≤ P<560 | Rules on the emission of gaseous and particulate pollutants from internal combustion engines installed in non-road mobile machinery (OJ RS, No. 85/2003, 22/2005 92/2005, 95/2007) Date of application: 2003.01 | 3.5 | 1.0 | 6.0 | 0.2 |
| 75≤ P<130 | Rules on the emission of gaseous and particulate pollutants from internal combustion engines installed in non-road mobile machinery (OJ RS, No. 85/2003, 22/2005, 92/2005, 95/2007) Date of application: 2004.01 | 5.0 | 1.0 | 6.0 | 0.3 |
| 37≤ P<75 | Rules on the emission of gaseous and particulate pollutants from internal combustion engines installed in non-road mobile machinery (OJ RS, No. 85/2003, 22/2005, 92/2005, 95/2007) Date of application: 2005.01 | 5.0 | 1.3 | 7.0 | 0.4 |
| 18≤ P<37 | Rules on the emission of gaseous and particulate pollutants from internal combustion engines installed in non-road mobile machinery (OJ RS, No. 85/2003, 22/2005, 92/2005, 95/2007) Date of application: 2002.01 | 5.5 | 1.5 | 8.0 | 0.8 |

1/ For further information, see annex VIII,  Table V.

- Question 55: With reference to article 3, paragraph 5, and annex VIII, please provide details of the most recent limit values applied in your country to new motorcycles and 3- and     4-wheelers (> 50 cm3; > 45 km/h). Please complete the table below.

Answer 55:

The EC Directives on limit values for new motorcycles and 3- and 4-wheelers are transposed into national legislation with Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005). LVs  from the national legislation are given in Table 30.

Table 30: Question 55

| Engine type1/ | National legislation and date of application | CO (g/km) | HC (g/km) | NOx (g/km) |
| --- | --- | --- | --- | --- |
| 1. Two-stroke | Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003 61/2004 142/2004 75/2005) Date of application: 1999.06.17 |  |  |  |
| (a) Motorcycles |  | 8 | 4.0 | 0.1 |
| (b) 3- and 4- wheelers |  | 12 | 6 | 0.15 |
| 2. Four-stroke | Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003 61/2004 142/2004 75/2005) Date of application: 1999.06.17 |  |  |  |
| (a) Motorcycles |  | 13 | 3.0 | 0.3 |
| (b) 3- and 4- wheelers |  |  |  |  |

1/ For further information, see annex VIII, Table VI.

- Question 56: With reference to article 3, paragraph 5, and annex VIII, please provide details of the most recent limit values applied in your country to new mopeds (≤ 50 cm3; < 45 km/h) 1/. Please complete the table below.

Answer 56:

The EC Directives on limit values for new mopeds are transposed into national legislation with Technical specifications for motor vehicles and trailer vehicles (OJ RS, No. 33/2003, 61/2004, 142/2004, 75/2005). LVs  from the national legislation are given in Table 31.

Table 31: Question 56

| National legislation and date of application | CO (g/km) | HC+ NOx (g/km) |
| --- | --- | --- |
| Technical specifications for motor vehicles and trailer vehicles (OJ RS No. 33/2003, 61/2004, 142/2004, 75/2005) Date of application: 2002.06.17 | 1.0 (For 3- and 4-wheelers: 3.5 g/km ) | 1.2 |

1/ For further information, see annex VIII, Table VII.

- Question 57: With reference to article 3, paragraph 5, and annex VIII, Tables VIII and X please provide details of the limit values applied in your country to petrol. Please complete the table below.

Answer 57:

Environmental specifications for petrol are set in Decree on the physical and chemical properties of liquid fuels (OJ RS, No. 63/2006, 92/2006). They are in force since June 2006.

Footnotes in Table 32: /1/ Use of regular unleaded petrol with minimum motor octane number (MON) of 81 and minimum research octane number (RON) of 91 is allowed; /2/ The summer period begins on 1 May and ends on 30 Sept.; /3/ Unleaded motor regular gasoline can contain at the most 21 % (v/v) olefins; /4/ Other mono-alcohols with a final distillation point no higher than the final distillation point in industrial specifications for motor fuels; /5/ Since 1 Jan. 2009 limit value is 10 mg/kg.

Table 32: Question 57

| Parameter1/ | Limits |  | National legislation and date of application |
| --- | --- | --- | --- |
|  | Minimum | Maximum |  |
| 1. Research octane number | 95 /1/ |  | Decree on the physical and chemical properties of liquid fuels (OJ RS, No. 63/06, 92/06). Date of application: June 2006 |
| 2. Motor octane number | 85 /1/ |  | See above. |
| 3. Reid vapour pressure, summer period (in kPa) |  | 60.0 /2/ | See above. |
| 4. Distillation: |  |  | See above. |
| (a) Evaporated at 100o C (in %v/v) | 46.0 |  | See above. |
| (b) Evaporated at 150o C (in %v/v) | 75.0 |  | See above. |
| 5. Hydrocarbon analysis: |  |  | See above. |
| (a) Olefins ( in %v/v) |  | 18.0 /3/ | See above. |
| (b) Aromatics |  | 35.0 | See above. |
| (c) Benzene |  | 1.0 | See above. |
| 6. Oxygen content (in %m/m) |  | 2.7 | See above. |
| 7. Oxygenates (in %v/v): |  |  | See above. |
| (a) Methanol, stabilizing agents must be added |  | 3 | See above. |
| (b) Ethanol, stabilizing agents may be necessary |  | 5 | See above. |
| (c) Iso-propyl alcohol |  | 10 | See above. |
| (d) Tert-butyl alcohol |  | 7 | See above. |
| (e) Iso-butyl alcohol |  | 10 | See above. |
| (f) Ethers containing 5 or more carbon atoms per molecule |  | 15 /4/ | See above. |
| 8. Other oxygenates ( in %v/v) |  | 10 | See above. |
| 9. Sulphur content (in mg/kg) |  | 50 /5/ | See above. |

1/ For further information, see annex VIII, Tables VIII and X.

- Question 58: With reference to article 3, paragraph 5 and annex VIII, Tables IX and XI, please provide details of the limit values applied in your country to diesel. Please complete the table below.

Answer 58:

Environmental specifications for diesel are set in Decree on the physical and chemical properties of liquid fuels (OJ RS, No. 63/2006, 92/2006). They are in force since June 2006.

Table 33: Question 58

|  | Parameter1/ | Limits |  | National legislation and date of application |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |  |
|  | 1. Cetane number | 51 |  | Decree on the physical and chemical properties of liquid fuels (OJ RS, No. 63/06, 92/06). Date of application: June 2006. |  |
|  | 2. Density at 15o C (in kg/m3 ) |  | 845 | See above. |  |
|  | 3. Distillation point: 95% (in oC) |  | 360 | See above. |  |
|  | 4. Polycyclic aromatic hydrocarbons (in % m/m) |  | 11 | See above. |  |
|  | 5. Sulphur content (in mg/kg) |  | 50 (Since 1 Jan. 2009: 10 mg/kg. | See above. |  |

1/ For further information, see annex VIII, Tables IX and XI.

- Question 59: With reference to article 3, paragraph 8 (a) and annex IX, paragraph 3, have you established, published and disseminated an advisory code on good agricultural practice to control ammonia emissions? If so, please provide details of its provisions, relevant to:

(a)	Nitrogen management, taking account of the whole nitrogen cycle;

(b)	Livestock feeding strategies;

(c)	Low-emission manure spreading techniques;

(d)	Low-emission manure storage systems;

(e)	Low-emission animal housing systems;

(f)	Possibilities for limiting ammonia emissions from the use of mineral fertilizers.

Answer 59:

The first Slovenian Advisory Code on good agricultural practice was published and disseminated in 65000 copies in connection with the Rules concerning good agricultural practice and manuring (OJ RS, No. 130/2004). The Code was upgraded by the end of 2006; it includes all the provisions on ammonia control emissions as listed under (a)-(f) in question 59. Internet link to the latest Slovenian Advisory Code (in Slovenian language): http://www.kis.si/datoteke/File/kis/SLO/Publikacije/drugo/Kodeks_DKP.pdf

Slovenia also developed Operationale programme for environmental protection with nitrates from agriculture use for years 2004-2008. Its basic goal is improvement of the water environment, by reducing emissions from dispersed sources (e.g. intensive agriculture, disperse settlement, with no sewage sludge pre-treatment). Operational programme is focused on nitrate pollution input into the water from agriculture sources. The priority is effective agricultural practice, which provides special founded fertilization, sufficient capacities of storage of livestock manure, etc.

National legislation also related to good agricultural practice to control ammonia emissions: Decree on the input of dangerous substances and plant nutrients into the soil (OJ RS, No. 68/1996, 35/2001, 2/2004, 29/2004, 41/2004); Decree on limit values, alert thresholds and critical levels of dangerous substances into the soil (OJ RS, No. 68/1996, 41/2004) and Regulation about cross compliance (OJ RS, No. 34/2007, 66/2007).

- Question 60: With reference to article 3, paragraph 8 (a), and annex IX, paragraph 4, please provide details of the steps taken in your country to limit ammonia emissions from the use of solid fertilizers based on urea.

Answer 60:

The use of urea in Slovenia is very limited, comprising 15% of total N in mineral fertilizers. It is used mainly for fertilizing the maize during vegetation. It was estimated that in this particular and specific case its replacement would technically and economically not be feasible. For this reason, no further steps are foreseen to further limit its use.

- Question 61: With reference to article 3, paragraph 8 (a), and annex IX, paragraph 5, please indicate whether the use of ammonium carbonate fertilizers is prohibited in your country and specify the relevant legislation.

Answer 61:

The use of ammonium carbonate is prohibited, according to the Mineral Fertilizers Act (OJ RS, No. 58/2002 29/06) and Article 12 of Rules on quality of mineral fertilizers (OJ RS, No. 105/2006)

- Question 62:  With reference to article 3, paragraph 8 (a), and annex IX, paragraph 6, please explain how your country ensures the use of the low-emission slurry application techniques listed in guidance document V (ECE/EB.AIR/WG.5/2007/13), taking into account local soil and geomorphological conditions, slurry type and farm structure.

Answer 62:

Low-emission slurry application techniques that are listed in Guidance are recommended by the Slovenian Advisory Code of good agricultural practice. The Code has an advisory status, allowing farmers to take any measure into account as far as they consider it applicable.

Generally, Category 1 techniques are not applicable. The exception is incorporating the slurry by the means of ploughing. The major limiting factors for other techniques (band spreading and injection) are the following:

(i) farm size (6.5 ha and 5.9 livestock units per farm on average),

(ii) land fragmentation (only 12 % of farmers have their land in one piece and more than 20 % of farmers have their land in more than 10 pieces) and

(iii) topography (shallow and stony soils; 72 % of agricultural land lies in less favoured areas).

Special equipment is too expensive for small farmers and due to land fragmentation it is also not possible to organize a special service for slurry application.

The most common technique for ammonia emissions control is Category 2, i.e. spreading in cool, humid conditions. This technique is used traditionally, passing from generation to generation.

- Question 63: With reference to article 3, paragraph 8 (a), and annex IX, paragraph 7, please provide details of the measures taken in your country to limit ammonia emissions from solid manure application, and in particular whether there is a requirement that solid manure applied to land to be ploughed is incorporated within at least 24 hours of spreading.

Answer 63:

Ploughing in of solid manure within at least 24 hours of spreading is recommended in the Slovenian Advisory Code of good agricultural practice. However, the Code has an advisory and not the legal status, allowing farmers to take any measure into account as far as they consider it applicable. Due to specific Slovenian conditions, i.e. the farm structure (small farms with mostly part-time farmers), this particular measure cannot be prescribed, although in practice many farmers apply it.

- Question 64: With reference to article 3, paragraph 8 (a), and annex IX, paragraph 8, please provide details on the use in your country of the low-emission storage systems for new slurry stores (construction commenced after 17 May 2006) on large pig and poultry farms (2,000 fattening pigs, or 750 sows or 40,000 poultry) or techniques that have been shown to reduce emissions by 40 per cent or more compared to the reference listed in guidance document V (ECE/EB.AIR/WG.5/2007/13).

Answer 64:

Low-emission storage systems for the slurry stores, as described in the Guidance Documents on Control Techniques and Economic Instruments to the Gothenburg Protocol (EB.AIR/1999/2), Chapter V. (C) – p. 103-105, are obliged to be used on large pig and poultry farms in Slovenia.

According to the Slovenian legislation, large farms (new and existing farms) need to have environmental integrated permit (IPPC) for their operation. For issuing such permit, a farm has to meet requirements of the best available technique (BAT) as in the European BREF reference documents. Major BAT requirements for the farms are covered by the BREF ILF (IPPC Reference Document on Best Available Techniques for Intensive Rearing of Poultry and Pigs, BREF ILF, issued July 2003;

http://eippcb.jrc.es/pages/FActivities.htm). BREF ILF includes,  among others, all technical provisions on emissions of ammonia from agriculture from the Gothenburg Protocol Guidance Document (Chapter V).

The following BAT techniques to cover slurry stores are required in Slovenia:

(i) for slurry tanks: a rigid lid, roof or tent structure, or a floating cover (such as chopped straw, natural crust, canvas, foil peat, light expanded clay aggregate or expanded polystyrene).

(ii) for slurry storage lagoons: a plastic cover or a floating cover (such as chopped straw, light expanded clay aggregate or natural crust).

The decision on what type of cover is performed can only be taken on a case by case basis.

- Question 65: With reference to article 3, paragraph 8 (a) and annex IX, paragraph 9, please provide details of whether emission reductions of 40 per cent have been achieved in your country for existing slurry stores (construction commenced on or before 17 May 2006) on large pig and poultry farms (2,000 fattening pigs, or 750 sows or 40,000 poultry).

Answer 65:

The same BAT requirements apply to new and existing farms. Please see A. 64.

- Question 66: With reference to article 3, paragraph 8 (a) and annex IX, paragraph 10, please provide details of the use in your country of housing systems for new animal housing on large pig and poultry farms which have been shown to reduce emissions by 20 per cent or more compared to the reference listed in guidance document V.

Answer 66:

The following low-emission housing system (also listed in the Guidance Document, Chapter V. (D) is used in the existing large farms in Slovenia:

(i) for the large pig farms: partly slatted floor;

(ii) for laying hens: solid fully littered floor;

(iii) for broilers: solid fully littered floor (in the well insulated fan/naturally ventilated house and equipped with non-leaking drinking system).

Obligations for the new and existing farms on the use of BAT and control techniques for preventing/abating emissions are described in A.64