SLOVAK REPUBLIC

Informative Inventory and Projection Report 2010

according to the National Emission Ceiling Directive (NEC) 2001/81/EC Article 8 paragraph 1 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 23 October 2001 on national emission ceiling for certain atmospheric pollutants

Ministry of Environment of the Slovak Republic

Slovak Hydrometeorological Institute

December 31, 2010

1.	Executive Summary	3

2.	Introduction	3

2.1.	Institutional arrangements	3

2.2.	Inventory preparation process	3

3.	Recalculations	4

Table 3.1: The emissions in NFR classification for category 1A3bi in Gg	5

Table 3.2: The emissions in NFR classification for category 1A3bii in Gg	5

Table 3.3: The emissions in NFR classification for category 1A3biii in Gg	6

Table 3.4: The emissions in NFR classification for category 1A3biv in Gg	6

Table 3.5: The comparison of emission inventory NMVOC for categories Solid waste disposal on land (6A) and for Waste incineration (6C) in Gg	7

4.	Emission projections	8

Table 4.1: Indicators of historical data of population and GDP in the SR	9

5.	Conclusion	10

Figure 1: NOx emission projections	11

Figure 2: NMVOC emission projections	11

  - Executive Summary

Major recalculation in 2010 submission for NEC Directive were provided in road transport category. The recalculation were used new model of COPERT IV version 7.1 and the revised results back to 2000 for all pollutants are provided in the Chapter 3 of this report. Version 7.1 includes new Hot emission factors for motorcycles are a reasoned from the experimental data from the ARTEMIS project, which are reviewed and new observations contained in the EMPA. As a result of the call control review was carried out calculating the carbon content of CO2 and calculate the integrated CO2 emission factor for the Slovak Republic from gasoline and diesel fuels. The calculations presented itself and the data shows fuel consumption for petrol and diesel.

- Total energy content is expressed in TJ

- Implied Emission Factor (IEF) is expressed in t CO2/TJ

- Emissions per unit of energy contained in the fuel (this indicator is used for international comparisons)

- Emissions in t.

Waste sector was recalculated in case of NMVOC because of correction of activity data.

The recalculation was performed in sector Agriculture other (4G – Ammonium Phosphate) for NH3 emissions up to year 2000 because of double counting correction of activity data.

Emission projections were prepared for following scenarios:

- With measures (WM) – BAU scenario.

- With additional measures (WAM).

  - Introduction

## Institutional arrangements

Slovak Hydrometeorological Institute is responsible for preparation of emission inventories under CLRTAP and emission projections under NEC Directive. Data are send to the Ministry of the Environment from where are after verification send to UN ECE secretariat and copy send to UBA Vienna. Inventory and projection preparation under UN FCCC was exercised by the same way.

Official submissions of emission inventory and projection are archiving in computers of responsible expert on Slovak Hydrometeorological Institute and Ministry of the Environment of reporting under Convention LRTAP and NEC Directive and other experts included in system for preparation inventories and projections and are archiving on electronic media. Emission projections are also printing as an inside document for Ministry of Environment.

The last update of emission inventory and projections was performed in year 2010.

## Inventory preparation process

Preparation of emission inventories and projections were prepared according to the EMEP/EEA Guidebook 2009. Recalculations are described in Chapter 3.

Road transport was prepared by model of COPERT IV version 7.1.Calculation in model COPERT IV is based on EMEP/EEA Guidebook methodology. This methodology is balancing fifteen different emissions including greenhouse gases from road transport. Preparation of basic pollutants inventory is based on sequence calculation for each vehicle category and summing. Emission factors are set by model and are different for all types of fuel, different vehicle categories and different technological level. Vehicle fleet of road transport has been divided into 6 basic categories and 83 subcategories. These subcategories are divided to urban, road and highway transportation.

Methodology recognize several country specific parameters:

- Structure of vehicle fleet;

- Vehicle age;

- Predominant type of character traffic;

- Parameters of fuels;

- Climate conditions.

Emission calculation is based on following types of input data:

- Total fuel consumption;

- Vehicle fleet;

- Driving conditions;

- Emission factors;

- Additional parameters.

Exhaust emissions from road transport are divided on two types:

- so-called "cold emissions", which are additional emissions with start of cold motor;

- so-called "hot emissions", which produce motor of vehicle warmed on operating temperature.

Outputs from model COPERT IV are MS Excel files with model-user requirements. After export, data from model are rewriting to standard format (for example to NFR code).

  - Recalculations

This chapter compare results of NFR categories of emission inventory from road transport from last emission inventory Submission 2009 and new emission inventory submission 2010.

The table 3.5 compares NMVOC emissions before and after recalculation in waste sector. Solid waste disposal on land was recalculated per year 2002. Incineration of industrial waste was recalculated per years 2002, 2004 and per year 2005. Furthermore was done recalculation of incineration of clinical waste per year 2008. All changes was done due to revision of activity data in emission inventory of NMVOC.

### Table 3.1: The emissions in NFR classification for category 1A3bi in Gg

| Emissions (Gg) | NOx | NMVOC | SO2 | NH3 |
| --- | --- | --- | --- | --- |
| 2000 |  |  |  |  |
| Submission 2009 | 13,524 | 12,830 | 0,204 | 0,236 |
| Submission 2010 | 13,525 | 14,724 | 0,210 | 0,211 |
| 2001 |  |  |  |  |
| Submission 2009 | 14,482 | 13,945 | 0,208 | 0,317 |
| Submission 2010 | 14,482 | 15,819 | 0,221 | 0,282 |
| 2002 |  |  |  |  |
| Submission 2009 | 12,469 | 12,589 | 0,219 | 0,291 |
| Submission 2010 | 12,438 | 13,940 | 0,224 | 0,258 |
| 2003 |  |  |  |  |
| Submission 2009 | 11,193 | 11,126 | 0,072 | 0,379 |
| Submission 2010 | 11,152 | 12,326 | 0,073 | 0,350 |
| 2004 |  |  |  |  |
| Submission 2009 | 10,569 | 10,468 | 0,068 | 0,355 |
| Submission 2010 | 10,570 | 10,621 | 0,070 | 0,284 |
| 2005 |  |  |  |  |
| Submission 2009 | 9,638 | 9,934 | 0,081 | 0,425 |
| Submission 2010 | 9,637 | 10,764 | 0,081 | 0,359 |
| 2006 |  |  |  |  |
| Submission 2009 | 7,945 | 6,035 | 0,080 | 0,447 |
| Submission 2010 | 8,497 | 6,815 | 0,069 | 0,358 |
| 2007 |  |  |  |  |
| Submission 2009 | 7,706 | 5,974 | 0,082 | 0,424 |
| Submission 2010 | 7,706 | 6,534 | 0,082 | 0,358 |
| 2008 |  |  |  |  |
| Submission 2009 | 7,283 | 6,036 | 0,073 | 0,635 |
| Submission 2010 | 7,288 | 6,511 | 0,073 | 0,549 |

### Table 3.2: The emissions in NFR classification for category 1A3bii in Gg

| Emissions (Gg) | NOx | NMVOC | SO2 | NH3 |
| --- | --- | --- | --- | --- |
| 2000 |  |  |  |  |
| Submission 2009 | 1,136 | 0,167 | 0,073 | 0,010 |
| Submission 2010 | 1,137 | 0,170 | 0,067 | 0,009 |
| 2001 |  |  |  |  |
| Submission 2009 | 1,297 | 0,217 | 0,075 | 0,016 |
| Submission 2010 | 1,297 | 0,221 | 0,068 | 0,014 |
| 2002 |  |  |  |  |
| Submission 2009 | 1,760 | 0,354 | 0,095 | 0,016 |
| Submission 2010 | 1,761 | 0,273 | 0,090 | 0,014 |
| 2003 |  |  |  |  |
| Submission 2009 | 4,781 | 0,546 | 0,030 | 0,018 |
| Submission 2010 | 2,782 | 0,464 | 0,029 | 0,015 |
| 2004 |  |  |  |  |
| Submission 2009 | 1,859 | 0,314 | 0,018 | 0,023 |
| Submission 2010 | 1,860 | 0,319 | 0,016 | 0,019 |
| 2005 |  |  |  |  |
| Submission 2009 | 2,015 | 0,368 | 0,018 | 0,029 |
| Submission 2010 | 2,015 | 0,373 | 0,019 | 0,023 |
| 2006 |  |  |  |  |
| Submission 2009 | 2,117 | 0,276 | 0,019 | 0,029 |
| Submission 2010 | 1,531 | 0,430 | 0,013 | 0,023 |
| 2007 |  |  |  |  |
| Submission 2009 | 2,064 | 0,336 | 0,018 | 0,027 |
| Submission 2010 | 2,064 | 0,341 | 0,018 | 0,021 |
| 2008 |  |  |  |  |
| Submission 2009 | 1,77 | 0,276 | 0,019 | 0,039 |
| Submission 2010 | 1,771 | 0,281 | 0,019 | 0,034 |

### Table 3.3: The emissions in NFR classification for category 1A3biii in Gg

| Emissions (Gg) | NOx | NMVOC | SO2 | NH3 |
| --- | --- | --- | --- | --- |
| 2000 |  |  |  |  |
| Submission 2009 | 17,015 | 1,364 | 0,392 | 0,007 |
| Submission 2010 | 17,017 | 1,416 | 0,393 | 0,005 |
| 2001 |  |  |  |  |
| Submission 2009 | 19,023 | 1,511 | 0,386 | 0,008 |
| Submission 2010 | 19,024 | 1,585 | 0,387 | 0,006 |
| 2002 |  |  |  |  |
| Submission 2009 | 21,086 | 1,704 | 0,418 | 0,009 |
| Submission 2010 | 20,993 | 1,778 | 0,418 | 0,007 |
| 2003 |  |  |  |  |
| Submission 2009 | 17,019 | 0,957 | 0,048 | 0,007 |
| Submission 2010 | 19,148 | 1,759 | 0,053 | 0,005 |
| 2004 |  |  |  |  |
| Submission 2009 | 25,222 | 1,820 | 0,070 | 0,010 |
| Submission 2010 | 25,970 | 1,988 | 0,073 | 0,008 |
| 2005 |  |  |  |  |
| Submission 2009 | 31,448 | 2,038 | 0,089 | 0,013 |
| Submission 2010 | 31,457 | 2,159 | 0,089 | 0,009 |
| 2006 |  |  |  |  |
| Submission 2009 | 29,220 | 2,212 | 0,096 | 0,041 |
| Submission 2010 | 31,413 | 1,874 | 0,093 | 0,008 |
| 2007 |  |  |  |  |
| Submission 2009 | 34,514 | 2,548 | 0,104 | 0,016 |
| Submission 2010 | 34,514 | 2,685 | 0,103 | 0,011 |
| 2008 |  |  |  |  |
| Submission 2009 | 34,977 | 2,213 | 0,116 | 0,017 |
| Submission 2010 | 34,977 | 2,336 | 0,116 | 0,013 |

### Table 3.4: The emissions in NFR classification for category 1A3biv in Gg

| Emissions (Gg) | NOx | NMVOC | SO2 | NH3 |
| --- | --- | --- | --- | --- |
| 2000 |  |  |  |  |
| Submission 2009 | 0,010 | 0,844 | 0 | 0,0002 |
| Submission 2010 | 0,013 | 1,082 | 0,000863387 | 0,00156779 |
| 2001 |  |  |  |  |
| Submission 2009 | 0,014 | 1,107 | 0 | 0,0002 |
| Submission 2010 | 0,018 | 1,397 | 0,001128555 | 0,000207288 |
| 2002 |  |  |  |  |
| Submission 2009 | 0,008 | 0,569 | 0 | 0,0001 |
| Submission 2010 | 0,017 | 1,323 | 0,00114359 | 0,000205302 |
| 2003 |  |  |  |  |
| Submission 2009 | 0,011 | 0,853 | 0 | 0,0002 |
| Submission 2010 | 0,014 | 1,071 | 0,000400111 | 0,000177904 |
| 2004 |  |  |  |  |
| Submission 2009 | 0,012 | 0,698 | 0 | 0,0002 |
| Submission 2010 | 0,012 | 1,004 | 0,000294226 | 0,000172019 |
| 2005 |  |  |  |  |
| Submission 2009 | 0,018 | 0,648 | 0 | 0,0002 |
| Submission 2010 | 0,016 | 1,173 | 0,000446533 | 0,000163856 |
| 2006 |  |  |  |  |
| Submission 2009 | 0,013 | 0,966 | 0 | 0,0002 |
| Submission 2010 | 0,016 | 1,166 | 0,000397359 | 0,000199931 |
| 2007 |  |  |  |  |
| Submission 2009 | 0,015 | 0,784 | 0 | 0,0002 |
| Submission 2010 | 0,015 | 1,015 | 0,000420866 | 0,000142029 |
| 2008 |  |  |  |  |
| Submission 2009 | 0,02 | 0,966 | 0 | 0,0003 |
| Submission 2010 | 0,016 | 0,850 | 0,000421758 | 0,000197708 |

The following table describes comparison of emission inventory for emission NMVOC for categories Solid waste disposal on land and Waste incineration reported in submission 2009 and recalculation of emission inventory in submission 2010.

### Table 3.5: The comparison of emission inventory NMVOC for categories Solid waste disposal on land (6A) and for Waste incineration (6C) in Gg

| Category | 6A | 6C a | 6C b |
| --- | --- | --- | --- |
| 2002 |  |  |  |
| Submission 2009 | 0,00507 |  | 0,465 |
| Submission 2010 | 0,00499 |  | 0,455 |
| 2004 |  |  |  |
| Submission 2009 |  |  | 0,312 |
| Submission 2010 |  |  | 0,301 |
| 2005 |  |  |  |
| Submission 2009 |  |  | 0,405 |
| Submission 2010 |  |  | 0,407 |
| 2008 |  |  |  |
| Submission 2009 |  | 0,0027 |  |
| Submission 2010 |  | 0,0025 |  |

Several recalculations and improvements were implemented into ammonia inventory from agriculture sector in 2010 NECD submission:

- Reallocation and improvements in poultry categories (4B9a-d)

In the previous submission 2009, the total NH3 emissions from poultry were reported in category 4B9a – Laying Hens. According to the improvement in activity data, the disaggregation of poultry emissions into categories laying hens 4B9a, Broilers (4B9b) and other poultry (4B9d) were performed. The NH3 emissions from turkey (4B9c) are included in category other poultry (4B9d). The time series is provided in the following table.

Table 3.6: The overview of poultry emissions in category 4B9a-d in tons

|  |  | Submission 2009 |  | Submission 2010 |  |
| --- | --- | --- | --- | --- | --- |
| Year | Category 4B9a | Category 4B9a | Category 4B9b | Category 4B9d | Total 4B9 |
| 2000 | 4 472,71 | 2 192,92 | 2 002,17 | 277,62 | 4 472,71 |
| 2001 | 4 570,34 | 2 293,59 | 1 990,47 | 286,29 | 4 570,34 |
| 2002 | 4 534,03 | 2 330,59 | 1 884,22 | 319,22 | 4 534,03 |
| 2003 | 4 663,31 | 2 298,28 | 2 113,61 | 251,43 | 4 663,31 |
| 2004 | 4 477,17 | 2 118,43 | 2 106,57 | 252,18 | 4 477,17 |
| 2005 | 4 577,96 | 2 097,33 | 2 221,06 | 259,57 | 4 577,96 |
| 2006 | 4 335,44 | 2 138,95 | 1 865,79 | 330,70 | 4 335,44 |
| 2007 | 4 300,61 | 2 165,69 | 1 800,44 | 334,48 | 4 300,61 |
| 2008 | 3 766,33 | 2 139,43 | 1 431,84 | 195,06 | 3 766,33 |
| 2009 | 4 467,63 | 2 345,29 | 1 935,83 | 186,51 | 4 467,63 |

- Recalculation and improvement in swine category (4B8)

Recalculation in swine category was provided based on new input information about the young piglets under 20 kg. This category was included into inventory in young piglets under 50 kg and also in the separate category under 20 kg. This double counting was improved in 2010 submission. The recalculated NH3 emissions are described in the following table.

Table 3.7: The overview of recalculations in swine emissions in category 4B8 in tons

|  | Submission 2009 | Submission 2010 |  |
| --- | --- | --- | --- |
| Year | Category 4B8 | Category 4B8 | Difference in % |
| 2000 | 7 728 | 7 612 | 98,50% |
| 2001 | 7 855 | 7 755 | 98,73% |
| 2002 | 8 046 | 7 943 | 98,72% |
| 2003 | 7 468 | 7 375 | 98,75% |
| 2004 | 5 946 | 5 873 | 98,77% |
| 2005 | 5 733 | 5 663 | 98,78% |
| 2006 | 4 915 | 4 824 | 98,15% |
| 2007 | 4 303 | 4 219 | 98,05% |
| 2008 | 3 215 | 3 215 | 100,00% |

- Improvements in notation keys reporting (4B2, 4B13)

Buffalo and other animals are not occurring in the inventory, the notation key “NO” was used.

- Reporting of category 4G, no emissions are reported in this category.

  - Emission projections

The emission projections are based on the assumption of macro-economical indicators and the policy of relevant economical sectors (transport, agriculture etc.).

The emission scenario modelling was prepared with the use of software MESSAGE as well as the specific programs in MS EXCEL environment. The air protection legislation was implemented directly into the both models, considering existing and future requirements on the emission concentration limits and the emission ceilings.

Even use of wide range of input data and improvement of methodological approach at activity projection in relevant sectors, the results are influenced by the uncertainties of future development, preferably in the case of macro-economical data and elasticity of final energy consumption. These uncertainties are predominantly related to process of economical transformation and privatization and historical data can be hardly used for future development extrapolation. The emission projection from energy sector will be influenced by opening of electricity market in future as well as by the main pollutant and GHG emission caps.

Input data used for projections are based on detailed information from dbase NEIS. In accordance with this, we used bottom-up methodology for emission projections preparation.

In this time were prepared two emission scenarios:

- Scenario with measures – using year 2008 as base year and included policies and measures implemented to year 2008 and policies and measures planned after year 2008, but adopted at this time. Scenario WM is identical with BAU scenario.

- Scenario with additional measure – using year 2008 as base year and included policies and measure implemented to year 2008 and planned policies and measures.

Input parameters and assumptions for modelling are based on following documents:

- Historical data of GDP in current and fixed prices (year 2000, Statistical Yearbook of the SR 2000 – 2008, Statistical Office of the SR), see table 4.1.

- Historical data of population in the SR (Statistical Yearbook of the SR 2000 – 2008, Statistical Office of the SR), see table 4.1.

- Selected indicators of economical development of the SR in years 2000 – 2008.

- Structure of heat consumption in the households and services sector, based upon the data from the Ministry of building of the SR.

- The preliminary plan for introduction of new energy units and assumption of the electricity generation growth rate in power plants in the Slovak Republic (especially for SE-ENEL a.s. power plant).

- Structure of vehicles in road transport, annual transits and specific consumptions.

- Transport outputs in rail, water and aviation transport.

- Forecast for the development of industrial production.

- Expectations of oil processing in the SR.

- The growth rate in waste sector.

- Emission ceilings.

### Table 4.1: Indicators of historical data of population and GDP in the SR

| Indicators | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Population [thous. people] | 5 401 | 5 380 | 5 379 | 5 379 | 5 383 | 5 387 | 5 394 | 5 401 | 5 412 | 5 418 |
| GDP [Bio Euro 1995 constant prices] | 31,18 | 32,26 | 33,74 | 35,35 | 37,14 | 39,61 | 42,98 | 47,50 | 50,27 | 47,86 |

Policies and measures applicable for scenario with measure on national level:

- The draft European Commission Directive on industrial emissions of December 2007 (COM(2007)0844).

- Act N. 117/2007 which is change and fill up Act. N. 572/2004 about trading with emission quotas.

- The draft European Commission Directive 2009/29/ES from 23.4.2009 to improve and extend the European emissions trading scheme for greenhouse gases

- Act. N. 107/2007 which is change and amended Act. N. 276/2001 about regulation in network sector and change and amended some later regulations.

- Act N. 99/2007 which is change and amended Act. N. 657/2004 about heat energetic and which is amended Act. N. 455/1991 about craft  enterprise in amended later regulations.

- Act N. 112/2008 which is change and amended Act. N. 656/2004 about energy and change and amended in later regulations and Act. N. 283/2008, which is change and amended Act N. 656/2004 about energy and change and amended in later regulations.

- Act N. 476/2008 about effectiveness with using energy (act about energy effectiveness) and about change and emended Act. N. 555/2005 about energy efficiency of buildings and change and emended Act N. 17/2007.

Policies and measures applicable for scenario with additional measures:

- Climate-energy package.

- Act N. 309/2009 about support of renewable energy sources and high effective combination production heat.

- Action plan of using biomass for years 2008-2013.

- Strategy of higher utilization renewable sources in the SR.

- Common assumptions applied for all developed scenarios of modeling the projections emissions from fossil fuel combustion and transformation.

- Assumption of electricity consumption development corresponds to the reference level of annual growth rate.

- Dynamics of considered annual growth rate of heat consumption in residential and industrial sectors correspond with statistical data from the Statistical Office of the SR.

- Final fuel consumption in industry was determined on the basis of the expected share of this sector on GDP formation.

- We assume that economic activities in iron production will develop constantly, without jump changes.

- Assumptions of motor fuel consumption in road transport, as well as of transport outputs in railway, air and water transport, are in compliance with the conception of the Slovak Ministry of Transport, Posts and Telecommunications.

- Government plan for thermal insulation of buildings.

- Scenario with measures

In modeling this scenario, in addition to the above mentioned assumptions, adopted measures were taken into account, especially adopted legislation in the area of air protection and new  ETS regulation. These impacts should be summarized as follows.

- All the emission sources that do not meet the 2015 emission limits have to implement appropriate technologies to achieve them;

- The price of tradable CO2 in ETS framework  is based on assumptions of the EU.

- There is no restriction on the implementation of any technology for new sources in electricity generation sector.

- The ETS scheme  stimulates the penetration of renewable in fuel mix in order to decrease the CO2 emission rate. It has impact on the other emission too.

- WM scenario for emissions of NH3 and NMVOC contains current policies and measures, as in the scenario without measures being used for modeling of the agricultural sector model CAPRI, other data come from FAO (Food and Agriculture Organization).

- Scenario with additional measures

WAM scenario implements the impacts of climate and energy package, as they were defined in accordance with Directive 2009/28/EC on the promotion of energy from RES. Ministry of Economy - Ministry of Economy prepared a draft action plan for renewable energy, where in accordance with this decree established the trajectory of application of renewable  in electricity and heat. These values were used directly in the MESSAGE model and represents a scenario with additional measures.

  - Conclusion

For the NEC Directive submission 2010 of the Slovak Republic, emission projections for scenario with measure and with additional measures were prepared.

Emission projections were prepared in very detailed level especially for energy sector (NFR sector starts with 1) by model MESSAGE and for other sectors based on macro economical indicators and according to the expert analyses for several sectors (e.g. agriculture, transport, etc.). Some major polluters provided directly their own emission projections (e.g. Slovak power plant ENEL, etc.).

Based on results from emission projections, the Slovak Republic fulfils emission ceilings for year 2010 for all relevant pollutants (SO2, NOx, NH3 and NMVOC) as you can see on graphs 1 to 5. The Slovak Republic fulfils all obligations consequent from Directive NEC under Article 8. The Slovak Republic don’t fulfil preparing emission ceiling for year 2020 for all relevant emissions except emission NH3 (Figure 4).

### Figure 1: NOx emission projections

### Figure 2: NMVOC emission projections

Figure 3: SO2 emission projections

Figure 4: NH3 emission projections

Figure 5: PM2.5 emission projections